| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0417 |
| --- | --- |

| Title: | CE8-related: Combination test of CE8.2.2 and CE8.2.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Jian Lou | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This document reports the results of the combination of CE8.2.2 (palette mode and intra mode combination) and CE8.2.5 (separated palette design for the joint CUs containing both luma and chroma CBs). The tests are performed on top of CE8.2.1 (JVET-M0050). Compared with CE8.2.1, when the method is tested with CPR turned off in the setting, the results of the combination show that:

- For 4:2:0 TGM sequences, the proposed method provides 2.5%, 2.8%, and 3.0% BD-rate gains for luma with 92%, 99% and 98% for encoding time, and 101%, 97% and 92% in decoding time under AI, RA, and LD configurations, respectively.

- For Class F sequences, the proposed method provides 0.3%, 0.5%, and 1.5% BD-rate gains for luma, with 100%, 99% and 99% for encoding time, and 101%, 98% and 97% in decoding time under AI, RA, and LD configurations, respectively.

When the method is tested with CPR turned on in the setting,

- For 4:2:0 TGM sequences, the proposed method provides 0.9%, 1.3%, and 2.4% BD-rate gains for luma with 95%, 99% and 100% for encoding time, and 100%, 98% and 100% in decoding time under AI, RA, and LD configurations, respectively.

- For Class F sequences, the proposed method provides 0.0%, 0.2%, and 0.9% BD-rate gains for luma, with 99%, 100% and 103% for encoding time, and 100%, 101% and 98% in decoding time under AI, RA, and LD configurations, respectively.

It’s worthwhile to note that, compared with the results of CE8.2.2 and CE8.2.5 individually, the results of the proposed combination show further synergy between CE8.2.2 and CE8.2.5.

Description of the tests

CE8.2.2 tests a method that combines the palette mode and intra prediction. The method of CE8.2.2 is implemented on top of the separate palette structure. In CE8.2.5, separate palette is extended from intra slices to inter slices regardless of whether the separate tree structure is applied. As a result, if CE8.2.2 is combined with CE8.2.5, synergy will be expected because the method of CE8.2.2 also works for inter slices. In this proposal, we enable both CE8.2.2 and CE8.2.5 and test the combination of CE8.2.2 and CE8.2.5.

Simulation results

The proposed method is evaluated according to the CE8 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the VTM-3.0 software.

The combination is tested with/without CPR. Table 1 shows the results with CPR turned off in the setting. The results are compared with VTM3.0 and CE8.2.1.

Table 1: Results of the CE8.2.2+CE8.2.5 test (CPR is turned off)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.09% | 109% | 99% | -0.01% | -0.05% | 0.01% | 100% | 101% |
| Class A2 | 0.09% | 0.05% | 0.11% | 108% | 101% | -0.01% | -0.09% | 0.03% | 101% | 101% |
| Class B | 0.08% | 0.08% | 0.09% | 107% | 100% | -0.01% | -0.01% | 0.02% | 101% | 100% |
| Class C | 0.09% | 0.09% | 0.06% | 109% | 106% | -0.01% | 0.00% | 0.14% | 101% | 100% |
| Class E | 0.11% | 0.23% | 0.21% | 108% | 101% | -0.01% | 0.07% | 0.01% | 101% | 99% |
| Overall | 0.08% | 0.09% | 0.11% | 108% | 101% | -0.01% | -0.01% | 0.05% | 101% | 100% |
| Class D | 0.12% | 0.10% | 0.14% | 108% | 104% | 0.01% | 0.03% | 0.12% | 101% | 100% |
| Class F | -11.65% | -9.08% | -9.19% | 117% | 90% | -0.30% | -0.19% | -0.19% | 100% | 101% |
| Class SCC 1080p | -34.47% | -28.70% | -28.82% | 121% | 70% | -2.45% | -1.71% | -1.72% | 92% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.06% | 0.10% | 104% | 101% | -0.02% | -0.11% | -0.02% | 100% | 102% |
| Class A2 | 0.09% | 0.13% | 0.24% | 105% | 102% | -0.02% | -0.10% | 0.01% | 100% | 101% |
| Class B | 0.12% | 0.26% | 0.00% | 105% | 98% | -0.02% | -0.02% | 0.10% | 99% | 98% |
| Class C | 0.17% | 0.09% | 0.26% | 107% | 104% | 0.00% | -0.01% | 0.05% | 99% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.12% | 0.15% | 0.14% | 105% | 101% | -0.01% | -0.05% | 0.05% | 99% | 99% |
| Class D | 0.23% | -0.10% | 0.25% | 108% | 103% | 0.03% | 0.06% | 0.00% | 99% | 99% |
| Class F | -8.94% | -8.23% | -8.68% | 109% | 99% | -0.54% | -0.26% | -0.42% | 99% | 98% |
| Class SCC 1080p | -17.87% | -17.28% | -17.34% | 106% | 90% | -2.81% | -3.20% | -3.42% | 99% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.09% | -0.15% | 0.29% | 107% | 100% | 0.00% | -0.13% | 0.24% | 101% | 102% |
| Class C | 0.19% | 0.26% | 0.33% | 106% | 97% | 0.04% | -0.18% | -0.15% | 100% | 103% |
| Class E | 0.41% | 0.57% | 0.21% | 107% | 99% | 0.15% | -0.66% | -0.24% | 100% | 104% |
| Overall | 0.20% | 0.17% | 0.28% | 107% | 99% | 0.05% | -0.28% | -0.01% | 100% | 103% |
| Class D | 0.21% | 0.20% | 0.55% | 109% | 102% | -0.06% | -0.32% | 0.15% | 100% | 100% |
| Class F | -5.21% | -5.18% | -5.88% | 108% | 97% | -1.49% | -0.86% | -1.21% | 99% | 97% |
| Class SCC 1080p | -9.62% | -10.30% | -10.03% | 104% | 88% | -2.99% | -3.99% | -4.02% | 98% | 92% |

Table 2 shows the results with CPR turned on in the setting. The results of CE8.2.2 are compared with VTM3.0 and CE8.2.1.

Table 2: Results of the CE8.2.2+CE8.2.5 test (CPR is turned on)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.17% | 0.02% | 110% | 103% | -0.01% | 0.00% | -0.06% | 99% | 98% |
| Class A2 | 0.09% | 0.10% | 0.11% | 107% | 102% | 0.00% | 0.03% | 0.00% | 98% | 97% |
| Class B | 0.08% | 0.12% | 0.11% | 109% | 105% | -0.01% | 0.04% | 0.05% | 103% | 103% |
| Class C | 0.10% | 0.20% | 0.10% | 105% | 101% | 0.02% | 0.09% | 0.03% | 99% | 100% |
| Class E | 0.13% | 0.18% | 0.02% | 107% | 103% | -0.01% | -0.10% | 0.00% | 101% | 102% |
| Overall | 0.09% | 0.15% | 0.08% | 108% | 103% | 0.00% | 0.02% | 0.01% | 100% | 100% |
| Class D | 0.08% | 0.31% | 0.06% | 108% | 104% | -0.01% | 0.12% | 0.11% | 100% | 101% |
| Class F | -4.49% | -2.41% | -2.46% | 110% | 96% | -0.03% | 0.05% | 0.05% | 99% | 100% |
| Class TGM | -13.43% | -8.20% | -8.16% | 107% | 83% | -0.92% | -0.31% | -0.36% | 95% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.26% | 0.14% | 102% | 99% | 0.01% | 0.02% | 0.00% | 95% | 95% |
| Class A2 | 0.10% | -0.03% | 0.06% | 103% | 100% | -0.01% | -0.11% | 0.02% | 95% | 94% |
| Class B | 0.11% | 0.12% | 0.10% | 106% | 99% | 0.02% | 0.12% | -0.03% | 98% | 97% |
| Class C | 0.15% | 0.01% | 0.31% | 109% | 104% | -0.02% | -0.04% | 0.01% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.11% | 0.09% | 0.15% | 105% | 101% | 0.00% | 0.01% | 0.00% | 97% | 97% |
| Class D | 0.25% | 0.34% | 0.30% | 110% | 104% | -0.02% | 0.10% | 0.50% | 100% | 100% |
| Class F | -2.82% | -2.24% | -2.53% | 109% | 102% | -0.22% | 0.12% | 0.06% | 100% | 101% |
| Class TGM | -5.80% | -5.44% | -5.33% | 103% | 94% | -1.28% | -1.93% | -2.01% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.11% | 0.35% | 0.15% | 112% | 105% | 0.00% | 0.28% | -0.10% | 101% | 99% |
| Class C | 0.14% | 0.17% | 0.39% | 108% | 98% | -0.03% | 0.03% | 0.35% | 99% | 95% |
| Class E | 0.07% | 0.47% | -0.06% | 106% | 97% | -0.12% | 0.49% | 0.11% | 100% | 98% |
| Overall | 0.11% | 0.32% | 0.18% | 109% | 100% | -0.04% | 0.25% | 0.10% | 100% | 98% |
| Class D | 0.25% | 0.90% | -0.36% | 106% | 98% | -0.01% | 0.08% | -0.32% | 98% | 97% |
| Class F | -2.01% | -2.41% | -2.74% | 107% | 96% | -0.87% | -0.81% | -0.93% | 103% | 98% |
| Class TGM | -4.07% | -4.67% | -4.28% | 102% | 96% | -2.43% | -3.45% | -3.23% | 100% | 100% |

Table 3 summarizes the results of CE8.2.2, CE8.2.5, and CE8.2.2+8.2.5 with CPR turned off in the setting. The results of CE8.2.2 are compared with CE8.2.1. Results of the proposed combination show synergy between CE8.2.2 and CE8.2.5. For example, for Class TGM in RA, the combination shows 2.8% BD-rate gains for luma, while CE8.2.2 and CE8.2.5 show only 1.1% and 1.2% gain, respectively.

Table 3: Results of the CE8.2.2, CE82.2.5 and CE8.2.2+CE8.2.5 test (CPR is turned off)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE8.2.2 over CE8.2.1 |  |  | CE8.2.5 over CE8.2.1 |  |  | CE8.2.2+CE8.2.5 over CE8.2.1 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -0.01% | -0.05% | 0.01% | 0.00% | 0.00% | 0.00% | -0.01% | -0.05% | 0.01% |
| Class A2 | -0.01% | -0.09% | 0.03% | 0.00% | 0.00% | 0.00% | -0.01% | -0.09% | 0.03% |
| Class B | -0.01% | -0.01% | 0.02% | 0.00% | 0.00% | 0.00% | -0.01% | -0.01% | 0.02% |
| Class C | -0.01% | 0.00% | 0.14% | 0.00% | 0.00% | 0.00% | -0.01% | 0.00% | 0.14% |
| Class E | -0.01% | 0.07% | 0.01% | 0.00% | 0.00% | 0.00% | -0.01% | 0.07% | 0.01% |
| Overall | -0.01% | -0.01% | 0.05% | 0.00% | 0.00% | 0.00% | -0.01% | -0.01% | 0.05% |
| Class D | 0.01% | 0.03% | 0.12% | 0.00% | 0.00% | 0.00% | 0.01% | 0.03% | 0.12% |
| Class F | -0.30% | -0.19% | -0.19% | 0.00% | 0.00% | 0.00% | -0.30% | -0.19% | -0.19% |
| Class TGM | -2.45% | -1.71% | -1.72% | 0.00% | 0.00% | 0.00% | -2.45% | -1.71% | -1.72% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | CE8.2.2 over CE8.2.1 |  |  | CE8.2.5 over CE8.2.1 |  |  | CE8.2.2+CE8.2.5 over CE8.2.1 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -0.02% | 0.05% | 0.02% | -0.04% | -0.06% | 0.00% | -0.02% | -0.11% | -0.02% |
| Class A2 | -0.01% | -0.05% | 0.09% | -0.03% | -0.03% | -0.03% | -0.02% | -0.10% | 0.01% |
| Class B | -0.01% | -0.01% | 0.04% | -0.03% | -0.04% | 0.13% | -0.02% | -0.02% | 0.10% |
| Class C | 0.04% | -0.04% | -0.09% | -0.03% | 0.05% | -0.03% | 0.00% | -0.01% | 0.05% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | -0.03% | -0.02% | 0.03% | -0.01% | -0.05% | 0.05% |
| Class D | 0.02% | 0.20% | -0.08% | 0.01% | 0.05% | 0.00% | 0.03% | 0.06% | 0.00% |
| Class F | -0.36% | -0.30% | -0.40% | -0.16% | 0.06% | -0.01% | -0.54% | -0.26% | -0.42% |
| Class TGM | -1.06% | -0.68% | -0.73% | -1.18% | -0.79% | -0.71% | -2.81% | -3.20% | -3.42% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | CE8.2.2 over CE8.2.1 |  |  | CE8.2.5 over CE8.2.1 |  |  | CE8.2.2+CE8.2.5 over CE8.2.1 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | -0.25% | -0.22% | -0.01% | -0.01% | -0.13% | 0.00% | -0.13% | 0.24% |
| Class C | 0.00% | -0.09% | -0.02% | 0.00% | -0.04% | -0.22% | 0.04% | -0.18% | -0.15% |
| Class E | 0.12% | -0.75% | -0.27% | -0.06% | 0.05% | -0.19% | 0.15% | -0.66% | -0.24% |
| Overall | 0.04% | -0.32% | -0.16% | -0.02% | -0.01% | -0.18% | 0.05% | -0.28% | -0.01% |
| Class D | -0.02% | -0.27% | 0.13% | -0.04% | 0.12% | -0.14% | -0.06% | -0.32% | 0.15% |
| Class F | -0.17% | -0.26% | -0.48% | -1.32% | -0.61% | -0.78% | -1.49% | -0.86% | -1.21% |
| Class TGM | -0.08% | 0.03% | -0.01% | -1.97% | -1.91% | -1.52% | -2.99% | -3.99% | -4.02% |

Reference

- Y.-C. Sun and Jian Lou,	“CE8: Palette mode and intra mode combination (Test8.2.2)”, JVET-M0051, 13th JVET meeting, Jan. 2019.

- Y.-C. Sun, Jian Lou, Yung-Hsuan Chao, Hongtao Wang, Vadim Seregin, Marta Karczewicz, Roman Chernyak, Sergey Ikonin, Jianle Chen,	“CE8: Separate palette coding for luma and chroma (Test8.2.5)”, JVET-M0052, 13th JVET meeting, Jan. 2019.

- X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, "Description of Core Experiment 8: Screen Content Coding Tools”, JVET-L1028, 2018.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).