| oint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0422 |
| --- | --- |

| Title: | CE4-related: Simplified MVD coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Xiaozhong Xu, Xin Zhao, Shan Liu | Tel: Email: | {xlxiangli, xiaozhongxu, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In MVD coding, abs_mvd_greater0_flag and abs_mvd_greater1_flag are context coded. In this contribution, it is proposed to use equal probability to code these two syntax elements to simplify MVD coding process. It is reported that bypass coding abs_mvd_greater0_flag leads to 0.12% (RA) and 0.09% (LDB) luma BD-rate increase, while bypass coding abs_mvd_greater1_flag results in 0.03% (RA) and -0.02% (LDB) luma BD-rate increase.

# Introduction

In current VVC working draft JVET-L1001 [1], the syntax table for MVD coding is as follows, where abs_mvd_greater0_flag and abs_mvd_greater1_flag are context coded while abs_mvd_minus2 and mvd_sign_flag are coded with equal probability.

| mvd_coding( x0, y0, refList ,cpIdx ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

# Proposed methods

It is proposed to 1) use equal probability to code abs_mvd_greater0_flag; 2) use equal probability to code abs_mvd_greater1_flag.

# Simulation results

Two experiments are conducted to evaluate the proposed methods. Each experiment is performed under CTC condition [2]. The anchor for both the experiments is VTM-3.0.

Table 1 Bypass coding abs_mvd_greater0_flag

Table 2 Bypass coding abs_mvd_greater1_flag

# Conclusions

In this contribution, simplification to MVD coding, i.e., bypass coding abs_mvd_greater0_flag and abs_mvd_greater1_flag is proposed. Simulations show that minor coding loss is observed if abs_mvd_greater1_flag is bypass coded. It is proposed to adopt it into VTM.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).