| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0425-v2 |
| --- | --- |

| Title: | CE10: Multi-hypothesis inter prediction (Test 10.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Winken, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-570 martin.winken@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

Version 2 of the document with the following changes:

- Complete experimental results

- New section showing cache hit ratios from the built-in cache model of VTM-3.0

# Abstract

This document reports results for tests CE10.1.2.a – CE10.1.2.d on multi-hypothesis inter prediction.  The total number of motion-compensated prediction signals (per block) which constitute the overall prediction signal is limited to 4 (i.e., up to 2 additional prediction signals).  The multi-hypothesis inter prediction mode is restricted to be available only for blocks that are larger than 8x8 luma samples and have both width and height greater than or equal to 8 luma samples (i.e., the smallest applicable block sizes are 8x16 and 16x8).  The additional prediction signals use full-pel reference sample positions for both luma and chroma (i.e., there is no sub-pel interpolation filtering).  Multi-hypothesis inter prediction cannot be used together with BIO, triangular partition, or combined intra/inter prediction within the same PU.

For test CE10.1.2.a, the following results are reported for this test relative to VTM-3.0:

RA: 	-0.29% (Y),	-0.16% (Cb), -0.12% (Cr)	107% enc. time		97% dec. time

LB:		-0.41% (Y), -0.04% (Cb), -0.04% (Cr)	110% enc. time		99% dec. time

For test CE10.1.2.b, the additional prediction parameters are not re-used for merging.  The following results are reported for this test relative to VTM-3.0:

RA:	-0.20% (Y), -0.02% (Cb), -0.05% (Cr)	107% enc. time		96% dec. time

LB:		-0.30% (Y),	-0.10% (Cb), -0.17% (Cr)	111% enc. time		102% dec. time

For test CE10.1.2.c, only 2 different reference pictures can be used within each PU.  The following results are reported for this test relative to VTM-3.0:

RA:	-0.20% (Y), -0.10% (Cb), -0.08% (Cr)	105% enc. time		97% dec. time

LB:		-0.20% (Y), -0.19% (Cb), -0.24% (Cr)	107% enc. time		100% dec. time

For test CE10.1.2.d, the additional predictions are only used for the luma component.  The following results are reported for this test relative to VTM-3.0:

RA:	-0.29% (Y), 0.03% (Cb), 0.04% (Cr)		107% enc. time		96% dec. time

LB:		-0.40% (Y),	0.06% (Cb), -0.02% (Cr)	110% enc. time		99% dec. time

# Introduction

In the multi-hypothesis inter prediction mode, one or more additional prediction signals are signaled, in addition to the conventional uni/bi prediction signal.  The resulting overall prediction signal is obtained by sample-wise weighted superposition.  With the uni/bi prediction signal puni/bi and the first additional inter prediction signal/hypothesis h3, the resulting prediction signal p3 is obtained as follows:

p3=(1-α)puni/bi+αh3

The weighting factor α is specified by the new syntax element add_hyp_weight_idx, according to the following mapping:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

Note that for the additional prediction signals, in the tests CE10.1.2.a, CE10.1.2.b, and CE10.1.2.d, the concept of prediction list0/list1 is abolished, and instead one combined list is used.  This combined list is generated by alternatingly inserting reference frames from list0 and list1 with increasing reference index, omitting reference frames which have already been inserted, such that double entries are avoided.  In test CE10.1.2.c, only 2 different reference pictures can be used within each PU, and therefore it is indicated by one flag which reference frame is used.

Analogously to above, more than one additional prediction signal can be used.  The resulting overall prediction signal is accumulated iteratively with each additional prediction signal.

pn+1=1-αn+1pn+αn+1hn+1

The resulting overall prediction signal is obtained as the last pn (i.e., the pn having the largest index n).  Within this CE, up to two additional prediction signals can be used (i.e., n is limited to 2).  Note that due to the iterative accumulation approach, the number of required PU sample buffers for storing intermediate prediction signals is not increased relative to bi-prediction (i.e., two buffers are sufficient).

## Multi-Hypothesis Motion Estimation

First, the inter modes with no explicitly signaled additional inter prediction parameters are tested.  For the best two of these modes (i.e., having lowest Hadamard RD cost), additional inter prediction hypotheses are searched.  For that purpose, for all combinations of the following parameters, a motion estimation with a restricted search range of 16 is performed:

- Weighting factor α

- Reference frame for the additional prediction hypothesis

For determining the best combination of these two parameters, a simplified RD cost using Hadamard distortion measure and approximated bit rate is used.  The chosen parameter combination is then used to compute a more accurate RD cost, using forward transform and quantization, which is compared against the so-far best found coding mode for the current block.

# Interaction with other coding tools

## Normal merge mode (non-MMVD, non-sub-block)

- Additional prediction signals can be explicitly signaled, but not in SKIP mode

- Additional prediction signals can also be inherited from spatially neighboring blocks as part of the merging candidate, but this is limited to

  - neighboring blocks within the current CTU, or

  - neighboring blocks from the left CTU

- Additional prediction signals cannot be inherited from the top CTU or from a temporally co-located block.

- All explicitly signaled additional prediction signals use the same AMVP candidate list which is generated for the first explicitly signaled additional prediction signal, so there has to be done

  - one merging candidate list construction process

  - one AMVP candidate list construction process

- The total of explicitly signaled and inherited (merged) additional prediction signals is limited to be less than or equal to 2.

## MMVD

- Additional prediction signals can be explicitly signaled, but not in MMVD SKIP mode

- There is no inheritance/merging of additional prediction signals from merging candidates

- All explicitly signaled additional prediction signals use the same AMVP candidate list which is generated for the first explicitly signaled additional prediction signal, so there has to be done

  - one MMVD list construction process

  - one AMVP candidate list construction process

## Sub-block merge mode

- Additional prediction signals can be explicitly signaled, but not in SKIP mode

- There is no inheritance/merging of additional prediction signals from merging candidates

- All explicitly signaled additional prediction signals use the same AMVP candidate list which is generated for the first explicitly signaled additional prediction signal, so there has to be done

  - one sub-block merging candidate list construction process

  - one AMVP candidate list construction process

## Non-affine AMVP mode

- Additional prediction signals can be explicitly signaled in case of bi-prediction

- Only two AMVP candidate lists have to be constructed (for the first two, i.e. non-additional prediction signals)

- For the additional prediction signals, one of the two AMVP candidate lists is used:

  - If the POC of the reference picture of the additional prediction signal equals the POC of the used list1 reference picture, the list1 AMVP candidate list is used.

  - Otherweise the list0 AMVP candidate list is used.

## Affine AMVP mode

- Additional (translational) prediction signals can be explicitly signaled in case of bi-prediction

- Two affine AMVP candidate lists have to be constructed (for the first two, i.e. non-additional prediction signals)

- For the additional prediction signals, one of the two AMVP candidate lists is used:

  - If the POC of the reference picture of the additional prediction signal equals the POC of the used list1 reference picture, the list1 AMVP candidate list is used.

  - Otherweise the list0 AMVP candidate list is used.

- The affine LT mv predictor is used as the mv predictor for the additional prediction signal

## BIO

Multi-hypothesis inter prediction cannot be used together with BIO within one PU:

- If there are additional prediction signals, BIO is disabled for the current PU

## Combined intra/inter

Multi-hypothesis inter prediction cannot be used together with combined intra/inter within one PU:

- If combined intra/inter is selected with a merging candidate that has additional prediction signals, those additional prediction signals are not inherited/merged.

- Additional prediction signals cannot be explicitly signaled in combined intra/inter mode.

  - Triangular mode

Multi-hypothesis inter prediction cannot be used together with triangular mode within one PU:

- If triangular mode is selected with a merging candidate that has additional prediction signals, those additional prediction signals are not inherited/merged.

- Additional prediction signals cannot be explicitly signaled in triangular mode.

# Description of the tests

For all the four tests CE10.1.2.a, CE10.1.2.b, CE10.1.2.c, and CE10.1.2.d, up to 4 motion-compensated prediction signals can be used for blocks that are larger than 8x8 luma samples and have both width and height greater than or equal to 8 luma samples (i.e., the smallest applicable block sizes are 8x16 and 16x8).  The additional prediction hypotheses always use the translational motion model, also in case of an affine coded block.  Multi-hypothesis inter prediction cannot be used together with BIO, triangular partition, or combined intra/inter prediction within the same PU.

In the following, the specifics of the individual tests are described:

- CE10.1.2.a: multi-hypothesis prediction is used for both luma and chroma, additional prediction parameters can be merged, up to 4 different reference pictures can be used for each PU

- CE10.1.2.b: like CE10.1.2.a, but additional prediction parameters cannot be merged

- CE10.1.2.c: like CE10.1.2.a, but only two different reference pictures can be used for each PU

- CE10.1.2.d: like CE10.1.2.a, but multi-hypothesis prediction is only applied to luma

# Software and Settings

The source code was made available to all CE10 participants in the branches CE10.1.2.a, CE10.1.2.b, CE10.1.2.c, CE10.1.2.d of the git repository:

https://vcgit.hhi.fraunhofer.de/JVET-L-CE10/VVCSoftware_VTM.git

The encoder can be run using the default config files, all necessary settings are accordingly pre-defined in the corresponding branches.

# Test results

According to the CE10 description in JVET-L1030 [1] and the common test conditions in JVET-L1010 [2], simulations were performed for random-access (RA) and low-delay B (LB) configuration with VTM-3.0 software.  The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

## Test CE10.1.2.a

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -0.46% |  | -0.36% |  | 106% |  | 97% |  |
| Class A2 | -0.16% | -0.16% |  | 0.05% |  | 106% |  | 100% |  |
| Class B | -0.52% | -0.13% | -0.16% | 107% | 95% |
| Class C | -0.14% | 0.01% | -0.03% | 107% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.29% |  | -0.16% |  | -0.12% |  | 107% |  | 97% |  |
| Class D | -0.08% | 0.00% | 0.06% | 108% | 96% |
| Class F | -0.62% | -0.43% | -0.32% | 106% | 94% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.56% | -0.25% | -0.19% | 110% | 97% |
| Class C | -0.25% | 0.13% | 0.15% | 111% | 104% |
| Class E | -0.36% | 0.06% | -0.05% | 110% | 97% |
| Overall | -0.41% | -0.04% | -0.04% | 110% | 99% |
| Class D | -0.30% | -0.18% | -0.24% | 110% | 102% |
| Class F | -1.23% | -1.04% | -0.86% | 109% | 97% |

## Test CE10.1.2.b

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.14% | -0.21% | 106% | 97% |
| Class A2 | -0.14% | -0.03% | 0.08% | 106% | 100% |
| Class B | -0.35% | 0.02% | -0.07% | 107% | 96% |
| Class C | -0.11% | 0.02% | -0.02% | 107% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.02% | -0.05% | 107% | 96% |
| Class D | -0.04% | 0.01% | 0.09% | 107% | 97% |
| Class F | -0.52% | -0.30% | -0.33% | 106% | 96% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -0.31% | -0.11% | 114% | 104% |
| Class C | -0.19% | 0.19% | -0.13% | 110% | 103% |
| Class E | -0.36% | -0.15% | -0.32% | 109% | 98% |
| Overall | -0.30% | -0.10% | -0.17% | 111% | 102% |
| Class D | -0.28% | 0.12% | 0.34% | 108% | 99% |
| Class F | -1.05% | -0.55% | -0.40% | 110% | 99% |

## Test CE10.1.2.c

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.33% | -0.15% | 105% | 98% |
| Class A2 | -0.12% | -0.09% | 0.09% | 105% | 100% |
| Class B | -0.37% | -0.03% | -0.16% | 105% | 96% |
| Class C | -0.12% | -0.04% | -0.06% | 105% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.10% | -0.08% | 105% | 97% |
| Class D | -0.09% | -0.05% | -0.06% | 106% | 98% |
| Class F | -0.57% | -0.29% | -0.28% | 104% | 94% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.31% | -0.35% | 106% | 97% |
| Class C | -0.17% | 0.14% | -0.04% | 110% | 107% |
| Class E | -0.13% | -0.42% | -0.33% | 106% | 98% |
| Overall | -0.20% | -0.19% | -0.24% | 107% | 100% |
| Class D | -0.20% | 0.10% | -0.06% | 105% | 100% |
| Class F | -0.59% | -0.63% | -0.17% | 107% | 100% |

## Test CE10.1.2.d

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.23% | -0.03% | 106% | 98% |
| Class A2 | -0.12% | 0.13% | 0.19% | 106% | 100% |
| Class B | -0.53% | 0.09% | -0.01% | 107% | 96% |
| Class C | -0.17% | 0.10% | 0.03% | 107% | 93% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | 0.03% | 0.04% | 107% | 96% |
| Class D | -0.08% | 0.04% | 0.12% | 107% | 99% |
| Class F | -0.62% | -0.14% | -0.16% | 107% | 93% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | -0.08% | -0.09% | 110% | 96% |
| Class C | -0.27% | 0.27% | 0.22% | 111% | 102% |
| Class E | -0.33% | 0.00% | -0.23% | 110% | 98% |
| Overall | -0.40% | 0.06% | -0.02% | 110% | 99% |
| Class D | -0.27% | -0.34% | 0.11% | 110% | 101% |
| Class F | -1.10% | -0.83% | -0.61% | 110% | 98% |

# Computational complexity and memory bandwidth analysis

In this section, the computational complexity of the proposed methods in terms of additions and multiplications as well as the resulting memory bandwidth is studied.  Bit-shifts are not counted as they are considered to be almost cost-free from a hardware implementation perspective.  A similar approach as done in [3] for DMVR is followed.  Note that the worst-case computational complexity and memory bandwidth of the decoder is studied here, not the complexity of the search at the encoder.

## Computational complexity of interpolation filtering

For a prediction block of with w and height h and a K-tap interpolation filter, the following numbers of additions and multiplications are required in the worst case:

Additions:			(K – 1) * w * (2*h + K – 1)

Multiplications:		K * w * (2*h + K – 1)

## Computational complexity of signal superposition for bi-prediction

For bi-prediction, two prediction signals p0 and p1 are equally weighted and superposed in order to obtain the resulting overall prediction signal q, i.e. like this:

q = (p0 + p1 + 1) >> 1

Consequently, for a prediction block of with w and height h the total number of additions amounts to:

Additions:			w * h

(Note that the extra rounding “+ 1” is not separately counted in here, but is instead considered to be part of the specifically designed adder.)

## Computational complexity of signal superposition for weighted multi-hypothesis prediction

For multi-hypothesis prediction, two prediction signals p0 and p1 are unequally weighted and superposed in order to obtain the resulting overall prediction signal q.  As described above, the sets of weights {3/4, 1/4} and {9/8, -1/8} can be used. It is indicated in the bit-stream per additional hypothesis which set of weights is to be used.  For the set {3/4, 1/4}, the superposition can be done without multiplication as follows:

q = (3*p0 + p1 + 2) >> 2

= (p0 + (p0<<1) + p1 + 2) >> 2

Similarly, for the set {9/8, -1/8}, the superposition can be done without multiplication as follows:

q = (9*p0 - p1 + 4) >> 3

= (p0 + (p0<<3) - p1 + 4) >> 3

Consequently, in both cases, for a prediction block of with w and height h the total number of additions amounts to:

Additions:			2 * w * h

(Again, the rounding “+ 2” and “+ 4” are not separately counted, but are instead considered to be part of the specifically designed adder.  Furthermore, the subtraction is also counted as one specifically designed adder.)

## Memory bandwidth for interpolation filtering

For a prediction block of with w and height h and a K-tap interpolation filter, the following number of reference samples has to be accessed in the worst cases:

(w + K – 1) * (h + K – 1)

## Worst-case computational complexity and memory bandwidth

In the following table, the worst-case computational complexity and memory bandwidth with respect to (w.r.t.) 8x8 bi-prediction is presented.  For a detailed derivation, reference is made to the accompanying Excel file.  (In the Excel file, the computational complexity is derived as the number of multiplications per luma sample where an addition is counted as 0.25 multiplications, as is done in [3]).  Note that in this worst-case analysis, BIO and weighted averaging are not considered.

| Test | worst-case computational complexity w.r.t. 8x8 bi | worst-case memory bandwidth w.r.t. 8x8 bi |
| --- | --- | --- |
| CE10.1.2.a-c (luma+chroma) | 102% | 107% |
| CE10.1.2.d (only luma) | 101% | 97% |

# Cache-model analysis

The proposed methods have also been evaluated using the built-in cache model of VTM-3.0 in conjunction with the cache cfg file of JVET-K0451.  The cache model is not compiled in by default and has to be enabled by setting #define JVET_J0090_MEMORY_BANDWITH_MEASURE 1 in Lib/CommonLib/TypeDef.h.  Note that VTM-3.0 as is does not compile with this #define enabled, therefore the changes related to commit 6147d36e (“fix for ticket_#121 (JVET-J0090 related)”) has been applied.

For each bit stream, the decoder outputs a total hit ratio when using the cache model.  The following table compares the average hit ratios (in percentage) of the proposed methods with those of VTM-3.0:

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-3.0 | CE10.1.2.a | CE10.1.2.b | CE10.1.2.c | CE10.1.2.d |
| A1 | 99.4888 | 99.4781 | 99.4878 | 99.4858 | 99.4866 |
| A2 | 99.5513 | 99.5474 | 99.5505 | 99.5488 | 99.5505 |
| B | 99.5340 | 99.5259 | 99.5330 | 99.5326 | 99.5345 |
| C | 99.3629 | 99.3593 | 99.3615 | 99.3630 | 99.3634 |
| Overall | 99.4843 | 99.4777 | 99.4832 | 99.4826 | 99.4838 |
| D | 99.3416 | 99.3379 | 99.3412 | 99.3414 | 99.3415 |
| F | 98.9343 | 98.9258 | 98.9267 | 98.9297 | 98.9291 |
|  |  |  | Low Delay B |  |  |
|  | VTM-3.0 | CE10.1.2.a | CE10.1.2.b | CE10.1.2.c | CE10.1.2.d |
| B | 99.6547 | 99.6462 | 99.6531 | 99.6539 | 99.6567 |
| C | 99.5650 | 99.5548 | 99.5594 | 99.5656 | 99.5628 |
| E | 99.5242 | 99.5147 | 99.5187 | 99.5224 | 99.5225 |
| Overall | 99.5813 | 99.5719 | 99.5771 | 99.5806 | 99.5807 |
| D | 99.5715 | 99.5657 | 99.5683 | 99.5727 | 99.5737 |
| F | 99.1095 | 99.0934 | 99.0978 | 99.1081 | 99.1034 |

In the following table, the worst-case hit ratios are shown, i.e. the lowest per-frame cache hit ratio for each configuration and class:

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-3.0 | CE10.1.2.a | CE10.1.2.b | CE10.1.2.c | CE10.1.2.d |
| A1 | 98.81 | 98.60 | 98.69 | 98.79 | 98.77 |
| A2 | 99.04 | 99.03 | 99.06 | 99.04 | 99.06 |
| B | 91.68 | 91.68 | 91.68 | 91.68 | 91.68 |
| C | 97.82 | 97.82 | 97.82 | 97.82 | 97.82 |
| Overall | 91.68 | 91.68 | 91.68 | 91.68 | 91.68 |
| D | 97.91 | 97.73 | 97.73 | 97.73 | 97.73 |
| F | 96.57 | 95.03 | 95.70 | 95.40 | 95.96 |
|  |  |  | Low Delay B |  |  |
|  | VTM-3.0 | CE10.1.2.a | CE10.1.2.b | CE10.1.2.c | CE10.1.2.d |
| B | 97.99 | 98.05 | 97.59 | 97.90 | 97.61 |
| C | 98.16 | 98.16 | 98.16 | 98.16 | 98.16 |
| E | 98.78 | 98.66 | 98.70 | 98.70 | 98.70 |
| Overall | 97.99 | 98.05 | 97.59 | 97.90 | 97.61 |
| D | 98.77 | 98.81 | 98.68 | 98.87 | 98.56 |
| F | 66.67 | 66.67 | 66.67 | 66.67 | 66.67 |

# Syntax changes

The changes to the coding unit syntax structure are shown below:

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| if(cu_skip_flag[ x0 ][ y0 ] = = 0&& slice_type = = B && mh_intra_flag[ x0 ][ y0 ] = = 0 && merge_triangle_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight > 64 && cbWidth >= 8 && cbHeight >= 8) |  |
| mh_pred_data( x0, y0, cbWidth, cbHeight ) |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] == PRED_BI && cbWidth * cbHeight > 64 && cbWidth >= 8 && cbHeight >= 8) |  |
| mh_pred_data( x0, y0, cbWidth, cbHeight ) |  |
| if( sps_amvr_enabled_flag && inter_affine_flag = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) |  |
| amvr_mode[ x0 ][ y0 ] | ae(v) |
| if( sps_gbi_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && cbWidth * cbHeight >= 256 ) |  |
| gbi_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

| mh_pred_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| i = 0 |  |
| readMore = 1 |  |
| while( i < MaxNumAdditionalHypotheses && readMore ) { |  |
| additional_hypothesis_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( additional_hypothesis_flag[ x0 ][ y0 ][ i ] ) { |  |
| ref_idx_add_hyp[ x0 ][ y0 ][ i ] | ae(v) |
| mvd_coding( x0, y0, 2+i, 0 ) |  |
| mvp_add_hyp_flag[ x0 ][ y0 ][ i ] | ae(v) |
| add_hyp_weight_idx[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| readMore = additional_hypothesis_flag[ x0 ][ y0 ][ i ] | ae(v) |
| i++ |  |
| } |  |
| } |  |

additional_hypothesis_flag[ x0 ][ y0 ][ i ] equal to 1 specifies that the i-th explicit additional inter prediction hypothesis is signaled for this CU.

ref_idx_add_hyp[ x0 ][ y0 ][ i ] specifies the reference frame to be used for the i-th explicit additional inter prediction hypothesis.

mvp_add_hyp_flag [ x0 ][ y0 ][ i ] specifies the motion vector predictor index to be used for the i-th explicit additional inter prediction hypothesis.

add_hyp_weight_idx[ x0 ][ y0 ][ i ] specifies the weighting factor αi to be used for the i-th explicit additional inter prediction hypothesis, according to the following mapping:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

# Conclusion

This contribution reports results for tests CE10.1.2.a – CE10.1.2.d on multi-hypothesis inter prediction with up to 4 motion-compensated prediction signal per block.

For test CE10.1.2.a, the following results are reported for this test relative to VTM-3.0:

RA: 	-0.29% (Y),	-0.16% (Cb), -0.12% (Cr)	107% enc. time		97% dec. time

LB:		-0.41% (Y), -0.04% (Cb), -0.04% (Cr)	110% enc. time		99% dec. time

For test CE10.1.2.b, the additional prediction parameters are not re-used for merging.  The following results are reported for this test relative to VTM-3.0:

RA:	-0.20% (Y), -0.02% (Cb), -0.05% (Cr)	107% enc. time		96% dec. time

LB:		-0.30% (Y),	-0.10% (Cb), -0.17% (Cr)	111% enc. time		102% dec. time

For test CE10.1.2.c, only 2 different reference pictures can be used within each PU.  The following results are reported for this test relative to VTM-3.0:

RA:	-0.20% (Y), -0.10% (Cb), -0.08% (Cr)	105% enc. time		97% dec. time

LB:		-0.20% (Y), -0.19% (Cb), -0.24% (Cr)	107% enc. time		100% dec. time

For test CE10.1.2.d, the additional predictions are only used for the luma component.  The following results are reported for this test relative to VTM-3.0:

RA:	-0.29% (Y), 0.03% (Cb), 0.04% (Cr)		107% enc. time		96% dec. time

LB:		-0.40% (Y),	0.06% (Cb), -0.02% (Cr)	110% enc. time		99% dec. time

# References

- C.-W. Hsu, M. Winken, “Description of Core Experiment 10: Combined and multi-hypothesis prediction,” JVET-L1030, Macao, CN, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring (editors), “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, October 2018.

- M. Zhou, B. Heng, “Non-CE9: A computational complexity analysis for DMVR,” JVET-K0480 Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Fraunhofer may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).