| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0452 |
| --- | --- |

| Title: | AHG8: Hemisphere cubemap projection format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Maxym Dmytrychenko | Email: | jill.boyce@intel.com maxym.dmytrychenko@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

A hemisphere cubemap projection format is proposed to represent hemisphere 180⁰ x 180⁰ omnidirectional images or video, such as those captured by a single fisheye lens.  The proposed hemisphere cubemap format, called HCMP, contains one full cube face and four half cube faces, representing the hemisphere. The faces and half faces are arranged in a different, more compact layout than used in the existing CMP format, in order to reduce the picture size, by eliminating inactive regions. The proposed layout can be used with any cube-based projection format, and is proposed for a hemispherical version of the Equi-Angular Cubemap (EAC) format, called HEAC.  It is proposed to add the new HCMP and HEAC formats to the 360Lib software.

# Introduction

360⁰ x 180⁰ omnidirectional content can be captured using two back-to-back fisheye camera lenses, and stitching together the content. A single fisheye camera lens can capture 180⁰ x 180⁰ content, representing a single hemisphere, instead of the full sphere.

Some of content being used in MPEG’s 3DoF+ project in MPEG-I Visual is 180⁰ x 180⁰ content in an ERP format, e.g. the TechnicolorHijack sequence.

OMAF has defined a fisheye format, and JCT-VC is developing a fisheye projection format SEI message, which enables coding of fisheye content in its native captured format. However, it is expected that fisheye format is not a bitrate efficient format for video encoding, because of image warping. Unfortunately, there is no software available for the fisheye format under consideration by JCT-VC that implements the fisheye projection equations.

Figure 1 shows an example cubemap format image, from the Trolley sequence. Figure 2 indicates the position of the faces of the cube in the CMP format layout. Figure 3 shows the ERP version of the Trolley sequence. Figure 4 shows a cropping of the 180⁰ x 180⁰ content in the center, with a black background, in the ERP format. Figure 5 shows a semi-ERP format, where only the active region is retained, as is used in the 3DoF+ TechnicolorHijack sequence.

Figure 6 shows a mapping of the center cropped 180⁰ x 180⁰ content to the CMP format, with grey used in the inactive areas. Figure 7 shows an example fisheye version of the content, with black background in the inactive area.

Figure 1. CMP format Trolley

Figure 2. Face layout in CMP format

Figure 3. ERP format Trolley

Figure 4. 180⁰ x 180⁰ cropped Trolley, in ERP format

Figure 5. 180⁰ x 180⁰ cropped Trolley, in Semi-ERP format

Figure 6. 180⁰ x 180⁰ cropped Trolley, in CMP format, with gray in the inactive region.

Figure 7. 180⁰ x 180⁰ cropped Trolley Trolley sequence converted to fisheye format

# Proposed format

A 180⁰ x 180⁰  hemisphere can be projection mapped to half of a cube, with active content in one full face (front), and four half faces (top, left, right, bottom), as illustrated by the example in Figure 6.

In the proposed HCMP format, these full and half faces are arranged into a compact area, avoiding the inactive regions if the CMP format were used. Neighbor continuity in the hemi-sphere is maintained where possible in the layout, with a single continuous region across the half left face, front face, and half right face. The half top face and half bottom face are rotated and arranged on either side of the single continuous region, with active padding added at these discontinuous boundaries, which was copied (and rotated) from the corresponding neighboring location in the picture. Figure 8 illustrates an example mapping for the Trolley sequence from semi-ERP format to HCMP. Figure 9 shows the arrangement of cube faces and half faces and padding within the HCMP format.

Figure 8. HCMP Example

Figure 9. HCMP Layout

The proposed layout is also applicable to other cube-based formats, such as EAC and HEC. The proposed modified EAC format is referred to as HEAC.

# Experiments

## Experimental conditions

The JVET 360 CTC sequences were cropped in the center to create 180⁰ x 180⁰ sequences, such as in the example in Figure 4, and treated as the original sequences. Sequences with 8192 x 4096 resolution were cropped to 4096 x 4096 resolution. All other stages in the JVET 360 CTC were followed, except for halving the horizontal resolution. HM 16.15/360Lib-3.0 and VTM 3.0 were used. The conditions of JVET-K1012-v1 were adapted for a hemisphere format: for ERP a face size of 2216x2216 is used, and for HCMP/HEAC face sizes are 1280x1280. In addition, HCMP used 8pixels width of padding for each one, therefore total width calculated as: 3 * 1280 (faces) + 2 * 8 (padding) = 3856 pixels.

The 360Lib software was modified to support 180⁰ x 180⁰ content, and support was added for the proposed HCMP and HEAC formats:

- New projection formats conversation to/from 360⁰ normalized sphere

- 2d yuv layout read/write

- 360Lib’s config file support extended to support half faces,

SourceFPStructure / CodingFPStructure parameters, padding control defined per: CodingPCMP/ InputPCMP

- WS-PSNR weight table defined as shown in Figure 10

Figure 10. WS-PSNR weights for HCMP format

## Experimental results

Table 1 shows the RA BD-rate results for HM16.15 for the hemisphere sequences for the HCMP format vs. semi-ERP. For comparison purposes, the CMP vs PERP results for the sequences are also provided in the table.  Table 2 shows the RA BD-rate results for HM16.15 for the hemisphere sequences for the HEAC format vs. semi-ERP. For comparison purposes, the EAC vs PERP results for the sequences are also provided in the table.

The per-sequence coding gain results for the hemisphere formats are similar to but not as good as the corresponding full spherical formats. It is not expected that the formats would have identical gains, because the region of the coded picture differs for the cropped hemisphere from the full sphere.

Table 1.HM16.15 HCMP vs. Semi-ERP and CMP vs PERP comparison

|  | WS-PSNR (End to End) |  |
| --- | --- | --- |
|  | HCMP vs. Semi-ERP | CMP vs. PERP |
| Trolley | -1.7% | -2.6% |
| GasLamp | 2.1% | 2.3% |
| Skateboarding_in_lot | -6.6% | -7.5% |
| Chairlift | -21.7% | -18.4% |
| KiteFlite | -4.0% | -5.2% |
| Harbor | 3.2% | 1.0% |
| Balboa | -0.7% | -3.3% |
| Broadway | 4.9% | 1.5% |
| Landing2 | 10.4% | 3.3% |
| BranCastle2 | -4.1% | -1.2% |
| Overall | -1.8% | -2.4% |
| 8K | -4.8% | -5.1% |
| 6K | 2.6% | 0.0% |

Table 2.HM16.15 HEAC vs. Semi-ERP and EAC vs PERP comparison

|  | WS-PSNR (End to End) |  |
| --- | --- | --- |
|  | HEAC vs. Semi-ERP | EAC vs. PERP |
| Trolley | -6.8% | -8.1% |
| GasLamp | -3.4% | -4.4% |
| Skateboarding_in_lot | -15.9% | -17.1% |
| Chairlift | -27.8% | -25.5% |
| KiteFlite | -7.2% | -10.3% |
| Harbor | -5.4% | -9.3% |
| Balboa | -7.3% | -10.3% |
| Broadway | -4.5% | -7.9% |
| Landing2 | 2.0% | -3.8% |
| BranCastle2 | -11.4% | -9.9% |
| Overall | -8.8% | -10.7% |
| 8K | -11.1% | -12.5% |
| 6K | -5.3% | -8.0% |

Table 3 shows the RA BD-rate results for VTM3.0 for the hemisphere sequences for the HCMP format vs. semi-ERP. Table 4 shows the RA BD-rate results for VTM3.0 for the hemisphere sequences for the HEAC format vs. semi-ERP. The gains are similar for VTM3.0 and for HM16.15 for EAC, averaging 10.8% bitrate reduction for VTM3.0 for the 8K sequences vs 11.1% bitrate reduction for the HM16.15.

Table 3. VTM 3.0 HCMP vs. Semi-ERP

| VTM 3.0 | WS-PSNR (End to End) |
| --- | --- |
|  | HCMP vs. Semi-ERP |
| Trolley | -3.1% |
| GasLamp | 0.5% |
| Skateboarding_in_lot | -7.5% |
| Chairlift | -10.1% |
| KiteFlite | -3.8% |
| Harbor | 2.6% |
| Balboa | 1.5% |
| Broadway | 5.9% |
| Landing | 9.3% |
| BranCastle | -3.5% |
| Overall | -0.8% |
| 8K | -3.6% |
| 6K | 3.3% |

Table 4. VTM 3.0 HEAC vs. Semi-ERP

| VTM 3.0 | WS-PSNR (End to End) |
| --- | --- |
|  | HEAC vs. Semi-ERP |
| Trolley | -9.1% |
| GasLamp | -5.8% |
| Skateboarding_in_lot | -17.3% |
| Chairlift | -16.9% |
| KiteFlite | -8.3% |
| Harbor | -7.7% |
| Balboa | -5.0% |
| Broadway | -3.8% |
| Landing | 2.9% |
| BranCastle | -10.6% |
| Overall | -8.2% |
| 8K | -10.8% |
| 6K | -4.1% |

## Fisheye comparison

An informal comparison was made of the proposed HEAC format with natively coding a fisheye format, only for the Harbor sequence. Conversion of the cropped ERP sequences to fisheye format was done using the Hugin open source project, with an intermediate conversion to RGB, and a resolution of 2048x2048. The precise fisheye projection equations used in Hugin are unknown. There is currently no support for fisheye format in the 360Lib software. The authors are not aware of the availability of any software supporting conversions to or from fisheye format in JVET, JCT-VC, or their parent bodies.

Table 5 shows that the HEAC format saves 78.6% bitrate vs. the Fisheye format. Because of the multiple intermediate conversion steps required which could introduce some coding loss, this number can only be used as a rough estimate. However, it does provide some evidence to confirm the expectation that the fisheye format is not an efficient format for use in video coding, because of how it deforms the image.

Table 5. HEAC vs. Fisheye format

|  | WS-PSNR (End to End) |
| --- | --- |
|  | HEAC vs. Fisheye |
| Harbor | -78.6% |

Tools used for fisheye conversion:

- HDRConvert, version: V.0.18 / https://gitlab.com/standards/HDRTools

to convert .yuv to/from .tiff file formats.

- Hugin, version: 2018.0.0.5abfb4de7961 / http://hugin.sourceforge.net

for fisheye projection change (ERP to/from fisheye projection changes).

- Convert, version: ImageMagick 7.0.8-20 Q16 x64 2018-12-24 / http://www.imagemagick.org

to fix .tiff related problems after hugin’s support of .tiff picture format

# Conclusion

It is proposed to add support to 360Lib for 180⁰ x 180⁰ content in the HCMP and HEAC formats.

Use of cubemap-based formats such as EAC has previously been shown to provide a significant bitrate reduction vs ERP format for 360⁰ x 180⁰ spherical content. This contribution shows similar benefits for 180⁰ x 180⁰ hemispherical content, such as captured using a single fisheye lens camera.

Using the proposed layout requires smaller picture sizes vs. using the existing CMP or EAC format, which could enable using a lower level to code a hemispherical video sequence.

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).