| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0454-v1 |
| --- | --- |

| Title: | CE10-related: Multi-Hypothesis Intra with Weighted Combination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andre Seixas Dias Gosala Kulupana Saverio Blasi 56 Wood Lane, Centre House, Block D W12 7SB, London, UK | Tel: Email: | andre.seixasdias@bbc.co.uk gosala.kulupana@bbc.co.uk saverio.blasi@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution presents a modification to the Multi-Hypothesis Intra tool included in VTM 3.0. In this contribution, an Intra prediction can be combined with a merge Inter prediction using a weighted average. To allow this weighted combination of Intra and Inter predictions, additional syntax elements are signalled indicating if unequal weights are used and if so, which pair of weights is needed. This contribution is built on top of the method in JVET-L0375, where only Planar mode is allowed for Intra prediction when used in combination with an Inter prediction. Average BD-rates of -0.11% (Y), 0.00% (U) and -0.01% (V) are reported for RA with 103% encoding time and 100% decoding time with respect to VTM 3.0. For LD, the reported BD-rates are 0.05% (Y), 0.21% (U) and 0.19% (V) with 102% and 101% encoding and decoding times, respectively.

# Introduction

In VTM 3.0, Intra and Inter predictions can be combined to generate a new prediction [1]. This method, referred to as Multi-Hypothesis Intra (MH Intra), allows a given Inter merge prediction to be combined with an Intra prediction of the same block. Any merge candidate prediction can be combined with one of 4 different Intra prediction modes (Planar, DC, Pure Vertical or Pure Horizontal). In VTM 3.0, when an Inter merge prediction is combined with Intra modes Planar and DC, the combined prediction is given by the average of the two sources of prediction. When Intra modes Pure Vertical and Pure Horizontal are used in the combination, a weighted average is used to combine the two sources of prediction and different weights are used for different parts of the block being predicted, according to the Intra mode used. Regarding signalling, whenever merge mode is selected, a flag is sent indicating if MH Intra is used and in case it is, additional signalling specifies which of the 4 Intra prediction modes is used in the combination.

In this contribution, the Intra prediction used in the MH Intra combination is restricted to Planar mode. This corresponds to the configuration being tested in test CE10.1.3.a [2], proposed in [3]. Additionally, the proposed contribution allows a weighted combination of the two prediction sources. This is potentially advantageous as simply performing the average of each sample may not be ideal. Depending on the accuracy of either the Intra- or Inter-predicted candidate, there may be many cases in which one of the two predictions may be better suited to represent the content of the current block, but still the other prediction can provide some missing information which would improve the final prediction. In such cases, performing weighted prediction of the two candidates can result in a better combined prediction.

# Proposed method

In this contribution, in order to combine the Inter merge prediction with an Intra prediction (only Planar mode allowed), 3 possible pairs of weights can be used. Once the pair of weights to be used in a given block is determined, all samples within the block are then predicted using the same combined prediction obtained with such weights. The combined prediction Pcombi,j is given by

Pcombi,j=(WInter∙PInteri,j+WIntra∙PIntrai,j)≫3

where PInteri,j and PIntrai,j are the Inter and Intra prediction sources and WInter and WIntra are the weights associated with each prediction. The allowed pairs of weights {WInter, WIntra} are {4, 4}, {6, 2} and {2, 6}.

When the MH Intra mode is used, an additional binary flag is signalled to the decoder indicating if the weights used in the Inter + Intra combination are the same or not (unequalWeightFlag). In case the unequalWeightFlag is 0, no additional signalling is required and equal weights {4, 4} are used in the combination. If unequalWeightFlag is 1, an additional binary flag (UnequalWeight) is sent indicating which pair of weights is used in the combination. Table 1 summarises the extra signalling required by the proposed method.

Table 1- Summary of the signalling required by the proposed method.

| UnequalWeightFlag | UnequalWeight | WInter | WIntra |
| --- | --- | --- | --- |
| 0 | - | 4 | 4 |
| 1 | 0 | 6 | 2 |
| 1 | 1 | 2 | 6 |

It is important to emphasize that, since only the Planar mode is allowed for the Intra prediction in MH Intra, the mh_intra_luma_mpm_flag and the mh_intra_luma_mpm_idx syntax elements currently needed in VTM3.0 are not required by the proposed method. The proposed changes to the relevant part of the draft specification text are highlighted below.

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| mh_intra_unequal_weight_flag[ x0 ][ y0 ] | ae(v) |
| if(mh_intra_unequal_weight_flag[ x0 ][ y0 ] ) |  |
| mh_intra_unequal_weight[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

# Performance results

The performance of the proposed contribution is summarised in Table 2 and Table 3. The tests were performed using the JVET common test conditions [4] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 3.0, compiled with gcc 4.8.2. All experiments were run using AVX.

Table 2 - Performance of the proposed weighted MH Intra method for RA.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.19% | -0.14% | -0.08% | 105% | 101% |
| Class A2 | -0.10% | 0.03% | 0.05% | 103% | 102% |
| Class B | -0.12% | 0.09% | -0.04% | 102% | 100% |
| Class C | -0.05% | -0.03% | 0.05% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | 0.00% | -0.01% | 103% | 100% |
| Class D | -0.04% | -0.12% | -0.07% | 103% | 101% |
| Class F | -0.10% | -0.12% | -0.08% | 102% | 99% |

Table 3 - Performance of the proposed weighted MH Intra method for LD.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.20% | 0.40% | 102% | 102% |
| Class C | 0.09% | 0.30% | 0.45% | 102% | 101% |
| Class E | 0.03% | 0.11% | -0.50% | 100% | 100% |
| Overall | 0.05% | 0.21% | 0.19% | 102% | 101% |
| Class D | 0.05% | 0.38% | 0.53% | 102% | 100% |
| Class F | -0.06% | 0.13% | -0.30% | 102% | 100% |

# Conclusions

In this contribution, a weighted combination of the Intra and Inter predictions used in the MH Intra mode in VTM 3.0 is proposed. The proposed contribution limits the Intra mode used in MH Intra to be predicted using Planar mode and allows 3 different weighting options to combine an Inter merge prediction with an Intra prediction. No additional complexity is observed at the decoder.

# References

- M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode”, in JVET-L0100, October 2018.

- C.-W. Hsu, M. Winken “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction”, in JVET-L1030, October 2018.

- W. Xu, H. Yang, Y. Zhao, J. Chen “CE10-related: Inter prediction sample filtering”, in JVET-L0375, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring “JVET common test conditions and software reference configurations for SDR video”, in JVET-L1010, October 2018.

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).