| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0475-v1 |
| --- | --- |

| Title: | CE3: Multiple neighbour LM (Test 3.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu Yao-Jen Chang | Email: | HongJheng@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of multiple neighbour-based linear model (MNLM) in CE3-2.2. On top of CCLM +MDLM, it enables 3 MMLM modes with different neighbouring sets for model derivation: MMLM with Above & Left neighbouring data, MMLM with Above-only neighbouring data, and MMLM with Left-only neighbouring data. The MaxMin derivation for model parameter derivation is also applied to the proposed MNLM. On top of VTM, AI results show -0.29%/-1.94%/-2.51% YUV-BDrate with 100/100% coding time change, and RA results show -0.15%/-1.70%/-1.99% YUV-BDrate with 101/100% coding time change.

Introduction

In JVET-L0388 [1], the multiple neighbor-based linear model (MNLM) uses multiple neighbor sets for MMLM to cover various linear relationships between luma samples and chroma samples in CU. It was included into Core Experiment (CE) 3 [2] to evaluate the performance on top of common test conditions [3]. In the test, the MaxMin derivation for model parameter derivation is also applied to the proposed MNLM. One MMLM flag and an index signaling are used to indicate the MMLM mode in decoder syntax. Two lines above are used for model derivation, i.e., 4 lines above and 5 lines left of the current block for luma, and 2 lines above and left of the current block for chroma, and when the top boundary of the current block shares the CTU boundary, only one luma line is used from above the block. The following table shows the description for CE test reported in this document:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.2 | Multiple neighbour LM (MNLM) (CCLM + MDLM + MMLM + Above-MMLM + Left-MMLM) | Hong-Jheng Jhu (Foxconn) | Adarsh K. Ramasubramonian (Qualcomm) |

Simulation results

The following experiments were conducted using the VTM-3.0 software with the VTM configuration and the JVET common test conditions [2] to collect the B-D rate, encoding time and decoding time of the proposed techniques in the All Intra and Random Access configurations. The proposed method has a BD-rate performance of (−0.29%, −1.94%, −2.51%) for (Y, Cb, Cr) with 100% encoding time and 100% decoding time for the AI configuration, and BD-rate performance of (−0.15%, −1.70%, −1.99%) for (Y, Cb, Cr) with 101% encoding time and 100% decoding time for the RA configuration.

Table 2: Simulation results of the proposed technique

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -2.89% | -4.88% | 99% | 100% |
| Class A2 | -0.14% | -1.27% | -1.10% | 102% | 100% |
| Class B | -0.22% | -2.33% | -2.78% | 100% | 100% |
| Class C | -0.30% | -2.21% | -2.76% | 101% | 100% |
| Class E | -0.02% | -0.64% | -0.75% | 100% | 100% |
| Overall | -0.29% | -1.94% | -2.51% | 100% | 100% |
| Class D | -0.09% | -1.66% | -1.72% | 101% | 100% |
| Class F | -0.41% | -2.08% | -2.75% | 99% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -2.34% | -3.68% | 101% | 100% |
| Class A2 | -0.06% | -0.98% | -0.66% | 100% | 100% |
| Class B | -0.07% | -1.96% | -2.26% | 100% | 100% |
| Class C | -0.04% | -1.44% | -1.40% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -1.70% | -1.99% | 101% | 100% |
| Class D | -0.02% | -1.60% | -1.57% | 101% | 100% |
| Class F | -0.17% | -1.49% | -1.91% | 101% | 100% |

Conclusions

This contribution shows the experimental results of Test 2.2. The results show that the test provides significant improvement on coding efficiency in terms of luma and chroma BD-rate with negligible increase in EncT/DecT. It is suggested to adopt the proposed tool as part of VVC.

Reference

- H.-Y. Jiang, H.-J. Jhu, and Y.-J. Chang, “CE3: Multiple neighbor-based linear model (Test 5.3.1),” JVET-L0388, the 12th JVET meeting, Oct. 2018, Macao, CN.

- G. Van der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-L1023, the 12th JVET meeting, Oct. 2018, Macao, CN.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

Acknowledgements

We would like to thank Qualcomm for cross-checking this CE test.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).