| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0487 |
| --- | --- |

| Title: | CE9.1: Simplifications on bi-directional optical flow (BDOF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA Chen-Yen Lai, Yu-Chi Su, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | 1-858-210-4819 Yuwen.He@InterDigital.com +886-3-5670766 {jenny.lai, yu-chi.su, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | InterDigital Communications, Inc. MediaTek Inc |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment (CE) 9.1 on simplifications on bi-directional optical flow (BDOF). In this CE test, three solutions are tested to reduce the BDOF computational complexity by reducing or skipping the interpolation of prediction samples in the extended region of one BDOF CU. Compared to VTM-3.0 anchor, the performance of the proposed BDOF solutions are summarized as follows:

Solution one: {Y, U, V} BD-rates {-0.01%, 0.03%, 0.02%}, EncT=103%, DecT=102%

Solution two: {Y, U, V} BD-rates {0.05%, 0.02%, 0.02%}, EncT=99%, DecT=98%

Solution three: {Y, U, V} BD-rates {0.10%, 0.03%, 0.01%}, EncT=99%, DecT=98%

# Introduction

Bi-directional optical flow (BDOF) is one coding tool that is adopted into the VVC at Macao meeting [1]. The BDOF tool was designed to enhance the conventional bi-prediction by improving both the granularity and the accuracy of the motion vectors (MVs) that are used at the motion compensation. Although the BDOF can improve the coding efficiency, it also increases computational complexity, due to the interpolation of additional prediction samples in one extended region around one CU for the BDOF derivation.

In [2], three solutions are proposed to reduce the BDOF complexity by reducing or skipping the interpolation of predication samples in the extended regions around the boundaries of one BDOF CU. The three solutions were included in Core Experiment (CE) 9 as test 1.1 for further evaluation [3]. This document reports the simulation results of the CE test.

# Technical details

The main aspects of the proposed three solutions are described as follows.

- Solution One:

In the first solution, the extended samples in one single row/column on each CU boundary, i.e., (W+2)x(H+2), are involved in the BDOF derivation. Additionally, the same 8-tap interpolation filters that are used to generate the prediction samples within the CU are applied for interpolating the extended samples. Moreover, the extended samples are only used for calculating gradients of the prediction samples on the CU boundaries. During the BDOF refinement, the method sets both the sample values and the gradients of the positions outside the CU area (as indicated by the blank squares in Figure 1) to the values of their nearest neighbors on CU boundaries. Figure 1 illustrates the idea of the proposed BIO derivation method.

Figure 1 Illustration of the BIO simplification method (Solution One)

- Solution Two:

To further reduce the complexity, in the second solution, it is proposed to directly use integer reference samples (without interpolation) as the extended samples (as indicated by the blank squares in Figure 1) in the extension region (W+2)×(H+2) for the BDOF derivation. Additionally, as the same as the first solution, the second solution pads both the samples and gradients of the position in the extended region from their nearest neighbors on CU boundaries.

- Solution Three:

In the third solution, to avoid the interpolation of the extended samples, it is proposed to only use the prediction samples that are inside the current CU (as indicated by the shadow squares in Figure 1) for the BDOF derivation. However, because the calculation for each sample’s gradients needs both the sample values of its left/right or top/bottom neighbors, different methods are applied to derive the gradients of samples on the CU boundaries and the samples inside the CU. Specifically, with the third solution, the horizontal and vertical gradients used by the BDOF are calculated as

| ∂Ik∂xi, j=Iki+1, j-Iki-1, j≫40<i<W-1Iki+1, j-Iki, j≫3i=0Iki, j-Iki-1, j≫3i=W-1 | (5) |
| --- | --- |
| ∂Ik∂yi, j=Iki, j+1-Iki, j-1≫40<j<H-1Iki, j+1-Iki, j≫3j=0Iki, j-Iki, j-1≫3j=H-1 | (6) |

where Iki, j, ∂Ik∂xi, j and ∂Ik∂yi, j are the sample values, the horizontal and vertical gradients of the sample at coordinate i, j of the prediction signal in list k, k=0,1. Meanwhile, as the same as the other two solutions, the third solution pads the samples and gradients of the position on the CU boundaries to the extended region to derive the BIO refinement of the current CU.

# Simulation results

The proposed solutions were implemented based on the VTM-3.0 reference software [4]. The test condition and evaluation criteria that are used in CE9 [3] are applied.

Table 1. The performance of Solution One, compared to VTM-3.0 anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.01% | 104% | 101% |
| Class A2 | -0.02% | 0.04% | 0.05% | 104% | 103% |
| Class B | -0.01% | 0.01% | 0.01% | 103% | 103% |
| Class C | 0.00% | 0.01% | 0.02% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.02% | 103% | 102% |
| Class D | 0.01% | -0.06% | 0.05% | 103% | 103% |

Table 2. The performance of Solution Two, compared to VTM-3.0 anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.01% | 99% | 98% |
| Class A2 | 0.02% | 0.03% | 0.04% | 99% | 98% |
| Class B | 0.05% | 0.01% | 0.00% | 98% | 98% |
| Class C | 0.09% | 0.02% | 0.04% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.02% | 0.02% | 99% | 98% |
| Class D | 0.24% | 0.01% | 0.14% | 98% | 97% |

Table 3. The performance of Solution Three, compared to VTM-3.0 anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.00% | 99% | 98% |
| Class A2 | 0.06% | -0.01% | 0.01% | 99% | 98% |
| Class B | 0.10% | 0.02% | -0.05% | 99% | 98% |
| Class C | 0.19% | 0.09% | 0.09% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.03% | 0.01% | 99% | 98% |
| Class D | 0.39% | 0.16% | 0.18% | 98% | 97% |

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001, October 2018, Macao, China.

X. Xiu, Y. He, Y. Ye, C,-Y. Lai, Y.-C. Su, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE9-related: A simplified design of bi-directional optical flow”, JVET-L0591, October 2018, Macao, China.

X. Xiu, S. Esenlik, “Description of Core Experiment 9 (CE9): Decoder-side motion derivation”, JVET-L1029, October 2018, Macao, China.

VTM-3.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).