| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0491 |
| --- | --- |

| Title: | CE7-related: Reduced maximum number of context-coded bins for transform coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Gao, Xiang Li, Xin Zhao, Shan Liu | Tel: Email: | maxwellgao@tencent.com xlxiangli@tencent.com xinzzhao@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In the transform coefficient coding of VTM-3.0, the averaged maximum number of context-coded bins per 4x4 sub-block is 32. In this contribution, to further reduce the averaged maximum number of context-coded bins per 4x4 sub-block, it is proposed to reduce the maximum number of context-coded bins for 4x4 sub-blocks in high frequency region such that the averaged maximum number of context-coded bins per 4x4 sub-block is 30. It is reported that xxx, xxx and xxx luma BD-rate increase over VTM-3.0 is observed for AI, RA and LD configurations, respectively.

Introduction

In VVC, the maximum number of context-coded bins is constrained to 32 for each 4x4 sub-block regardless of the frequency position of the 4x4 sub-block [1], so the averaged maximum number of context-coded bins per 16 samples is 32. Since the number of the context-coded bins in high frequency region is usually smaller than that in low frequency region, this contribution proposes to reduce the maximum number of context-coded bins for 4x4 sub-blocks in high frequency region such that the averaged maximum number of context-coded bins per 16 samples is reduced from 32 to 30.

Proposed method

To limit the worst-case number of context-coded bins, the maximum number of context-coded bins maxCtxBinsNum for 4x4 sub-block in TU is determined as follows:

If (CGX + CGY < CGDiag)

maxCtxBinsNum = 32

Else If(CGX + CGY > CGDiag)

maxCtxBinsNum = 28

Else

maxCtxBinsNum = 30

where (CGX, CGY) is the 4x4 sub-block position and CGDiag is calculated as follows with TU width and height:

maxCGX = (width >> 2) – 1;

maxCGY = (height >> 2) – 1;

CGDiag = (maxCGX + maxCGY) >> 1

Figure 1 illustrates an example for 8x16 and 16x16 TU. In this example, TU is grouped into 3 sets by CGDiag: The first group consists of sub-blocks in the top-left region (white region), the second group consists of sub-blocks in middle region (red region) and the third group consists of sub-blocks in the bottom-right region (grey region). The maximum number of context-coded bins for sub-blocks in the first group is 32, the maximum number of context-coded bins for sub-blocks in second group is 30, and the maximum number of context-coded bins for the sub-blocks in the third group is 28. So, the average maximum number of context-coded bins per 4x4 sub-block is 30.

Figure 1. Illustration of grouping 4x4 sub-blocks in 16x16 TU.

Simulation results

The proposed method is implemented on the top of VTM-3.0. The common test conditions defined in [2] is used in the simulation. Table 1 shows the BD-Rate of the proposed method relative to VTM-3.0. Please note that the running time in Table 1 is not reliable due to system issues.

Table. 1. The BD-Rate of the proposed method relative to VTM-3.0.

Conclusion

This contribution proposes to reduce the averaged maximum number of context-coded bins per 4x4 sub-blocks from 32 to 30 by reducing the maximum number of context-coded bins for 4x4 sub-blocks in high frequency region. It is reported that xxx, xxx and xxx luma BD-rate increase over VTM-3.0 is observed for AI, RA and LD configurations, respectively.

Reference

[1] H. Schwarz, T. Nguyen, D.Marpe, T. Wiegand, M. Karczewicz, M. Coban, J. Dong, JVET-L0274, “CE7: Transform coefficient coding with reduced number of regular-coded bins ”, Oct. 2018.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, JVET-L1010, “JVET common test conditions and software reference configurations for SDR video”, Oct. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).