| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0499 |
| --- | --- |

| Title: | CE6: RQT-like transform sub-block splitting (Test 6.4.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-4.3 on RQT-like transform sub-block splitting. With the proposed RQT scheme, the transform block can be only split once (maximum transform depth is 1), and splitting type is implicitly derived based on the block shape. For inter prediction residuals, transform types are also implicitly derived for each sub-block based on its relative location inside a coding block. It is reportedly shown that, on top of VTM-3.0, average -0.30%, -0.42% and -0.45% luma BD-Rate is achieved under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively.

# Introduction

In VVC Draft 3 [1], coding blocks which have width and/or height being larger than 64 are further split horizontally and/or vertically into multiples of smaller blocks with width and/or height equal to 64, and transform is performed on each sub-block. Unlike RQT in HEVC, the transform split is performed implicitly and signaling is not needed.

# Proposed method

An RQT-like transform splitting scheme is proposed for coding blocks with height and width smaller than or equal to 64. Unlike RQT in HEVC, the proposed method allows transform split only once (maximum transform depth is 1), and both quad-tree splitting and binary-tree splitting types are used.

## Transform splitting type

Totally three transform splitting types are supported, including quad-tree split (QT), horizontal binary tree split (HBT), and vertical binary tree split (VBT). The selection of transform splitting type is decided based on block width (W) and height (H), as listed in Table 1.

Table 1: Selection of transform split type based on block shape

|  | Square block | Horizontal block (W > H) | Vertical block (H > W) |
| --- | --- | --- | --- |
| Transform split type | QT | VBT | HBT |
| Sub-block size | 0.5W × 0.5H | 0.5W × H | W × 0.5H |

## Sub-block transform type

For inter prediction residuals, if transform splitting is not used, then only DCT-2 is applied. Otherwise, according to the relative location (i.e., top-left (TL), top-right (TR), bottom-left (BL), bottom-right (BR), left (L), right (R), top (T) and bottom (B)) of sub-block inside a coding block, the horizontal and vertical transform types are implicitly selected for each sub-block, as described in the following Table 2.

Table 2: Selection of transform type

| Transform Split type | Relative location | Horizontal transform | Vertical transform |
| --- | --- | --- | --- |
| QT | top-left | DCT-8 | DCT-8 |
|  | top-right | DST-7 | DCT-8 |
|  | bottom-left | DCT-8 | DST-7 |
|  | bottom-right | DST-7 | DST-7 |
| VBT | left | DCT-8 | DST-7 |
|  | right | DST-7 | DST-7 |
| HBT | top | DST-7 | DCT-8 |
|  | bottom | DST-7 | DST-7 |

For intra prediction residual, transform type is selected using MTS for each sub-block.

## Transform split signaling

The similar syntax design of RQT in HEVC is used with further restriction that maximum transform depth is limited to 1. In addition, the CBF signaling for the last sub-block partition is avoided if it is redundant, e.g., the root_cbf is 1 but all previous partitions are coded with zero CBF flag.

# Results

Experiments are conducted under CTC condition [2] for the following test as defined in Core Experiment 6 (CE6) [3],

Table 3: Descriptions on CE6 Test 6.4.3

| Test # | Description |
| --- | --- |
| 6-4.3a | RQT-like concept of transform sub-block splitting |

Detailed results are reported in Table 4.

Table 4: Coding performance of Test 6.4.3

# Conclusions

In this contribution, an RQT-like transform sub-block splitting scheme is proposed. With the proposed method, the transform splitting type is implicitly derived based on the block shape and the transform type is also implicitly decided for each sub-block according to the relative position inside the coding block. It is recommended to further study RQT-like transform scheme in the next Core Experiment.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling,” JVET-L1026, 12th meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).