| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0504 |
| --- | --- |

| Title: | CE3: adaptive multiple cross-component linear model(Test 3.2.3 ) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sheng-Po Wang, Chang-Hao Yau1, Chun-Lung Lin2 and Ching-Chieh Lin3 2870 Zanker Road, Suite 140, San Jose, CA 95134, USA | Email: | +886-3-5915059 changhao.yau@itri.com1 chunlung@itri.com2 jacklin@itri.com3 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

This contribution presents the experimental results of adaptive multiple cross-component linear model under test 2.3 in CE3. This method was proposed in JVET-K0536. The results show that the proposed grouping mechanism provides 0.12% and 0.07% coding gain for Cb, Cr respectively while cause 0.01% loss for luma with VTM all intra configuration and 0.00%, 0.03% and 0.01% coding loss with VTM random access configuration.

Introduction

An adaptive multiple linear model mechanism for cross component prediction is proposed in JVET-K0536[2] which utilizes an adaptive grouping algorithm to dynamically generate a single or multiple groups and derives a linear model for each group respectively. Then, chroma samples can be predicted by a linear model of the nearest group. The proposed method includes the following steps: classifying neighbouring samples into groups, generating a linear model for each group and predicting chroma samples based on reconstructed luma samples.

Proposed method

This test utilizes a grouping process to generate a single or multiple groups for linear model prediction. Firstly, samples from the top and left neighbouring reconstructed blocks are classified into groups according to both luma and chroma values. The classification method scans each neighbouring sample and determines if it can join an existing group according to the criteria (1) and (2). If the neighbouring sample cannot satisfy the criteria of any existing group, a new group will be created. As a result, all the neighbouring samples will be classified into a number of groups as illustrated in Figure 1.

Ygroup_min – Ymargin < Yp < Ygroup_max + Ymargin  (1)

Cgroup_min – Cmargin < Cp < Cgroup_max + Cmargin   (2)

Note that Yp and Cp denotes the luma and chroma value of a neighboring sample. Ygroup_max and Cgroup_max denotes the maximum luma and chroma value in a group. Ygroup_min and Cgroup_min denotes the minimum luma and chroma value in a group. Ymargin and Cmargin denotes the margin of the group for luma and chroma respectively.

Figure 1. Group classification of predictor pixels

After that, LM parameters αg and βg for each group will be derived by minimizing the regression error following the equation defined in CCLM. When predicting a chroma sample, the nearest group will be determined according to an averaged luma value. LM parameters αg and βg of the nearest group is then used to derive the chroma value with the following equation:

Results

According to the CE3 description in JVET-L1023 [1] and the common test conditions in JVET-L1010 [3], the proposed method is evaluated under all intra (AI) and random access (RA) configuration with VTM3.0 software under VTM configuration.

Table 1. Result for VTM AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.39% | -0.33% | 100% | 100% |
| Class A2 | 0.01% | -0.09% | -0.05% | 104% | 100% |
| Class B | 0.01% | -0.15% | -0.12% | 101% | 100% |
| Class C | 0.00% | -0.10% | -0.07% | 100% | 100% |
| Class E | 0.02% | 0.16% | 0.23% | 100% | 100% |
| Overall | 0.01% | -0.12% | -0.07% | 101% | 100% |
| Class D | 0.02% | 0.06% | 0.00% | 100% | 100% |
| Class F (optional) | 0.02% | -0.12% | -0.17% | 101% | 100% |

Table 2. Result for VTM RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.06% | -0.15% | 101% | 99% |
| Class A2 | -0.02% | 0.07% | 0.07% | 101% | 99% |
| Class B | -0.02% | 0.13% | 0.01% | 101% | 100% |
| Class C | 0.01% | -0.04% | 0.07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.01% | 101% | 100% |
| Class D | -0.01% | -0.12% | 0.06% | 101% | 100% |
| Class F (optional) | 0.01% | -0.09% | -0.09% | 101% | 100% |

Conclusion

The results show that the proposed grouping mechanism provides 0.12% and 0.07% coding gain for Cb, Cr respectively while cause 0.01% loss for luma with VTM all intra configuration and 0.00%, 0.03% and 0.01% coding loss with VTM random access configuration.

Reference

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, JVET-L1023, Macau, October 2018.

- S. Wang et al., “Adaptive multiple cross-component linear model”, JVET-K0536, Ljubljana, SI, July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macau, October 2018.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).