#### Changes to VVC WD highlighted in yellow

#### Derivation process for collocated motion vectors

Inputs to this process are:

- a variable currCb specifying the current coding block,

- a variable colCb specifying the collocated coding block inside the collocated picture specified by ColPic,

- a luma location ( xColCb, yColCb ) specifying the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic,

- a reference index refIdxLX, with X being 0 or 1,

- a flag indicating a subblock temporal merging candidate sbFlag.

Outputs of this process are:

- the motion vector prediction mvLXCol in 1/16 fractional-sample accuracy,

- the availability flag availableFlagLXCol.

The variable currPic specifies the current picture.

The arrays predFlagL0Col[ x ][ y ], mvL0Col[ x ][ y ] and refIdxL0Col[ x ][ y ] are set equal to PredFlagL0[ x ][ y ], MvL0[ x ][ y ] and RefIdxL0[ x ][ y ], respectively, of the collocated picture specified by ColPic, and the arrays predFlagL1Col[ x ][ y ], mvL1Col[ x ][ y ] and refIdxL1Col[ x ][ y ] are set equal to PredFlagL1[ x ][ y ], MvL1[ x ][ y ] and RefIdxL1[ x ][ y ], respectively, of the collocated picture specified by ColPic.

[Ed. (BB): Define ColPic NoBackwardPredFlag.]

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If colCb is coded in an intra prediction mode, or its reference picture is ColPic, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol and the reference list identifier listCol are derived as follows:

  - If sbFlag is equal to 0, availableFlagLXCol is set to 1 and the following applies:

  - If predFlagL0Col[ xColCb ][ yColCb ] is equal to 0, mvCol, refIdxCol and listCol are set equal to mvL1Col[ xColCb ][ yColCb ], refIdxL1Col[ xColCb ][ yColCb ] and L1, respectively.

  - Otherwise, if predFlagL0Col[ xColCb ][ yColCb ] is equal to 1 and predFlagL1Col[ xColCb ][ yColCb ] is equal to 0, mvCol, refIdxCol and listCol are set equal to mvL0Col[ xColCb ][ yColCb ], refIdxL0Col[ xColCb ][ yColCb ] and L0, respectively.

  - Otherwise (predFlagL0Col[ xColCb ][ yColCb ] is equal to 1 and predFlagL1Col[ xColCb ][ yColCb ] is equal to 1), the following assignments are made:

      - If NoBackwardPredFlag is equal to 1, mvCol, refIdxCol and listCol are set equal to mvLXCol[ xColCb ][ yColCb ], refIdxLXCol[ xColCb ][ yColCb ] and LX, respectively.

      - Otherwise, mvCol, refIdxCol and listCol are set equal to mvLNCol[ xColCb ][ yColCb ], refIdxLNCol[ xColCb ][ yColCb ] and LN, respectively, with N being the value of collocated_from_l0_flag.

  - Otherwise (sbFlag is equal to 1), the following applies:

  - If PredFlagLXCol[ xColCb ][ yColCb ] is equal to 1, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xColCb ][ yColCb ], refIdxLXCol[ xColCb ][ yColCb ], and LX, respectively, availableFlagLXCol is set to 1.

  - Otherwise (PredFlagLXCol[ xColCb ][ yColCb ] is equal to 0), the following applies:

      - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current tile group and PredFlagLYCol[ xColCb ][ yColCb ] is equal to 1, mvCol, refIdxCol, and listCol are set to mvLYCol[ xColCb ][ yColCb ], refIdxLYCol[ xColCb ][ yColCb ] and LY, respectively, with Y being equal to !X where X being the value of X this process is invoked for. availableFlagLXCol is set to 1.

      - Both the components of mvLXCol are set to 0 and availableFlagLXCol is set equal to 0.

  - When availableFlagLXCol is equal to TRUE, mvLXCol and availableFlagLXCol are derived as follows:

  - If LongTermRefPic( currPic, currCb, refIdxLX, LX ) is not equal to LongTermRefPic( ColPic, colCb, refIdxCol, listCol ), both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - Otherwise, the variable availableFlagLXCol is set equal to 1, refPicListCol[ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the tile group containing coding block colCb in the collocated picture specified by ColPic, and the following applies:

colPocDiff = DiffPicOrderCnt( ColPic, refPicListCol[ refIdxCol ] )	(8-359)

currPocDiff = DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] )	(8-360)

      - The temporal motion buffer compression process for collocated motion vectors as specified in clause A is invoked with mvCol as input, and the output is assigned to mvCol.

      - If RefPicListX[ refIdxLX ] is a long-term reference picture, or colPocDiff is equal to currPocDiff, mvLXCol is derived as follows:

mvLXCol = mvCol	(8-361)

      - Otherwise, mvLXCol is derived as a scaled version of the motion vector mvCol as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-362)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-363)

mvLXCol =  Clip3( −32768, 32767, Sign( distScaleFactor * mvCol ) * 
				( ( Abs( distScaleFactor * mvCol ) + 127 )  >>  8 ) )	(8-364)

where td and tb are derived as follows:

td = Clip3( −128, 127, colPocDiff )	(8-365)

tb = Clip3( −128, 127, currPocDiff )	(8-366)

A. Temporal motion buffer compression process for collocated motion vectors

Input to this process is:

- A motion vector mv,

Outputs of this process is:

- A rounded motion vector rmv

For each motion vector component compIdx, rmv[compIdx] is derived from mv[compIdx] as follows:

s = mv[compIdx] >> 17
f = Floor(Log2((mv[compIdx]  ^ s) | 31)) – 4
mask = (-1 << f) >> 1
round = (1 << f) >> 2
rmv[compIdx] = (mv[compIdx] + round) & mask

NOTE – This process enables storage of collocated motion vectors using a bit reduced representation. Each signed 18-bit motion vector component can be represented in a mantissa plus exponent format with a 6-bit signed mantissa and a 4-bit exponent.