| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0521-v1 |
| --- | --- |

| Title: | CE6: Replacement of 4-point DST7/DCT8 with DST4/DCT4 in MTS (Test 6.1.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Vadim Seregin, Amir Said, Ted Hsieh, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the CE6.1.6 results and proposes to use the 4-point transform tables tested in CE6.1.6a. Specifically, in CE6.1.6a, the 4-point DST7/DCT8 transform tables in VTM-3.0 are replaced with 4-point DST4/DCT8 tables. The experimental results show that the proposed changes provide 0.17% AI, 0.07% RA and 0.07% LDB average luma BD-rate gains over VTM-3.0 with Inter MTS enabled. The proposed modifications do not introduce additional memory or computational complexity on VTM-3.0.

# Introduction

In VTM-3.0, pairs of DCT-2, DST-7 or DCT-8 are used for separable transformation of residual block, where DCT-2 is used up to size 64x64 and DST-7 and DCT-8 are allowed up to size 32x32. When MTS (AMT) flag is 0, DCT-2 is used for both horizontal and vertical transformation. If the MTS flag is 1, one of the four separable transform options derived from DST-7 and DCT-8 is signalled and used to transform a block.

The current VTM-3.0 has the following issue that it uses the same type of DST/DCT type across different block sizes, and DST-7 and DCT-8 may not be good candidates for certain block sizes as the average block statistics vary depending on the block size. In CE6.1.6, it is observed that DST-4 and DCT-8 leads to a better coding gains for small blocks.

# Proposed method: Replacement of 4-point DST-7 and DCT-8 with DST-4 and DCT-4

In order to better capture the average residual statistics for small blocks, 4-point DST-7 and DCT-8 are replaced with DST-4 and DCT-4 respectively. Based on the experimental results, on average, DST-4 and DCT-4 provide better coding gains as compared to DST-7 and DCT-8 for small blocks.

The 4-point DST-4 below

{  18   50   75   89 }

{  50   89   18  -75 }

{  75   18  -89   50 }

{  89  -75   50  -18 }

is used to replace 4-point DST-7 in VTM-3.0:

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

The 4-point DCT-4 below

{  89   75   50   18 }

{  75  -18  -89  -50 }

{  50  -89   18   75 }

{  18  -50   75  -89 }

is used to replace 4-point DCT-8 in VTM-3.0:

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

# Experimental Results

In Section 3.1, the results of the proposed method, that is only replacing 4-pt transforms, are presented. Moreover, Section 3.2 presents the results of (i) replacing both 4-point and 8-point transforms and (ii) replacing of 4-point, 8-point and 16-point with DST4/DCT4, which are tested in CE6.1.6b and CE6.1.6c, respectively.

## Results of the proposed method, tested in CE 6.1.6a

The proposed method is implemented based on VTM-3.0 software and tested (in CE 6.1.6a) under CTC (see Table 1), and also by enabling the inter MTS (Inter AMT) via EMT=3 and EMTFast =3 (see Table 2).

Table 1: Results of the proposed method (CE6.1.6a) over VTM-3.0 under CTC.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.09% | 99% | 101% |
| Class A2 | -0.06% | -0.05% | -0.02% | 100% | 102% |
| Class B | -0.14% | -0.13% | -0.12% | 100% | 106% |
| Class C | -0.29% | -0.21% | -0.31% | 99% | 100% |
| Class E | -0.34% | -0.28% | -0.13% | 99% | 104% |
| Overall | -0.17% | -0.14% | -0.11% | 99% | 103% |
| Class D | -0.31% | -0.32% | -0.31% | 99% | 101% |
| Class F | -0.18% | -0.19% | -0.21% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.05% | 100% | 102% |
| Class A2 | -0.02% | 0.17% | 0.09% | 100% | 101% |
| Class B | -0.07% | 0.06% | 0.01% | 101% | 101% |
| Class C | -0.11% | -0.04% | 0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.05% | 0.04% | 100% | 101% |
| Class D | -0.14% | -0.44% | -0.34% | 100% | 100% |
| Class F | -0.11% | -0.03% | -0.08% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.26% | 0.08% | 99% | 100% |
| Class C | 0.01% | 0.44% | 0.17% | 100% | 99% |
| Class E | 0.10% | 1.06% | 0.16% | 100% | 97% |
| Overall | 0.02% | 0.30% | 0.13% | 100% | 99% |
| Class D | 0.02% | 0.20% | 0.08% | 100% | 101% |
| Class F | -0.04% | -0.49% | -0.22% | 100% | 98% |

Table 2: Results of the proposed method (CE6.1.6a) over VTM-3.0 with Inter MTS enabled.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.09% | 99% | 101% |
| Class A2 | -0.06% | -0.05% | -0.02% | 100% | 102% |
| Class B | -0.14% | -0.13% | -0.12% | 100% | 106% |
| Class C | -0.29% | -0.21% | -0.31% | 99% | 100% |
| Class E | -0.34% | -0.28% | -0.13% | 99% | 104% |
| Overall | -0.17% | -0.14% | -0.11% | 99% | 103% |
| Class D | -0.31% | -0.32% | -0.31% | 99% | 101% |
| Class F | -0.18% | -0.19% | -0.21% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | 0.08% | 100% | 102% |
| Class A2 | 0.00% | 0.13% | 0.11% | 100% | 103% |
| Class B | -0.08% | -0.03% | 0.03% | 101% | 102% |
| Class C | -0.17% | 0.02% | -0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.01% | 0.03% | 100% | 102% |
| Class D | -0.19% | -0.47% | -0.13% | 100% | 102% |
| Class F | -0.10% | -0.14% | -0.11% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.19% | -0.32% | 99% | 97% |
| Class C | -0.12% | 0.03% | 0.26% | 100% | 103% |
| Class E | -0.03% | 0.87% | 0.31% | 100% | 98% |
| Overall | -0.07% | 0.31% | 0.03% | 100% | 99% |
| Class D | -0.07% | 0.08% | -0.64% | 99% | 100% |
| Class F | -0.27% | -0.35% | -0.46% | 99% | 98% |

Since the proposed method changes the transform kernels, as part of the CE requirements, the results obtained at low QPs  [1] are also presented in Tables 3 and 4.

Table 3: Results of the proposed method (CE6.1.6a) over VTM-3.0 with only intra MTS enabled under the low QP configurations [1].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 + low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.02% | 91% | 102% |
| Class A2 | 0.03% | -0.01% | -0.01% | 96% | 98% |
| Class B | 0.02% | -0.05% | -0.05% | 95% | 96% |
| Class C | -0.04% | -0.13% | -0.14% | 93% | 97% |
| Class E | -0.02% | -0.04% | -0.06% | 95% | 95% |
| Overall | 0.00% | -0.06% | -0.06% | 94% | 97% |
| Class D | -0.07% | -0.13% | -0.15% | 95% | 96% |
| Class F | -0.02% | -0.14% | -0.18% | 93% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 + low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | 0.01% | 0.00% | 0.00% | 97% | 102% |
| Class B | 0.00% | -0.01% | -0.01% | 95% | 102% |
| Class C | 0.01% | -0.04% | -0.01% | 103% | 107% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.02% | -0.04% | 100% | 105% |
| Class F | -0.02% | -0.15% | -0.11% | 92% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 + low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | 0.01% | 87% | 98% |
| Class C | -0.01% | 0.01% | -0.03% | 107% | 107% |
| Class E | -0.01% | 0.02% | -0.01% | 96% | 102% |
| Overall | -0.01% | 0.00% | -0.01% | 96% | 102% |
| Class D | 0.00% | 0.00% | 0.02% | 103% | 108% |
| Class F | -0.13% | -0.06% | -0.03% | 107% | 119% |

Table 4: Results of the proposed method (CE6.1.6a) over VTM-3.0 with both intra and inter MTS enabled under the low QP configurations [1].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 + Inter MTS + low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.02% | 91% | 102% |
| Class A2 | 0.03% | -0.01% | -0.01% | 96% | 98% |
| Class B | 0.02% | -0.05% | -0.05% | 95% | 96% |
| Class C | -0.04% | -0.13% | -0.14% | 93% | 97% |
| Class E | -0.02% | -0.04% | -0.06% | 95% | 95% |
| Overall | 0.00% | -0.06% | -0.06% | 94% | 97% |
| Class D | -0.07% | -0.13% | -0.15% | 95% | 96% |
| Class F | -0.02% | -0.14% | -0.18% | 93% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS + low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | 0.01% | 0.01% | 0.01% | 99% | 103% |
| Class B | 0.00% | -0.04% | -0.03% | 95% | 96% |
| Class C | -0.03% | -0.06% | -0.07% | 96% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.07% | -0.11% | -0.12% | 93% | 92% |
| Class F | 0.08% | 0.02% | 0.04% | 90% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS + low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.03% | -0.02% | 92% | 96% |
| Class C | -0.04% | -0.06% | -0.06% | 93% | 102% |
| Class E | -0.01% | 0.00% | 0.00% | 91% | 94% |
| Overall | -0.01% | -0.03% | -0.03% | 92% | 97% |
| Class D | -0.10% | -0.13% | -0.12% | 94% | 94% |
| Class F | -0.19% | -0.24% | -0.04% | 94% | 96% |

## Additional results tested in CE 6.1.6b and 6.1.6c

In this section, CE6.1.6b and CE6.1.6c results are presented as additional results to demonstrate that the replacement of DST7/DCT8 with DST4/DCT4 beyond 4-point transforms do not improve coding gains. Therefore, it is proposed to replace only 4-pt transforms.

Tables 5 and 6 show results of CE6.1.6b, and Tables 7 and 8 present the results of CE6.1.6c.

Table 5: Results of CE6.1.6b over VTM-3.0 under CTC.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.06% | 0.15% | 99% | 102% |
| Class A2 | 0.03% | 0.06% | 0.00% | 99% | 101% |
| Class B | -0.06% | -0.23% | -0.19% | 99% | 101% |
| Class C | -0.33% | -0.31% | -0.35% | 100% | 100% |
| Class E | -0.30% | -0.42% | -0.34% | 99% | 103% |
| Overall | -0.12% | -0.18% | -0.16% | 99% | 101% |
| Class D | -0.39% | -0.43% | -0.36% | 100% | 99% |
| Class F | -0.21% | -0.30% | -0.34% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.21% | 0.18% | 101% | 101% |
| Class A2 | 0.02% | 0.28% | 0.15% | 100% | 101% |
| Class B | -0.02% | 0.24% | -0.16% | 99% | 99% |
| Class C | -0.12% | -0.05% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.16% | 0.02% | 100% | 100% |
| Class D | -0.18% | -0.44% | -0.26% | 101% | 101% |
| Class F | -0.03% | -0.16% | -0.08% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.28% | 0.00% | 99% | 102% |
| Class C | -0.02% | 0.20% | 0.18% | 100% | 101% |
| Class E | 0.10% | 0.64% | -0.51% | 99% | 94% |
| Overall | 0.02% | 0.11% | -0.07% | 100% | 100% |
| Class D | -0.10% | 0.02% | -0.20% | 100% | 101% |
| Class F | 0.02% | 0.17% | 0.22% | 100% | 97% |

Table 6: Results of CE6.1.6b over VTM-3.0 with Inter MTS enabled.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.06% | 0.15% | 99% | 102% |
| Class A2 | 0.03% | 0.06% | 0.00% | 99% | 101% |
| Class B | -0.06% | -0.23% | -0.19% | 99% | 101% |
| Class C | -0.33% | -0.31% | -0.35% | 100% | 100% |
| Class E | -0.30% | -0.42% | -0.34% | 99% | 103% |
| Overall | -0.12% | -0.18% | -0.16% | 99% | 101% |
| Class D | -0.39% | -0.43% | -0.36% | 100% | 99% |
| Class F | -0.21% | -0.30% | -0.34% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.05% | 0.11% | 101% | 103% |
| Class A2 | 0.09% | 0.13% | 0.07% | 101% | 103% |
| Class B | -0.03% | -0.03% | -0.25% | 99% | 99% |
| Class C | -0.22% | -0.25% | -0.19% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.04% | -0.10% | 100% | 101% |
| Class D | -0.29% | -0.50% | -0.30% | 101% | 102% |
| Class F | -0.09% | -0.22% | -0.13% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | 0.04% | -0.65% | 99% | 97% |
| Class C | -0.24% | -0.10% | -0.09% | 100% | 100% |
| Class E | 0.14% | 0.28% | -0.24% | 99% | 97% |
| Overall | -0.08% | 0.05% | -0.36% | 100% | 98% |
| Class D | -0.25% | -0.12% | -0.31% | 100% | 100% |
| Class F | -0.27% | -0.58% | -0.18% | 100% | 100% |

Table 7: Results of CE6.1.6c over VTM-3.0 under CTC.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.37% | 0.25% | 0.35% | 102% | 100% |
| Class A2 | 0.26% | 0.13% | 0.12% | 101% | 101% |
| Class B | 0.17% | -0.06% | -0.12% | 100% | 100% |
| Class C | -0.24% | -0.24% | -0.29% | 101% | 101% |
| Class E | -0.12% | -0.34% | -0.12% | 102% | 104% |
| Overall | 0.08% | -0.06% | -0.04% | 101% | 101% |
| Class D | -0.29% | -0.37% | -0.33% | 101% | 101% |
| Class F | -0.13% | -0.20% | -0.20% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.20% | 0.38% | 100% | 101% |
| Class A2 | 0.14% | 0.14% | 0.21% | 100% | 102% |
| Class B | 0.12% | 0.35% | 0.10% | 99% | 99% |
| Class C | -0.08% | -0.16% | 0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.14% | 0.18% | 100% | 100% |
| Class D | -0.10% | -0.47% | -0.03% | 101% | 101% |
| Class F | 0.01% | 0.12% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.09% | 0.06% | 99% | 101% |
| Class C | 0.02% | 0.05% | 0.22% | 101% | 101% |
| Class E | 0.17% | 0.06% | 0.32% | 99% | 98% |
| Overall | 0.07% | 0.00% | 0.18% | 100% | 100% |
| Class D | -0.03% | -0.25% | -0.04% | 100% | 101% |
| Class F | -0.08% | -0.16% | 0.24% | 99% | 96% |

Table 8: Results of CE6.1.6c over VTM-3.0 with Inter MTS enabled.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.37% | 0.25% | 0.35% | 102% | 100% |
| Class A2 | 0.26% | 0.13% | 0.12% | 101% | 101% |
| Class B | 0.17% | -0.06% | -0.12% | 100% | 100% |
| Class C | -0.24% | -0.24% | -0.29% | 101% | 101% |
| Class E | -0.12% | -0.34% | -0.12% | 102% | 104% |
| Overall | 0.08% | -0.06% | -0.04% | 101% | 101% |
| Class D | -0.29% | -0.37% | -0.33% | 101% | 101% |
| Class F | -0.13% | -0.20% | -0.20% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | 0.19% | 0.42% | 100% | 102% |
| Class A2 | 0.18% | 0.14% | 0.15% | 101% | 103% |
| Class B | 0.11% | 0.05% | -0.09% | 99% | 98% |
| Class C | -0.20% | -0.11% | 0.11% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.05% | 0.11% | 100% | 100% |
| Class D | -0.24% | -0.65% | -0.19% | 101% | 101% |
| Class F | -0.02% | -0.02% | -0.06% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 + Inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.03% | -0.07% | 100% | 97% |
| Class C | -0.30% | -0.40% | -0.10% | 100% | 102% |
| Class E | -0.02% | 0.34% | 0.43% | 100% | 98% |
| Overall | -0.14% | -0.06% | 0.04% | 100% | 99% |
| Class D | -0.24% | 0.01% | -0.21% | 100% | 100% |
| Class F | -0.31% | -0.11% | -0.33% | 100% | 99% |

# Complexity Metrics

## Operation and memory counts: The proposed method tested in CE6.1.6 introduces no additional computational complexity and memory requirements over VTM-3.0.

## The bit-precision required by the transforms: The same precision in the current VTM-3.0 is used.

## Specify if a transform requires multiple iterations: No additional iteration is required from VTM-3.0.

## Encoder and decoder times: Listed in the experimental results in Section 3.

# Proposed text for specification document

The proposed syntax changes for JVET-L1001 version 11 are as follows, where the changes are colored in red and the removed syntax elements are crossed out as red.

8.4.4.3	Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

…

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 						(8-821)

{

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

{  18   50   75   89 }

{  50   89   18  -75 }

{  75   18  -89   50 }

{  89  -75   50  -18 }

},

…

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-828)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

{  89   75   50   18 }

{  75  -18  -89  -50 }

{  50  -89   18   75 }

{  18  -50   75  -89 }

},

# Conclusions

In this contribution, it has been shown that replacing 4-point DST7 (resp. DCT8) with DST4 (resp. DCT4) improves coding gains with no additional computational and memory complexity over VTM-3.0. Based on the experimental results, the method tested in CE6.1.6a is proposed for adoption in VVC.

# References

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, 12th meeting, Macao, CN, Oct. 2018.

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, Slovenia, July 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).