| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0534-v2 |
| --- | --- |

| Title: | CE13-related: HEC with Pre-rotation + Adaptive Frame Packing(Test 4.2.a+4.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akhil Konda Chirag Pujara Anubhav Singh | Email: | akhil.konda@samsung.com chirag.p@samsung.com anubhav.23@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports simulation results of Pre-rotation followed by Adaptive Frame Packing(AFP) on Hybrid Equiangular Cubemap (HEC) implementation over 360Lib8.0 merged with VTM3.0. This test is implemented over test 1.1a (HEC with in-loop filters disabled) which shows  gain of {-1.14%, -1.25%, -1.49%} over CTC anchor and {-1.13%, -0.96%, -1.36%} over CE anchor (test 1.1b), while it shows a gain of {-0.57%, -0.67%, -1.12%} over test 1.1a under 360 Common Test Condition (CTC) for E2EWSPSNR. Maximum gain is observed for Gaslamp sequence, which is around -2.78%.

# Introduction

Simulations were performed under 360 video CTC [JVET-L1012] by merging the codes of both Test 4.2 and Test 4.1 available at https://vcgit.hhi.fraunhofer.de/JVET-L-CE13/VVCSoftware_VTM/. The simulations were run with HEC implementation over 360Lib8.0 merged with VTM3.0 software with inloop Filters Disabled.

The tests performed are as follows:

- Pre-Rotation+ AFP: Rotation is implemented over base HEC(CE13.1.1a). The Rotated HEC is taken as input for Adaptive frame Packing Algorithm. Algorithmic description for computation of  rotation angle as well as Adaptive Frame Packing Parameters can be found in CE13 document [JVET-L1033].

# Experimental Results

Snapshot of experimental results is as follows,

- HEC(1.1a) + Pre-rotation+ AFP  vs CTC anchor (PHEC)

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -1.48% | -1.86% | -2.15% | -1.53% | -1.88% | -2.17% |
| Class S2 | -0.62% | -0.33% | -0.49% | -0.62% | -0.32% | -0.46% |
| Overall | -1.14% | -1.25% | -1.49% | -1.16% | -1.25% | -1.49% |

- HEC(1.1a)  + Pre-rotation + AFP vs CE anchor (PHEC + In-loop filters disabled)- Test 13.1.1b

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -1.46% | -1.66% | -2.09% | -1.50% | -1.69% | -2.12% |
| Class S2 | -0.64% | 0.09% | -0.25% | -0.65% | 0.09% | -0.24% |
| Overall | -1.13% | -0.96% | -1.36% | -1.16% | -0.98% | -1.37% |

- HEC(1.1a)  + Pre-rotation + AFP  vs  Test 1.1a(HEC + In-loop filters disabled)

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -0.91% | -1.51% | -1.93% | -0.94% | -1.55% | -1.97% |
| Class S2 | -0.06% | 0.59% | 0.08% | -0.08% | 0.58% | 0.08% |
| Overall | -0.57% | -0.67% | -1.12% | -0.59% | -0.70% | -1.15% |

# Observations

Majority of the Gain for Rotation +AFP comes from 8k sequences which is around 0.9%. The low gain  for 6k Sequences could be because of high global motion present in those sequences. Highest gain is observed for Gaslamp sequence around 2.78% for the Luma component.

# Cross check

Coss-check is performed by P.Hanhart of InterDigital

# References

- P. Hanhart, J.-L. Lin, C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-L1033, Oct. 2018, Macau, CN.

- P. Hanhart, J. Boyce, K. Choi, J.-L. Lin “JVET common test conditions and evaluation procedures for 360° video”, JVET-L1012, Oct. 2018, Macau, CN.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).