| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0540-v1 |
| --- | --- |

| Title: | CE6-related: Software tool for computing transform throughput |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi E. Egilmez, Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Transform computational complexity is an important part of a codec’s implementation costs. It is becoming difficult to evaluate a transform-related proposal’s performance from only full encoder and decoder times, since their measurements are becoming less reliable in large clusters and parallel execution. Furthermore, current codecs include many high complexity tools that can mask very important differences. Standard software profilers do not help because they cannot differentiate the unique characteristics and measurement problems defined by separable transforms. This proposal shows that, due to the VTM code organization, it is easy to implement a profiler specifically tailored to measure transform throughputs (i.e., transform coefficients per second), provides code for that purpose, and discusses how to enable consistent results and cross-checking.

# Introduction

It is increasingly more difficult to measure the performance of different transform methods in current high-complexity codecs. Practical problems include

- Full encoder and decoder times can be used to evaluate control tools that affect general code execution (like partitioning), but are not accurate enough to evaluate specific tasks, like entropy coding, quantization, and transforms.

- Execution in large clusters with multiple CPUs, configurations, and loads, provide very unreliable time measurements.

- Transforms in current codecs are applied to non-square blocks (and we may expect even more complicated geometries), making it very hard to design stand-alone programs that replicate the conditions expected in a real codec.

- Standard theoretical evaluation tools, like counting number of arithmetic operations, provide only very rough estimates, since actual computational costs are also defined by control operations, memory access, and many other factors.

# Proposed solution and implementation

A good way to evaluate a coding tool is to measure its performance, whenever possible, in the conditions it is going to be used. Fortunately, the structure of VTM software greatly facilitates measuring transform times, because all transforms computations start from a few function calls, in a single code file (TrQuant.cpp).

Included with this proposal is a version of TrQuant.cpp that includes a class implementation that accurately measures the times used to compute the separable transforms in a block, separating the time measurements according to transform dimension and direction (i.e., forward or inverse), also measuring the number of elements in the block.

Tracking the number of elements is necessary due to the non-square blocks, and enables normalizing execution times per transform coefficients. While measuring time per coefficient is equivalent to measuring its inverse, the throughput in coefficients per second, the latter is more commonly used, since it is used for estimates of frames per second.

Accurately measuring execution times in different computer platforms is a famously complicated problem, but has been made easier with the introduction in C++11 of resources for better time measurement, in the chrono library, and the std::chrono::high_resolution_clock features, which are used by the proposed code.

The code has been set to only measure times for transforms different from the DCT-2, and of dimensions 16, 32, and 64, since those are the ones in the most important discussions related to complexity.

To simplify integration the code has no user interface or options, and the user only has to do the following steps.

- Add the profiler code and corresponding modifications to file TrQuant.cpp, and re-compile.

- Execute the encoder with desired configuration and sequence.

- Execute the decoder in the same directory.

- Read measurements written in file vtm_transform_throughputs.txt. Figure 1 has an example of such file.

Encoder forward DCT-8 + DST-7 transforms : 183.76 s (  35721.41 Mcf )

Dimension 16 :   199.94 Mcf/s =  21320.79 Mcf / 106.64 s

Dimension 32 :   169.05 Mcf/s =  11580.19 Mcf /  68.50 s

Dimension 64 :   327.04 Mcf/s =   2820.42 Mcf /   8.62 s

Encoder inverse DCT-8 + DST-7 transforms : 158.66 s (  30843.68 Mcf )

Dimension 16 :   176.36 Mcf/s =  18019.81 Mcf / 102.18 s

Dimension 32 :   208.42 Mcf/s =  10042.92 Mcf /  48.18 s

Dimension 64 :   335.00 Mcf/s =   2780.95 Mcf /   8.30 s

Decoder inverse DCT-8 + DST-7 transforms :   0.88 s (    253.89 Mcf )

Dimension 16 :   215.86 Mcf/s =     40.97 Mcf /   0.19 s

Dimension 32 :   259.52 Mcf/s =     93.40 Mcf /   0.36 s

Dimension 64 :   366.77 Mcf/s =    119.52 Mcf /   0.33 s

Figure 1 – Example of the output file generated by the throughput code, where “Mcf” means millions of transform coefficients.

# Experimental results

Table I has some examples of throughput measurements for code in current CE6 tests. Those cases correspond to encoding and decoding 20 frames of the sequence “Tango2” with VTM-3.0, using the CTC AI configuration, and QP = 32. This should provide a good estimate, even though there is still some variance in the results. More stable results are expected with larger numbers of encoded frames.

Table I – Example of throughput measurements for different transform dimensions (16, 32, and 64), and some primary transform CEs, and CE-related proposals. Table values correspond to 106 transform coefficients per second.

| Transform method | Encoder |  |  |  |  |  | Decoder |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Forward |  |  | Inverse |  |  | Inverse |  |  |
|  | 16 | 32 | 64 | 16 | 32 | 64 | 16 | 32 | 64 |
| VTM (matrix-based) | 170 | 125 | 208 | 80 | 49 | 67 | 81 | 49 | 66 |
| CE6-1.4d | 205 | 173 | 336 | 180 | 212 | 342 | 214 | 257 | 361 |
| CE6-1.1c | 169 | 124 | — | 79 | 42 | — | 86 | 51 | — |
| CE6-1.1d | 213 | 132 | — | 173 | 114 | — | 200 | 125 | — |
| CE6-1.5d | 148 | 139 | 167 | 130 | 115 | 132 | 145 | 130 | 135 |
| CE6-2.1a | 176 | 155 | — | 141 | 131 | — | 179 | 156 | — |
| CE6-2.2a | 43 | 46 | — | 43 | 45 | — | 44 | 46 | — |
| CE6-2.3a | 213 | 131 | — | 173 | 113 | — | 192 | 119 | — |
| CE6-1.4d SIMD | 222 | 200 | 389 | 280 | 334 | 723 | 351 | 429 | 838 |
| JVET-M0539 | 409 | 346 | 1278 | 184 | 219 | 339 | 213 | 256 | 360 |
| JVET-M0539 SIMD | 422 | 386 | 1370 | 282 | 338 | 750 | 373 | 449 | 876 |

Table II – Examples of throughput ratios, using matrix-based VTM transforms as reference.

| Transform method | Encoder |  |  |  |  |  | Decoder |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Forward |  |  | Inverse |  |  | Inverse |  |  |
|  | 16 | 32 | 64 | 16 | 32 | 64 | 16 | 32 | 64 |
| CE6-1.4d | 1.21 | 1.38 | 1.61 | 2.29 | 4.35 | 5.07 | 2.63 | 5.29 | 5.48 |
| CE6-1.1c | 1.00 | 1.00 | — | 1.00 | 0.86 | — | 1.06 | 1.05 | — |
| CE6-1.1d | 1.26 | 1.06 | — | 2.20 | 2.34 | — | 2.46 | 2.57 | — |
| CE6-1.5d | 0.87 | 1.12 | 0.80 | 1.65 | 2.36 | 1.97 | 1.79 | 2.68 | 2.05 |
| CE6-2.1a | 1.04 | 1.24 | — | 1.79 | 2.68 | — | 2.21 | 3.23 | — |
| CE6-2.2a | 0.26 | 0.36 | — | 0.54 | 0.92 | — | 0.54 | 0.95 | — |
| CE6-2.3a | 1.26 | 1.05 | — | 2.20 | 2.32 | — | 2.36 | 2.45 | — |
| CE6-1.4d SIMD | 1.31 | 1.60 | 1.86 | 3.56 | 6.85 | 10.72 | 4.32 | 8.85 | 12.73 |
| JVET-M0539 | 2.41 | 2.78 | 6.13 | 2.33 | 4.50 | 5.02 | 2.63 | 5.29 | 5.47 |
| JVET-M0539 SIMD | 2.49 | 3.09 | 6.57 | 6.94 | 6.94 | 11.11 | 4.59 | 9.25 | 13.32 |

While the results in Table I are interesting for comparing throughputs of different transforms, they are not as useful for comparing different methods for transform computations. For those comparisons it is best to use the ratio of throughput by a reference. For example, Table II shows the ratios of throughputs using the matrix-based VTM code as reference (similar to the method of comparing ratios of encoder and decoder times).

Also included with this proposal is a Python script called normalize_throughputs.py, which accepts as input parameters the name of a reference and output files, and outputs a file listing the throughput ratios of all files with names vtm_*.txt.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).