| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET- M0547-v1 |
| --- | --- |

| Title: | 360° coding tools using uncoded areas |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Draft text |  |  |
| Author(s) or Contact(s): | Johannes Sauer Max Bläser | Tel: Email: | +49241-80-27678 sauer@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This document describes how coding tools for cube based 360° video could be implemented without changes to the core design. VVC would be extended with a mechanism that specifies uncoded areas in the decoded picture buffer, which would be filled by specifically processed samples. For 360° video coding these areas would be filled with samples for the purposes of geometry padding or filtering across face boundaries. The specification of these processes does not necessarily have to be part of VVC, but could also be part in an independent standard which would need to be implemented in combination with VVC to enable 360° video applications.

It is suggested that such a mechanism might also be useful for other applications, such as scalable, multi-view or any kind of multi-layer coding with need for specific inter-layer processing.

# Introduction

The advantages of allowing uncoded areas in the picture buffer are explained mostly on the example of 360° video coding. Other possible applications are outlined at the end of the document.
In 360° video only a small portion of the content (a viewport) is displayed at a given time. This makes coding using tiles quite suitable for 360° video. The video can be split in tiles such that only the currently viewed viewport needs to be decoded.
From the already performed CE13 experiments it could be observed that geometry padding and in-loop filters using spherical neighbors can improve the objective coding performance and subjective quality (seam artifacts) significantly (JVET-L0647).
Here we describe how these operations can be done without changes at CTU level. All required operation can be done at buffer/tile level, if uncoded padding areas are provided around the faces, which can be filled with samples for the purposes of geometry padding or filtering of face boundaries.

# Providing uncoded areas in the picture buffer

For geometry padding samples need to be projected and interpolated from other cube faces. This can be done on the fly. However this would require changes of the design at CTU level. Further, it requires arbitrary buffer access, as the required samples can be located at up to three (cube corner) locations in the picture.
Alternatively a different reference picture can be used for each cube face, which only contains the cube face plus a padding area. This approach has the problem of increased memory usage and generation on not actually used samples at the decoder side. 
Here we describe how the problems of the buffer level approach can be avoided by using tiles and introducing a new type of tiles.
The problems of the buffer level approach can be avoided by using tiles and introducing a new type of tiles. 
Firstly, 360° video has to be coded using tiles and each cube face must consist of at least one tile. Secondly, for providing uncoded areas a change is proposed that allows tiles which do not hold coded CTU data.

## Reservation of padding space using empty tiles

JVET-L0686 (“Spec text for the agreed starting point on slicing and tiling”) was used as basis for the following, since the JVET-L1001 did not yet contain text on tiles at the time of writing.

We propose a change to the tile design, which allows “empty” tiles (ET). An ET does not hold any coded CTU data. However this allows the arrangement of the buffer space into different regions, while including space for samples, which are not coded, but used for geometry padding and filtering of face boundaries. Compare Figure 1 and Figure 2.

Figure 1: Division of picture into tile rows.

Figure 2: Further sub-division into tiles. Green: Tiles holding coded CTU data: the coded cube faces. White: Uncoded areas created using empty tiles (ET).

To enable ET the tile_group_header is changed as follows:

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| if ( tile_group_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| tile_coded_flag | u(1) |
| if( tile_coded_flag ) { |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

## Loop filters for uncoded areas

Loop filters should be turned off implicitly at boundaries between coded and uncoded areas, similar to the current handling of picture borders.
In coded areas loop filters should behave as currently.
Additionally loop filters may be applied in uncoded areas and between boundaries of different uncoded areas.

# 360° tools using uncoded areas

## Signalling of the 360° geometry

We propose a signalling of the 360° geometry which is heavily based on the region-wise packing SEI message of HEVC.
The coded cube faces can have different sizes and need not have any specific relative arrangement (continuous face boundaries), since each face is coded with at least one tile. The changed SEI is:

| cube_packing( payloadSize ) { | Descriptor |
| --- | --- |
| rwp_cancel_flag | u(1) |
| if( !rwp_cancel_flag ) { |  |
| 360_video_geometry_type | u(3) |
| if( 360_video_geometry_type == FORMAT_360_CMP ¦¦ |  |
| 360_video_geometry_type == FORMAT_360_EAC ¦¦ |  |
| 360_video_geometry_type == FORMAT_360_HEC) { |  |
| unit_cube_packing_width | u(32) |
| unit_cube_packing_height | u(32) |
| coded_cube_packing_width | u(16) |
| coded_cube_packing_height | u(16) |
| for( i = 0; i < num_cube_face_equal_to_6; i++ ) { |  |
| unit_face_top[ i ] | u(32) |
| unit_face_left[ i ] | u(32) |
| coded_face_width[ i ] | u(16) |
| coded_face_height[ i ] | u(16) |
| coded_face_top[ i ] | u(16) |
| coded_face_left[ i ] | u(16) |
| cube_face_transform_type[ i ] | u(3) |
| cube_face_padding_size[ i ] | u(16) |
| } |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Processes for buffer based / high level 360° tools

Here we describe processes which operate at buffer level for enabling 360° coding tools. These processes could be specified in a standard independent of VVC.
The padded regions in the buffer are processed at buffer/level. Inputs and outputs are the reference pictures. At CTU level only the actual samples from the neighborhood are used for prediction or loop filtering.
We only describe how geometry padding of samples could be done. Padding for modified boundary filtering does not require re-projection and could be done with a simpler process.
Further, the geometry padding is only described for the luma component so far and the interpolation process is still unspecified.

### Overview of generation of samples for geometry padding

The process for doing geometry padding is illustrated is as follows (compare Figure 3):

- A integer sample location in the padded region is selected

- The location is mapped to a unit cube packing location

- The unit cube packing location is projected to a 3D cube location

- The 3D cube location is back projected to a source unit cube packing location. This automatically moves the sample from the padding region inside another face.

- The source unit cube packing is mapped to a fractional sample location in the coded face arrangement.

- The sample of step 1 is interpolated using the found source sample location.

Figure 3: Derivation of source sample location for filling of padding region

#### Generation of unnecessary samples at decoder side

Not every side of a cube face has to be always geometry padded. Padding is mostly required when the camera is moving. However, if the movement across the face boundary is into the face, no padding is required.

Whether padding is required can be signalled with an SEI message. This could reduce the decoder complexity considerably.

### Detailed process for generation of samples for geometry padding

Input to this process is a pic_width_in_luma_samples by pic_height_in_luma_samples array of luma samples refPicLXL and two PicWidthInSamplesC by PicHeightInSamplesC arrays of chroma samples refPicLXCb and refPicLXCr.

#### Genration of luma samples for padding regions of cube faces

This clause specifies the semantics of the generation of luma samples for the padding regions of the coded cube faces.

offsetX is set equal to 0.5 and offsetY is set equal to 0.5.

The following applies for each packed padded cube face n in the range of 0 to 5, inclusive:

–	For each n-th packed padded cube face, the following applies:

  - PaddedFaceTop is set to coded_face_top[ n ] − cube_face_padding_size[ n ], PaddedFaceLeft is set to coded_face_top[ n ] − cube_face_padding_size[ n ], PaddedFaceWidth is set to coded_face_width[ n ] + 2 * cube_face_padding_size[ n ], PaddedFaceHeight is set to coded_face_height[ n ] + 2 * cube_face_padding_size[ n ].

  - The variables UnitFaceWidth, UnitFaceHeight, UnitFaceColumn and UnitFaceRow are set as following:
	UnitFaceWidth = unit_cube_packing_width / 3
	UnitFaceHeight = unit_cube_packing_height / 2
	UnitFaceColumn = Floor(unit_face_left ÷ lw )
	UnitFaceRow = Floor(unit_face_top ÷ lh )

  - The variable UnitFacePaddingWidth and UnitFacePaddingHeight are set as following:

if( transformType  = =  0  | |  transformType  = =  1  | |  transformType  = =  2  | |  transformType  = =  3 ) 	{
		horRatio = projRegWidth ÷ packedRegWidth
		verRatio = projRegHeight ÷ packedRegHeight
	} else if ( transformType  = =  4  | |  transformType  = =  5  | |  transformType  = =  6  | |
		transformType  = =  7 ) {
		horRatio = projRegWidth ÷ packedRegHeight
		verRatio = projRegHeight ÷ packedRegWidth
	}
	UnitFacePaddingWidth = horRatio * cube_face_padding_size[ n ]
	UnitFacePaddingHeight = verRatio * cube_face_padding_size[ n ]

  - For each sample location (xCodedFace, yCodedFace) in the range of [PaddedFaceLeft, PaddedFaceLeft + PaddedFaceWidth] and [PaddedFaceLeft, PaddedFaceTop + PaddedFaceHeight], respectively, the following applies:

    - Process 2.3.2.2 is called with xCodedFace, yCodedFace, PaddedFaceLeft, PaddedFaceTop, PaddedFaceWidth, PaddedFaceHeight and cube_face_padding_size[ n ] as inputs. The output is stored as isInFace.

    - If the value of isInFace is equal to 0 the following applies:

      - x is set equal to xCodedFace − coded_face_left[ n ].

      - y is set equal to yCodedFace − coded_face_top[ n ].

      - Clause  2.3.2.3 is invoked with x, y, coded_face_width[ n ], coded_face_height[ n ], UnitFaceWidth, UnitFaceHeight, TransformType[ n ], offsetX and offsetY as inputs, and the output is assigned to sample location (hPos, vPos).

      - xUnitFace is set equal to unit_face_left[ n ] + hPos.

      - yUnitFace is set equal to unit_face_top[ n ] + vPos.

      - Clause 2.3.2.4 is invoked with xUnitFace, yUnitFace, UnitFaceColumn, UnitFaceRow, UnitFaceWidth, UnitFaceHeight, UnitFacePaddingWidth, UnitFacePaddingHeight, pps_360_video_geometry_type as inputs and the output is assigned to position (xPos, yPos, zPos).

      - Clause 2.3.2.5 is invoked with position (xPos, yPos, zPos), UnitFaceWidth, UnitFaceHeight as inputs and the output is assigned to (hPosSrcUnitFace, vPosSrcUnitFace).

      - faceSrcIdx is set to lookUpTab(UnitFaceColumnSrc, UnitFaceRowSrc).

      - x2 is set equal to hPosSrcUnitFace − unit_face_left[ faceSrcIdx ].

      - y2 is set equal to vPosSrcUnitFace − unit_face_top[ faceSrcIdx ].

      - Clause 2.3.2.6 is invoked with (x2, y2), coded_face_width[ n ], coded_face_height[ n ], UnitFaceWidth, UnitFaceHeight, TransformType[ n ], offsetX and offsetY as inputs, and the output is assigned to (hPos3, vPos3).

      - hPosSrcCoded is set equal to hPos3 + coded_face_left[ faceSrcIdx ].

      - vPosSrcCoded is set equal to vPos3 + coded_face_top[ faceSrcIdx ].

      - Clause xxx is invoked with (hPosSrcCoded, vPosSrcCoded) as input and the output is assigned to the sample specified by (xCodedFace, yCodedFace).

#### Check if position in inside cube face or padding area

Inputs to this process are:

–	sample location (x, y) within the packed region, where x and y are in relative packed picture sample units, while the sample location is at an integer sample location within the packed picture,

- The location (PaddedFaceLeft, PaddedFaceTop) and size (PaddedFaceWidth, PaddedFaceHeight) of the padded cube face, in relative packed picture sample units,

- The size of the padding (FacePaddingSize)

Outputs of this process are:

–	A Boolean isInFace, indicating wheter the sample location is inside the coded region of the face (isInFace = = 1) or inside the padding region of the face (inInFace = = 0).

The output is derived as follows:

if( 	x < PaddedFaceLeft + FacePaddingSize || 
		x >= PaddedFaceWidth – FacePaddingSize ||
		y < PaddedFaceTop + FacePaddingSize || 
		y >= PaddedFaceHeight – FacePaddingSize ) {
			isInFace = 0
	}
	else {
			isInFace = 1
	}

#### Conversion of sample locations from coded cube face cords to unit cube face coords

Inputs to this process are:

–	sample location (x, y) within the packed region, where x and y are in relative packed picture sample units, while the sample location is at an integer sample location within the packed picture,

–	the width and the height (projRegWidth, projRegHeight) of the projected region, in relative projected picture sample units,

–	the width and the height (packedRegWidth, packedRegHeight) of the packed region, in relative packed picture sample units,

–	transform type (transformType), and

–	offset values for the sampling position (offsetX, offsetY) in the range of 0, inclusive, to 1, exclusive, in horizontal and vertical relative packed picture sample units, respectively.

NOTE – offsetX and offsetY both equal to 0.5 indicate a sampling position that is in the centre point of a sample in packed picture sample units.

Outputs of this process are:

–	the centre point of the sample location (hPos, vPos) within the projected region in relative projected picture sample units, where hPos and vPos may have non-integer real values.

The outputs are derived as follows:

if( transformType  = =  0  | |  transformType  = =  1  | |  transformType  = =  2  | |  transformType  = =  3 ) {
	horRatio = projRegWidth ÷ packedRegWidth
	verRatio = projRegHeight ÷ packedRegHeight
} else if ( transformType  = =  4  | |  transformType  = =  5  | |  transformType  = =  6  | |
	transformType  = =  7 ) {
	horRatio = projRegWidth ÷ packedRegHeight
	verRatio = projRegHeight ÷ packedRegWidth
}
if( transformType  = =  0 ) {
	hPos = horRatio * ( x + offsetX )
	vPos = verRatio * ( y + offsetY )
} else if ( transformType  = =  1 ) {
	hPos = horRatio * ( packedRegWidth − x − offsetX )
	vPos = verRatio * ( y + offsetY )
} else if ( transformType  = =  2 ) {
	hPos = horRatio * ( packedRegWidth − x − offsetX )
	vPos = verRatio * ( packedRegHeight − y − offsetY )	(D-59)
} else if ( transformType  = =  3 ) {
	hPos = horRatio * ( x + offsetX )
	vPos = verRatio * ( packedRegHeight − y − offsetY )
} else if ( transformType  = =  4 ) {
	hPos = horRatio * ( y + offsetY )
	vPos = verRatio * ( x + offsetX )
} else if ( transformType  = =  5 ) {
	hPos = horRatio * ( y + offsetY )
	vPos = verRatio * ( packedRegWidth − x − offsetX )
} else if ( transformType  = = 6 ) {
	hPos = horRatio * ( packedRegHeight − y − offsetY )
	vPos = verRatio * ( packedRegWidth − x − offsetX )
} else if ( transformType  = =  7 ) {
	hPos = horRatio * ( packedRegHeight − y − offsetY )
	vPos = verRatio * ( x + offsetX )
}

#### Projection for one sample location to face plane of unit cube

Inputs to this process are:

–	the centre point of a sample location (hPos, vPos) along the horizontal and vertical axes, respectively, in relative projected picture sample units, where hPos and vPos may have non-integer real values.

–	UnitFaceColumn and UnitFaceRow, which specify the location of the unit face to which the sample location belongs.

–	UnitFaceWidth and UnitFaceHeight, which are the width and height, respectively, of a monoscopic projected luma cube face, in relative projected picture sample units, and

–	UnitFacePaddingWidth and UnitFacePaddingHeight, which are the padding width and padding height, respectively, of a monoscopic projected luma cube face, in relative projected picture sample units, and

–	geometryType, which shall specify the type of cube based coding geometry, which is used. Values of geometryType are stated in xxx. And

Outputs of this process are:

- relative cube coordinates (x, y, z) for the sample location along the x-, y- and z-axes, respectively, in relative projected picture sample units, where x, y and z may have non-integer real values.

The projection for a sample location is derived as follows:

lw = UnitFaceWidth  + 2 * UnitFacePaddingWidth
lw_noPadding = UnitFaceWidth
lh = UnitFaceHeight + 2 * UnitFacePaddingHeight
lh_noPadding = UnitFaceHeight
w = UnitFaceColumn
h = UnitFaceWidthRow

/* conversion to face cords ranging from -1.0 to 1.0, however, since we are in the padding region we will be outside these */
tmpHorVal = hPos − w * lw − UnitFacePaddingWidth
tmpVerVal = vPos − h * lh − UnitFacePaddingHeight
hPos′ = −( 2 * tmpHorVal ÷ lw_noPadding) + 1
vPos′ = −( 2 * tmpVerVal ÷ lh_noPadding) + 1

/* adjustment of distance between samples */
if( geometryType = = HEC ) {
	t = 1.0 + 0.4 * (1.0- hPos′ * hPos′) * (1.0- vPos′ * vPos′)
	hPos′′ = stan(hPos′ *S_PI / 4.0)
	if( ( w  = =  2  &&  h  = =  1 ) || ( w  = =  0  &&  h  = =  1 ) ) { /*  top face or bottom face */
		vPos′′ = stan(vPos′ *S_PI / 4.0)
	}
	else {
		vPos′′ = vPos′ / t
	}
}
else if(geometryType = = EAC ) {
	hPos′′ = stan(hPos′ * S_PI / 4.0 )
	vPos′′ = stan(vPos′ * S_PI / 4.0 )
}
else { /* geometryType = = CMP */
	hPos′′ = hPos′
	vPos′′ = vPos′
}

/* mapping to unit cube coordinates */
if( w  = =  1  &&  h  = =  0 ) { /* positive x front face */
	x = 1.0
	y = hPos′′
	z = vPos′′
} else if( w  = =  1  &&  h  = =  1 ) { /* negative x back face */
	x = −1.0
	y = −vPos′′
	z = −hPos′′
} else if( w  = =  2  &&  h  = =  1 ) { /* positive z top face */	(D-57)
	x = −hPos′′
	y = −vPos′′
	z = 1.0
} else if( w  = =  0  &&  h  = =  1 ) { /* negative z bottom face */
	x = hPos′′
	y = −vPos′′
	z = −1.0
} else if( w  = =  0  &&  h  = =  0 ) { /* positive y left face */
	x = −hPos′′
	y = 1.0
	z = vPos′′
} else { /* ( w  = =  2  &&  h  = =  0 ), negative y right face */
	x = hPos′′
	y = −1.0
	z = vPos′′
}

#### Projection for a relative cube coordinate to one relative projected picture sample location

Inputs to this process are:

–	relative cube coordinates (xPos, yPos, zPos) for the sample location along the x-, y- and z-axes, respectively, in relative projected picture sample units, where xPos, yPos and zPos may have non-integer real values.

- UnitFaceWidth and UnitFaceHeight, which are the width and height, respectively, of a monoscopic projected luma cube face, in relative projected picture sample units.

Outputs of this process are:

- the centre point of a sample location (hPos, vPos) along the horizontal and vertical axes, respectively, in relative projected picture sample units, where hPos and vPos may have non-integer real values.

The projection for a sample location is derived as follows:

aX = abs(xPos)
aY = abs(yPos)
aZ = abs(zPos)

if( aX >= aY && aX >= aZ ) {
	if( xPos > 0 ) {
		w = 1
		h = 0
		hPos′′ = -zPos / aX
		vPos′′ = -yPos / aX
	}
	else {
		w = 1
		h = 1
		hPos′′ = zPos / aX
		vPos′′ = -yPos / aX
	}
}
else if( aY >= aX && aY >= aZ ) {
	if( yPos > 0 ) {
		w = 0
		h = 1
		hPos′′ = xPos / aY
		vPos′′ = zPos / aY
	}
	else {
		w = 2
		h = 1
		hPos′′ = xPos / aY
		vPos′′ = -zPos / aY
	}
}
else {
	if( zPos > 0 ) {
		w = 0
		h = 0
		hPos′′ = xPos / aZ
		vPos′′ = -yPos / aZ
	}
	else {
		w = 2
		h = 0
		hPos′′ = -xPos / aZ
		vPos′′ = -yPos / aZ
	}
}

/* adjustment of distance between samples */
if( geometryType = = HEC ) {
	hPos′ = 4.0 / S_PI*satan(hPos′′)
	if( ( w  = =  2  &&  h  = =  1 ) || ( w  = =  0  &&  h  = =  1 ) ) { /*  top face or bottom face */
		vPos′ = 4.0 / S_PI * satan( vPos′′ )
	}
	else {
		t = 0.4 * vPos′′ * (hPos′′*hPos′′-1.0)
		vPos′ = sfabs(t) < S_EPS ? vPos′′ : (1.0 - ssqrt(1.0-4.0*t*(vPos′′-t)))  /  (2.0 * t)
	}
}
else if(geometryType = = EAC ) {
	hPos′ =  4.0 / S_PI * satan( hPos′′ )
	vPos′ =  4.0 / S_PI * satan( vPos′′ )
}
else { /* geometryType = = CMP */
	hPos′ = hPos′′
	vPos′ = vPos′′
}


/* conversion from face cords ranging from -1.0 to 1.0, to cords scaled to unit face width/ height*/
// convert hPos′, vPos′ to [0, width], [0, height]
tmpHorVal = (( hPos′ + 1.0 ) * ( UnitFaceWidth >> 1 ) + ( -0.5 ))
tmpVerVal = (( vPos′ + 1.0 ) * ( UnitFaceHeight >> 1 ) + (-0.5))
hPos = w * UnitFaceWidth + tmpHorVal
vPos = h * UnitFaceHeight + tmpVerVal

#### Conversion of sample locations from unit cube face cords to coded cube face coords

Inputs to this process are:

–	the centre point of the sample location (hPos, vPos) within the projected region in relative projected picture sample units, where hPos and vPos may have non-integer real values.

–	the width and the height (projRegWidth, projRegHeight) of the projected region, in relative projected picture sample units,

–	the width and the height (packedRegWidth, packedRegHeight) of the packed region, in relative packed picture sample units,

–	transform type (transformType), and

–	offset values for the sampling position (offsetX, offsetY) in the range of 0, inclusive, to 1, exclusive, in horizontal and vertical relative packed picture sample units, respectively.

NOTE – offsetX and offsetY both equal to 0.5 indicate a sampling position that is in the centre point of a sample in packed picture sample units.

Outputs of this process are:

–	sample location (x, y) within the packed region, where x and y are in relative packed picture sample units

The outputs are derived as follows:

if( transformType  = =  0  | |  transformType  = =  1  | |  transformType  = =  2  | |  transformType  = =  3 ) {
	horRatio = packedRegWidth ÷ projRegWidth
	verRatio = packedRegHeight ÷ projRegHeight
} else if ( transformType  = =  4  | |  transformType  = =  5  | |  transformType  = =  6  | |
	transformType  = =  7 ) {
	horRatio = packedRegHeight ÷ projRegWidth
	verRatio = packedRegWidth ÷ projRegHeight
}
if( transformType  = =  0 ) {
	x = horRatio * ( hPos) + offsetX
	y = verRatio * ( vPos ) + offsetY
} else if ( transformType  = =  1 ) {
	x = horRatio * ( packedRegWidth − hPos) − offsetX
	y = verRatio * ( vPos) + offsetY
} else if ( transformType  = =  2 ) {
	x = horRatio * ( packedRegWidth − hPos) − offsetX
	y = verRatio * ( packedRegHeight – vPos ) − offsetY	(D-59)
} else if ( transformType  = =  3 ) {
	x = horRatio * ( hPos) + offsetX
	y = verRatio * ( packedRegHeight − vPos) − offsetY
} else if ( transformType  = =  4 ) {
	x = horRatio * ( vPos ) + offsetY
	y = verRatio * ( hPos ) + offsetX
} else if ( transformType  = =  5 ) {
	x = horRatio * ( packedRegHeight − vPos) − offsetY
	y = verRatio * ( hPos ) + offsetX
} else if ( transformType  = = 6 ) {
	x = horRatio * ( packedRegHeight − vPos ) − offsetY
	y = verRatio * ( packedRegWidth − hPos ) − offsetX
} else if ( transformType  = =  7 ) {
	x = horRatio * ( vPos ) + offsetY
	y = verRatio * ( packedRegWidth – hPos ) − offsetX
}

## Implementation status

The software of CE13.5.2b already uses a buffer based approach for geometry padding. The approach is also already used for DBF and ALF, but SAO still needs to be adapted. Since there are no tiles in VTM yet, the empty tile idea could not be implemented/tested yet.

# Other applications

Other possible applications for uncoded areas along with independent standards are:

- Additional reference pictures:
Instead of adding reference pictures, an uncoded area can be added to reference pictures, which is filled by an external process. The encoder only has to modify the motion vectors to enable this.

- Multi-layer coding:
An uncoded area of the same size as the picture can be used to generate a projection of another view which is then used for inter-view prediction.

- Scalable coding:
A picture could be partitioned into three areas. An area for the base layer, an area for the enhancement layer and an uncoded area. In the uncoded area an upsampled version of the base layer can be generated. In this case however, this would require access to the same picture when predicting the enhancement layer from the upsampled base layer.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.