| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0553-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0301 (Non-CE: Loop filter line buffer reduction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 chia-ming.tsai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-M0301 proposed by Huawei. JVET-M0301 proposes a mechanism of reducing the line buffer requirement of adaptive loop filter (ALF). The verification task was done, and the BD-rate results match exactly with those in JVET-M0301.

# Verification

Huawei provided the source code of their proposed method described in JVET-M0301 [1].

Three tests are carried out by using the VTM3.0 software [3] and conducted under the the common test conditions (CTC) defined in [2] with ALF SIMD turned off in both the anchor and tests.

The BD-rates of both tests from our experimental results match exactly of those in JVET-M0301.

Table 1. Crosscheck results for the virtual boundary is 4 lines above the CTU boundary and without conditional disabling of ALF filtering.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.15% | 0.14% | 105% | 107% |
| Class A2 | 0.08% | 0.07% | 0.07% | 96% | 102% |
| Class B | 0.05% | 0.05% | 0.05% | 102% | 107% |
| Class C | 0.06% | 0.04% | 0.06% | 104% | 105% |
| Class E | 0.08% | 0.06% | 0.02% | 92% | 91% |
| Overall | 0.06% | 0.07% | 0.06% | 100% | 103% |
| Class D | 0.03% | 0.04% | 0.05% | 93% | 91% |
| Class F | 0.07% | 0.04% | 0.04% | 101% | 106% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.03% | 0.02% | 105% | 104% |
| Class A2 | 0.12% | 0.13% | 0.13% | 105% | 95% |
| Class B | 0.08% | 0.15% | 0.07% | 101% | 104% |
| Class C | 0.10% | 0.12% | 0.21% | 103% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.12% | 0.11% | 103% | 104% |
| Class D | 0.05% | -0.14% | 0.02% | 99% | 102% |
| Class F | 0.06% | 0.01% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | 0.03% | -0.12% | 102% | 104% |
| Class C | 0.11% | 0.43% | 0.38% | 96% | 100% |
| Class E | 0.12% | 0.48% | 0.21% | 101% | 108% |
| Overall | 0.11% | 0.27% | 0.13% | 99% | 104% |
| Class D | 0.07% | 0.01% | 0.24% | 99% | 103% |
| Class F | 0.21% | -0.09% | 0.21% | 106% | 109% |

Table 2. Crosscheck results for the virtual boundary is 6 lines above the CTU boundary and without conditional disabling of ALF filtering.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.15% | 0.14% | 101% | 112% |
| Class A2 | 0.13% | 0.07% | 0.07% | 103% | 113% |
| Class B | 0.09% | 0.05% | 0.05% | 98% | 105% |
| Class C | 0.08% | 0.04% | 0.06% | 92% | 91% |
| Class E | 0.15% | 0.06% | 0.02% | 95% | 98% |
| Overall | 0.11% | 0.07% | 0.06% | 98% | 103% |
| Class D | 0.05% | 0.05% | 0.05% | 92% | 91% |
| Class F | 0.12% | 0.05% | 0.05% | 99% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.04% | 0.08% | 100% | 107% |
| Class A2 | 0.16% | 0.04% | 0.15% | 100% | 110% |
| Class B | 0.12% | 0.16% | 0.03% | 99% | 104% |
| Class C | 0.12% | 0.06% | 0.13% | 102% | 108% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | 0.09% | 0.09% | 100% | 107% |
| Class D | 0.07% | -0.03% | 0.14% | 100% | 105% |
| Class F | 0.12% | 0.02% | 0.04% | 98% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.15% | -0.21% | -0.17% | 100% | 103% |
| Class C | 0.14% | 0.29% | 0.29% | 98% | 105% |
| Class E | 0.33% | 0.22% | 0.05% | 100% | 106% |
| Overall | 0.19% | 0.06% | 0.04% | 99% | 104% |
| Class D | 0.09% | 0.38% | 0.00% | 100% | 105% |
| Class F | 0.31% | 0.28% | 0.07% | 102% | 109% |

Table 3. Crosscheck results for the virtual boundary is 4 lines above the CTU boundary and with conditional disabling of ALF filtering.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.22% | 0.20% | 110% | 113% |
| Class A2 | 0.09% | 0.09% | 0.08% | 99% | 105% |
| Class B | 0.05% | 0.06% | 0.06% | 104% | 109% |
| Class C | 0.04% | 0.05% | 0.07% | 100% | 105% |
| Class E | 0.08% | 0.12% | 0.10% | 93% | 92% |
| Overall | 0.07% | 0.10% | 0.09% | 101% | 105% |
| Class D | 0.03% | 0.05% | 0.05% | 100% | 101% |
| Class F | 0.05% | 0.04% | 0.04% | 94% | 93% |
|  | Random access Main10 |  |  |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.07% | 0.06% | 99% | 106% |
| Class A2 | 0.14% | 0.15% | 0.17% | 99% | 99% |
| Class B | 0.07% | 0.12% | 0.06% | 99% | 98% |
| Class C | 0.05% | 0.12% | 0.16% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.12% | 0.11% | 100% | 101% |
| Class D | 0.03% | -0.15% | 0.13% | 97% | 98% |
| Class F | 0.03% | 0.01% | 0.07% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.09% | 0.28% | 97% | 96% |
| Class C | 0.07% | 0.51% | 0.30% | 99% | 102% |
| Class E | 0.10% | 0.19% | 0.22% | 101% | 101% |
| Overall | 0.08% | 0.18% | 0.27% | 99% | 99% |
| Class D | 0.04% | 0.26% | -0.05% | 99% | 100% |
| Class F | 0.18% | 0.30% | 0.54% | 102% | 106% |

Table 4. Crosscheck results for the virtual boundary is 6 lines above the CTU boundary and with conditional disabling of ALF filtering.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.22% | 0.20% | 97% | 103% |
| Class A2 | 0.15% | 0.09% | 0.08% | 98% | 106% |
| Class B | 0.09% | 0.06% | 0.06% | 98% | 102% |
| Class C | 0.06% | 0.05% | 0.07% | 95% | 95% |
| Class E | 0.14% | 0.12% | 0.09% | 97% | 99% |
| Overall | 0.10% | 0.10% | 0.09% | 97% | 101% |
| Class D | 0.05% | 0.05% | 0.05% | 95% | 94% |
| Class F | 0.10% | 0.05% | 0.05% | 97% | 98% |
|  | Random access Main10 |  |  |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.04% | 0.06% | 102% | 90% |
| Class A2 | 0.18% | 0.03% | 0.12% | 100% | 100% |
| Class B | 0.11% | 0.17% | -0.01% | 100% | 103% |
| Class C | 0.10% | 0.11% | 0.15% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.10% | 0.07% | 100% | 100% |
| Class D | 0.07% | -0.03% | 0.14% | 100% | 100% |
| Class F | 0.08% | 0.00% | 0.01% | 98% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | 0.02% | -0.06% | 98% | 97% |
| Class C | 0.09% | 0.29% | 0.36% | 97% | 99% |
| Class E | 0.28% | 0.38% | -0.01% | 99% | 100% |
| Overall | 0.16% | 0.20% | 0.09% | 98% | 99% |
| Class D | 0.09% | -0.16% | -0.36% | 99% | 99% |
| Class F | 0.19% | 0.21% | 0.03% | 101% | 104% |

# References

- A.M. Kotra, S. Esenlik, B. Wang, H. Gao, J. Chen, “Non-CE: Loop filter line buffer reduction,” Document of Joint Video Experts Team, JVET-M0301, Jan. 2019.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010, Oct. 2018.

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).