| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0576-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0406 (CE2/4-related: Unified merge list size for block and sub-block merge modes) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 jenny.lai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-M0406 proposed by Tencent. JVET-M0406 proposes to signal one merge size for both block-based and subblock-based merge modes, instead of separately signalling flags for individual lists with different maximum capacity.The verification task was done, and the BD-rates results match exactly with those in JVET-M0406. The encoding time percentages and decoding time percentages are slightly different from those in JVET-M0406.

# Verification

Tencent provided the source code of their proposed method described in JVET-M0406 [1].

Two tests are conducted under the the common test conditions (CTC) defined in [2] and are carried out by using the VTM3.0 software [3]. In test 1, the unified merge list size is set to 6; in test 2, the unified merge list size is set to 5. In both tests, the VTM3.0 are used as the anchor.

The BD-rates of both tests from our experimental results match exactly of those in JVET-M0406. The encoding time percentages and decoding time percentages are slightly different from those in JVET-M0406.

Table 1. Unified merge list size signaling with Max Size = 6

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.00% | 101% | 99% |
| Class A2 | -0.02% | 0.01% | 0.08% | 100% | 99% |
| Class B | -0.03% | -0.01% | -0.10% | 101% | 101% |
| Class C | -0.02% | -0.01% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | -0.02% | 100% | 100% |
| Class D | -0.05% | 0.04% | -0.06% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.33% | 0.20% | 100% | 103% |
| Class C | -0.01% | 0.23% | 0.14% | 100% | 100% |
| Class E | -0.05% | 0.16% | 0.12% | 101% | 100% |
| Overall | 0.00% | -0.02% | 0.16% | 100% | 101% |
| Class D | 0.01% | 0.21% | 0.08% | 100% | 101% |
| Class F | -0.04% | -0.08% | 0.07% | 101% | 101% |

Table 2. Unified merge list size signaling with Max Size = 5

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.01% | 100% | 99% |
| Class A2 | 0.06% | 0.09% | 0.19% | 100% | 99% |
| Class B | 0.08% | 0.16% | 0.01% | 100% | 101% |
| Class C | 0.07% | 0.06% | 0.06% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.10% | 0.06% | 100% | 101% |
| Class D | -0.01% | -0.03% | 0.03% | 100% | 101% |
| Class F | 0.05% | 0.04% | 0.04% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | -0.02% | 0.30% | 100% | 100% |
| Class C | 0.05% | 0.34% | 0.26% | 100% | 100% |
| Class E | 0.10% | -0.02% | 0.29% | 100% | 101% |
| Overall | 0.10% | 0.10% | 0.28% | 100% | 100% |
| Class D | 0.06% | 0.14% | 0.12% | 101% | 105% |
| Class F | 0.09% | 0.24% | 0.21% | 101% | 99% |

# References

- X. Xu, X. Li, S. Liu, “CE2/4-related: Unified merge list size for block and sub-block merge modes” JVET-M0406, Jan 2019.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

- VTM3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).