| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0635-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0411 (Non-CE8: Inter mode related flag signaling when current picture is the only reference picture) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 jenny.lai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-M0411 proposed by Tencent. JVET-M0411 proposes not to signal the following inter coding related flags in condition that the current picture is the only reference picture for a slice. Specifically, subblock merge flag, affine flag, mmvd flag and intra-inter flag are not signalled. The verification task was done, and the BD-rate results match exactly with those in JVET-M0411 in AI case, and have minor difference in RA case because of a CPR-related bug in VTM-3.0. The encoding time percentages and decoding time percentages are not trustable.

# Verification

Tencent provided the source code of their proposed method described in JVET-M0411 [1].

The test is conducted under the the common test conditions (CTC) defined in [2], Class F, and Class SCC 1080p and is carried out by using the VTM3.0 software [3]. It is tested with/without CPR. In this test, the VTM3.0 with CPR on and CPR off condition are used as the anchor.

The BD-rates of the test from our experimental results match exactly of those in JVET-M0411 in AI case, and have minor difference in RA case caused by a CPR-related bug in the VTM software. The encoding time percentages and decoding time percentages are not trustable.

Table 1: Results of the test

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR-off |  |  |  |  | Over VTM-3.0-CPR-on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.08% | 0.06% | 101% | 72% | 0.00% | 0.02% | 0.00% | 77% | 71% |
| Class A2 | -0.48% | -0.53% | -0.51% | 102% | 73% | 0.00% | -0.03% | 0.03% | 78% | 73% |
| Class B | -0.30% | -0.40% | -0.40% | 106% | 72% | 0.00% | -0.01% | -0.04% | 77% | 72% |
| Class C | -0.10% | -0.25% | -0.24% | 112% | 72% | -0.01% | -0.01% | 0.01% | 79% | 73% |
| Class E | -0.64% | -0.78% | -0.73% | 112% | 75% | -0.01% | 0.02% | -0.14% | 79% | 75% |
| Overall | -0.28% | -0.40% | -0.36% | 107% | 73% | 0.00% | 0.00% | -0.03% | 78% | 73% |
| Class D | -0.12% | -0.19% | 0.10% | 108% | 72% | -0.02% | 0.05% | -0.11% | 79% | 70% |
| Class F (optional) | -12.11% | -12.11% | -12.07% | 125% | 71% | -0.02% | -0.08% | 0.02% | 79% | 72% |
| Class SCC 1080p | -36.95% | -36.13% | -36.38% | 112% | 66% | -0.06% | -0.03% | -0.05% | 78% | 72% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0-CPR-off |  |  |  |  | Over VTM-3.0-CPR-on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -0.01% | 0.26% | 80% | 67% | -0.02% | 0.03% | 0.03% | 79% | 67% |
| Class A2 | -0.01% | 0.09% | 0.17% | 77% | 68% | -0.01% | 0.02% | -0.07% | 79% | 67% |
| Class B | 0.01% | 0.00% | -0.22% | 78% | 65% | 0.00% | 0.00% | -0.01% | 79% | 65% |
| Class C | 0.21% | -0.16% | 0.07% | 80% | 65% | 0.01% | -0.10% | -0.01% | 79% | 65% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.08% | -0.03% | 0.03% | 79% | 66% | 0.00% | -0.02% | -0.01% | 79% | 66% |
| Class D | 0.20% | -0.22% | 0.26% | 81% | 65% | 0.05% | 0.05% | 0.11% | 79% | 65% |
| Class F (optional) | -9.89% | -9.88% | -9.84% | 85% | 54% | 0.00% | 0.03% | 0.12% | 79% | 55% |
| Class SCC 1080p | -22.01% | -21.54% | -22.08% | 81% | 53% | 0.03% | 0.06% | 0.03% | 78% | 56% |

# References

- X. Xu. X. Li, S. Liu, “Non-CE8: Inter mode related flag signaling when current picture is the only reference picture,” JVET-M0411, Jan. 2019.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Oct. 2018.

- VTM3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).