| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0668 |
| --- | --- |

| Title: | Crosscheck of JVET-M0198 (CE7-related: Unified rice parameter derivation for coefficient level coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Muhammed Coban | Tel: Email: | mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports the crosscheck results of “CE7-related: Unified rice parameter derivation for coefficient level coding (JVET-M0198)”. The source code was analyzed, and it matches the description provided in JVET-M0198. The results provided by the proponent were verified.

# Crosscheck results

The simulation was carried out under the JVET common test condition [1]. Samsung provided the source code of their proposal and results [2]. The results match those provided by the proponent.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.06% | 101% | 95% |
| Class A2 | 0.01% | 0.02% | 0.00% | 101% | 96% |
| Class B | 0.00% | -0.01% | 0.01% | 101% | 96% |
| Class C | 0.00% | -0.04% | -0.05% | 101% | 98% |
| Class E | 0.00% | 0.03% | 0.05% | 101% | 95% |
| Overall | 0.00% | -0.01% | 0.01% | 101% | 96% |
| Class D | -0.02% | 0.02% | 0.02% | 101% | 97% |
| Class F | 0.10% | 0.23% | 0.25% | 101% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.10% | 98% | 95% |
| Class A2 | -0.01% | 0.01% | 0.10% | 98% | 95% |
| Class B | -0.02% | 0.11% | -0.04% | 99% | 98% |
| Class C | 0.00% | -0.09% | 0.05% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.04% | 99% | 97% |
| Class D | -0.02% | -0.25% | -0.17% | 100% | 99% |
| Class F | 0.17% | 0.20% | 0.12% | 100% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.10% | -0.20% | 99% | 97% |
| Class C | 0.01% | 0.35% | 0.01% | 99% | 96% |
| Class E | 0.09% | 1.99% | -0.57% | 100% | 98% |
| Overall | 0.02% | 0.66% | -0.22% | 100% | 97% |
| Class D | 0.00% | 0.02% | -0.26% | 102% | 101% |
| Class F | 0.00% | 0.07% | -0.05% | 100% | 98% |

Table 1:Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to VTM-3.0 in high-complexity encoding mode.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, October 2018.

- Y. Piao, K. Choi, “CE7-related: Unified rice parameter derivation for coefficient level coding”, JVET-M0198, Marrakech, MA, January 2019.