| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0672_v1 |
| --- | --- |

| Title: | Cross-check of JVET-M0202: CE6-realted: Simplification related to MTS with reduced modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Xiaoqiang Cao | Email: | caoxiaoqiang@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

The document reports the cross-check for the results reported in input document JVET-M0202: CE6-related: Simplification related to MTS with reduced modes. The cross-check contribution reports the results of the proposed that simplification related to MTS with reduced modes. The cross-check validated the experimental results from the proponent given in JVET-M0202.

Introduction

The source code as well as the configuration, for JVET-M0202 [1], provided by the proponent Samsung were used for the cross-check. In order to reduce encoding complexity of using MTS, the proponent present 4 testing results based on reduced modes for MTS with non-normative and normative changes as follows:

- Test1: Non-normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test2:Non-normative change with 2modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test3:Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}

- Test4:Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}

In this contribution, I will cross-check the Test1 and Test3 using configuration as same as proponent Samsung.

Experimental Results

The proposed methods has been implemented based on VTM-3.0 reference software, and simulations are performed using VTM configurations under common test condition defined in JVET-L1010[2] and Core Experiment description in JVET-L1026[3]. The results are generated using configuration for JVET-M0202 provided by the proponents for simplification related to MTS with reduced modes.

1. VTM3.0 VS Test1

2. VTM3.0 VS Test3

3. （VTM3.0+interMTS）VS（Test3+interMTS）

Conclusion

No discrepancies were found in the software and the description of the test. For many rate points in the RA configurations, there is a minor mismatch in the BD-rate/PSNR values; however, none are significant enough to affect the BD-rate numbers by more than 0.01%. It may be attributed to the different cluster platforms used. The encoding time and decoding time are within the measurement error range.

Reference

[1] Kiho Choi, Minsoo Park, Min Woo Park, et al. “CE6-realted: Simplification related to MTS with reduced modes”, JVET-M0202, Jan. 2019.

[2] Frank Bossen, Jill Boyce, Karsten Suehring, et al, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Oct. 2018.

[3] Amir Said, Xin Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, JVET-L1026, JVET-L1026, Oct. 2018.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).