| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0680 |
| --- | --- |

| Title: | Crosscheck of JVET-M0158 (Non-CE3: LUT-free interpolation filters for intra prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fabien Racapé | Email: | fabien.racape@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This document reports the crosscheck results of JVET-M0158. It is reported that the average BD-rates match those provided by the proponents. No discrepancies have been found in the software and the description.

# Description

In tJVET-M0158, it is proposed to use the following analytic representations of filter coefficients for both interpolation filters (DCT-IF and Gaussian filter) rather than LUT-based ones.

For replacing the Gaussian filter:

tmpDelta = deltaFract >> 1;

f[0] = 16 - tmpDelta;

f[1] = 32 - tmpDelta;                                                                                                      (1)

f[2] = 16 + tmpDelta;

f[3] =      tmpDelta;

where tmpDelta denotes fractional sample position.

For the the DCT-IF:

tmpDelta = (deltaFract & 0x10 ? -(~(31 ^ deltaFract)) : deltaFract);

tmpDelta = ((tmpDelta << 1) + tmpDelta) >> 3;

f[0] =                        - tmpDelta;

f[1] = 64 - (deltaFract << 1) + tmpDelta;                                                                (2)

f[2] =      (deltaFract << 1) + tmpDelta;

f[3] =                        - tmpDelta;

| Test # | Description |
| --- | --- |
| 1 | Gaussian filter is replaced by its analytic representation according to (1) |
| 2 | Both DCT-IF and Gaussian filter are replaced by their analytic representations according to (1) and (2), respectively |
| 3 | Test 2, where the least significant bit of deltaFract is set to zero when computing sharpening filter coefficients (16 entries in Table 2) |
| 4 | Test 1, where DCT-IF table is reduced to 16 entries (the least significant bit of deltaFract is set to zero), chroma interpolation filter also uses 16 phases (the least significant bit of deltaFract is set to zero) |
| 5 | Test 3, where chroma interpolation filter uses 16 phases (the least significant bit of deltaFract is set to zero) |

# Experimental Results

Experiments were performed based on VTM3.0, using the common test conditions described in JVET-L1010. For test 2, some runtime inconsistences are noticed, which are due to the cluster.

## Test 1:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.06% | 100% | 102% |
| Class A2 | 0.00% | 0.05% | -0.01% | 102% | 105% |
| Class B | 0.00% | -0.02% | -0.05% | 102% | 103% |
| Class C | 0.00% | -0.09% | -0.11% | 103% | 102% |
| Class E | 0.01% | -0.05% | 0.10% | 104% | 103% |
| Overall | 0.00% | -0.02% | -0.01% | 102% | 103% |
| Class D | 0.01% | 0.08% | -0.13% | 104% | 107% |
| Class F | -0.01% | 0.02% | 0.09% | 103% | 103% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | 0.05% | 102% | 104% |
| Class A2 | -0.03% | 0.00% | 0.13% | 105% | 105% |
| Class B | -0.01% | 0.10% | 0.02% | 101% | 105% |
| Class C | -0.01% | -0.07% | 0.17% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.09% | 102% | 103% |
| Class D | 0.03% | -0.40% | -0.41% | 101% | 101% |
| Class F | -0.03% | 0.09% | 0.14% | 102% | 102% |

## Test 3:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.09% | 100% | 102% |
| Class A2 | -0.01% | -0.01% | 0.01% | 103% | 104% |
| Class B | 0.01% | 0.00% | -0.02% | 102% | 103% |
| Class C | 0.03% | -0.02% | -0.07% | 102% | 101% |
| Class E | 0.00% | -0.08% | 0.09% | 103% | 103% |
| Overall | 0.01% | -0.03% | 0.01% | 102% | 103% |
| Class D | 0.00% | -0.05% | -0.13% | 105% | 107% |
| Class F | 0.12% | 0.08% | 0.20% | 102% | 102% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.11% | 0.01% | 99% | 99% |
| Class A2 | -0.03% | -0.04% | 0.17% | 103% | 100% |
| Class B | 0.00% | 0.09% | -0.11% | 101% | 104% |
| Class C | 0.00% | 0.04% | 0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | 0.01% | 101% | 101% |
| Class D | -0.05% | -0.31% | -0.14% | 100% | 101% |
| Class F | 0.05% | 0.22% | 0.11% | 101% | 100% |

## Test 4:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.09% | 101% | 102% |
| Class A2 | -0.01% | -0.01% | 0.01% | 100% | 102% |
| Class B | 0.01% | 0.00% | -0.02% | 101% | 102% |
| Class C | 0.03% | -0.02% | -0.07% | 102% | 101% |
| Class E | 0.00% | -0.08% | 0.09% | 103% | 102% |
| Overall | 0.01% | -0.03% | 0.01% | 101% | 102% |
| Class D | 0.00% | -0.05% | -0.13% | 105% | 107% |
| Class F | 0.12% | 0.08% | 0.20% | 101% | 101% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.11% | 0.01% | 101% | 100% |
| Class A2 | -0.03% | -0.04% | 0.17% | 104% | 101% |
| Class B | 0.00% | 0.09% | -0.11% | 101% | 103% |
| Class C | 0.00% | 0.04% | 0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | 0.01% | 101% | 101% |
| Class D | -0.05% | -0.31% | -0.14% | 100% | 100% |
| Class F | 0.04% | 0.03% | -0.06% | 101% | 101% |

## Test 5:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.09% | 99% | 100% |
| Class A2 | -0.01% | -0.01% | 0.01% | 103% | 103% |
| Class B | 0.01% | 0.00% | -0.02% | 101% | 102% |
| Class C | 0.03% | -0.02% | -0.07% | 102% | 101% |
| Class E | 0.00% | -0.08% | 0.09% | 103% | 102% |
| Overall | 0.01% | -0.03% | 0.01% | 102% | 102% |
| Class D | 0.00% | -0.05% | -0.13% | 104% | 106% |
| Class F | 0.12% | 0.08% | 0.20% | 102% | 103% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.11% | 0.01% | 102% | 101% |
| Class A2 | -0.03% | -0.04% | 0.17% | 104% | 102% |
| Class B | 0.00% | 0.09% | -0.11% | 101% | 105% |
| Class C | 0.00% | 0.04% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | 0.01% | 101% | 102% |
| Class D | -0.05% | -0.31% | -0.14% | 100% | 100% |
| Class F | 0.04% | 0.03% | -0.06% | 101% | 101% |

# Patent rights declaration

Technicolor does not have any current or pending patent rights relating to the technology described in this contribution.