| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0685 |
| --- | --- |

| Title: | Non-CE7: On derivation of quantization parameter predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Misra, A. Segall | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

In the current VVC WD, the derivation of quantization parameter predictor qPY_PRED may in some cases be based on a previous quantization group in decoding order that is not spatially close to the current coding unit (CU). For example, the previous quantization group in decoding order may belong to the rightmost Coding Tree Unit (CTU) in the CTU row above. This contribution therefore proposes two possible options to address the above:

Option 1: qPY_PRED be derived based on spatial neighbors (when available)

Option 2: A high-level flag is used to control the quantization parameter predictor derivation process, which enables support for both Option 1 and the current design in the VVC WD if so desired.

# Introduction

In current VVC WD [1], qPY_PRED is typically computed as:

qPY_PRED = ( qPY_A + qPY_B + 1 ) >> 1

where

qPY_A in most cases, is set equal to the QpY of the coding unit (CU) to the left of the current CU;

qPY_B in most cases, is set equal to the QpY of the coding unit above the current CU.

However, in some cases, each of qPY_A and qPY_B may be set equal to a variable qPY_PREV, where qPY_PREV is set equal to the QpY of the last coding unit in the previous quantization group in decoding order or is set equal to SliceQpY. Such calculations of qPY_PRED may be less than ideal because the last CU in the previous quantization group in decoding order may not be spatially close to the current CU and might therefore not serve as a good predictor (or not even known at the encoder due to the use of non-normative, parallelization strategies). In some cases, the previous quantization group in decoding order may belong to the rightmost Coding Tree Unit (CTU) in the CTU row above. The contribution therefore proposes that qPY_PRED be computed as follows:

qPY_PRED = ( qPY_A + qPY_B + 1 ) >> 1; when both qPY_A and qPY_B are available;

qPY_PRED = qPY_A; when only qPY_A is available;

qPY_PRED = qPY_B; when only qPY_B is available; or

qPY_PRED = SliceQpY; when neither of qPY_A and qPY_B are available.

We label this approach Option 1.

Alternatively, a high-level flag is used to control the quantization parameter predictor derivation employed with Option 1 being used when the flag is 1, Otherwise the current design in VVC WD is employed

Changes to the VVC WD are listed in appendix included at the end of this document.

# Recommendations

The contribution proposes two possible options that enables derivation of quantization parameter predictor based solely on spatial neighbors. It is recommended that the committee adopt one of these options in the VVC WD.

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001 12th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, 3–12 Oct. 2018

# Patent rights declaration(s)

Sharp Laboratories of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## Proposed changes to VVC WD for Option 1

Modifications are highlighted in yellow.

## 8.4 Scaling, transformation and array construction process

### 8.4.1. Derivation process for quantization parameters

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left luma sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed.

In this process, the luma quantization parameter Qp′Y and the chroma quantization parameters Qp′Cb and Qp′Cr are derived.

The luma location ( xQg, yQg ), specifies the top-left luma sample of the current quantization group relative to the top left luma sample of the current picture. The horizontal and vertical positions xQg and yQg are set equal to CuQgTopLeftX and CuQgTopLeftY, respectively.

NOTE – : The current quantization group is a rectangluar region inside a coding tree block that shares the same qPY_PRED. Its width and height are equal to the width and height of the coding tree node of which the top-left luma sample position is assigned to the variables CuQgTopLeftX and CuQgTopLeftY.

When treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the predicted luma quantization parameter qPY_PRED is derived by the following ordered steps:

- The variable qPY_PREV is derived as follows:

–	If one or more of the following conditions are true, qPY_PREV is set equal to SliceQpY:

–	The current quantization group is the first quantization group in a slice.

–	The current quantization group is the first quantization group in a tile.

–	Otherwise, qPY_PREV is set equal to the luma quantization parameter QpY of the last luma coding unit in the previous quantization group in decoding order.

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg − 1, yQg ) as inputs, and the output is assigned to availableA. The variable qPY_A is derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREV:

–	availableA is equal to FALSE.

–	the CTB address ctbAddrA of the CTB containing the luma coding block covering the luma location ( xQg − 1, yQg ) is not equal to CtbAddrInTs, where ctbAddrA is derived as follows:

xTmp = ( xQg − 1 )  >>  MinTbLog2SizeY
yTmp = yQg  >>  MinTbLog2SizeY
minTbAddrA = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrA = minTbAddrA  >>  ( 2  * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-786)

–	Otherwise, When availableA is equal to TRUE, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg ).

3.  The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg, yQg − 1 ) as inputs, and the output is assigned to availableB. The variable qPY_B is derived as follows:

–	If one or more of the following conditions are true, qPY_B is set equal to qPY_PREV:

–	availableB is equal to FALSE.

–	the CTB address ctbAddrB of the CTB containing the luma coding block covering the luma location ( xQg,  yQg − 1 ) is not equal to CtbAddrInTs, where ctbAddrB is derived as follows:

xTmp = xQg  >>  MinTbLog2SizeY
yTmp = ( yQg − 1 )  >>  MinTbLog2SizeY
minTbAddrB = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrB = minTbAddrB  >>  ( 2 * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-787)

–	Otherwise, When availableB is equal to TRUE,  qPY_B is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ).

- The predicted luma quantization parameter qPY_PRED is derived as follows:

qPY_PRED = ( qPY_A + qPY_B + 1 )  >>  1		(8-788)

–	When both availableA and availableB are equal to TRUE, qPY_PRED is assigned ( qPY_A + qPY_B + 1 ) >> 1

–	When availableA is equal to TRUE and availableB is equal to FALSE, qPY_PRED is assigned qPY_A

–	When availableA is equal to FALSE and availableB is equal to TRUE, qPY_PRED is assigned qPY_B

–	When both availableA and availableB are equal to FALSE, qPY_PRED is assigned SliceQpY

## Proposed changes to VVC WD for Option 2

Modifications are highlighted in yellow.

## 8.4 Scaling, transformation and array construction process

### 8.4.1. Derivation process for quantization parameters

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left luma sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed.

In this process, the luma quantization parameter Qp′Y and the chroma quantization parameters Qp′Cb and Qp′Cr are derived.

The luma location ( xQg, yQg ), specifies the top-left luma sample of the current quantization group relative to the top left luma sample of the current picture. The horizontal and vertical positions xQg and yQg are set equal to CuQgTopLeftX and CuQgTopLeftY, respectively.

NOTE – : The current quantization group is a rectangluar region inside a coding tree block that shares the same qPY_PRED. Its width and height are equal to the width and height of the coding tree node of which the top-left luma sample position is assigned to the variables CuQgTopLeftX and CuQgTopLeftY.

When treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the predicted luma quantization parameter qPY_PRED is derived by the following ordered steps:

- When sps_spatial_qp_pred_flag is equal to 0, the variable qPY_PREV is derived as follows:

–	If one or more of the following conditions are true, qPY_PREV is set equal to SliceQpY:

–	The current quantization group is the first quantization group in a slice.

–	The current quantization group is the first quantization group in a tile.

–	Otherwise, qPY_PREV is set equal to the luma quantization parameter QpY of the last luma coding unit in the previous quantization group in decoding order.

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg − 1, yQg ) as inputs, and the output is assigned to availableA.

– When sps_spatial_qp_pred_flag is equal to 0, the variable qPY_A is derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREV:

–	availableA is equal to FALSE.

–	the CTB address ctbAddrA of the CTB containing the luma coding block covering the luma location ( xQg − 1, yQg ) is not equal to CtbAddrInTs, where ctbAddrA is derived as follows:

xTmp = ( xQg − 1 )  >>  MinTbLog2SizeY
yTmp = yQg  >>  MinTbLog2SizeY
minTbAddrA = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrA = minTbAddrA  >>  ( 2  * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-786)

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg ).

– Otherwise, when availableA is equal to TRUE, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg ).

3.  The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg, yQg − 1 ) as inputs, and the output is assigned to availableB.

– When sps_spatial_qp_pred_flag is equal to 0, the variable qPY_B is derived as follows:

–	If one or more of the following conditions are true, qPY_B is set equal to qPY_PREV:

–	availableB is equal to FALSE.

–	the CTB address ctbAddrB of the CTB containing the luma coding block covering the luma location ( xQg,  yQg − 1 ) is not equal to CtbAddrInTs, where ctbAddrB is derived as follows:

xTmp = xQg  >>  MinTbLog2SizeY
yTmp = ( yQg − 1 )  >>  MinTbLog2SizeY
minTbAddrB = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrB = minTbAddrB  >>  ( 2 * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-787)

–	Otherwise, When availableB is equal to TRUE,  qPY_B is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ).

– Otherwise, When availableB is equal to TRUE,  qPY_B is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ).

- The predicted luma quantization parameter qPY_PRED is derived as follows:

– When sps_spatial_qp_pred_flag is equal to 0

qPY_PRED = ( qPY_A + qPY_B + 1 )  >>  1		(8-788)

– Otherwise

–	When both availableA and availableB are equal to TRUE, qPY_PRED is assigned ( qPY_A + qPY_B + 1 ) >> 1

–	When availableA is equal to TRUE and availableB is equal to FALSE, qPY_PRED is assigned qPY_A

–	When availableA is equal to FALSE and availableB is equal to TRUE, qPY_PRED is assigned qPY_B

–	When both availableA and availableB are equal to FALSE, qPY_PRED is assigned SliceQpY

### 7.3.2 Raw byte sequence payloads, trailing bits and byte alignment syntax

#### 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_spatial_qp_pred_flag | u(1) |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### 7.4.3 Raw byte sequence payloads, trailing bits and byte alignment semantics

#### 7.4.3.1 Sequence parameter set RBSP semantics

sps_spatial_qp_pred_flag equal to 1 specifies that the luma quantization parameter predictor is derived using available spatial neighbors and slice QP. sps_spatial_qp_pred_flag equal to 0 specifies that the luma quantization parameter predictor is derived using available spatial neighbors, previous quantization group in decoding order and slice QP.