| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0712_v1 |
| --- | --- |

| Title: | Crosscheck for M0507- Hybrid Merge Estimation Region |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Email: Email: Email: | chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | HIKVISION |  |  |

_____________________________

# Abstract

This contribution is a partial crosscheck report for CE-related proposal (M0507: Hybrid Merge Estimation Region). A method of reducing the local inter-dependency as well as the number of merge candidate list generation operations is proposed. For splitting nodes with the size of N2, all merge candidate list generation operations within the area can be performed in parallel. And for all blocks within a splitting area with the size of N1, a shared merge candidate list is used.

- Introduction

In order to test the proposed method, a macro needed to be set as follows in the Typedef.h:

#define MRG_SHARELIST                   1

#if MRG_SHARELIST

#define MRG_SHARELIST_DECGENONLEAF      1

#define MRG_SHARELIST_OFFIMGSIZEEN      0

#if !MRG_SHARELIST_OFFIMGSIZEEN

#define OFFIMG_FIX                      1

#endif

#define MRG_SHARELIST_SUBMRGLIST        1

#define MRG_SHARELIST_TRIMERGE          1

#define NODE_MULTI_PARTITION            1

#define MY_DEBUG                        0

#define VERIFY_MSL_MER                  0

#define MRG_SHARELIST_OFF_NORMERGESHARE 0

#define MRG_SHARELIST_SYNTAX            1

#if MRG_SHARELIST_SYNTAX

#define MRG_SHARELIST_SHARSIZE          0

#define MRG_SHARELIST_SYNTAX_MINTHD     32 //original config : 64     ifwant to chg to 32, it's ok (but var naming become strange)

#endif

#endif//MRG_SHARELIST

Besides, some configurations are needed to set the MRG size and share list size.

Test1:

Share list size = 128

SEIDecodedPictureHash: 	1

SharedMrg : 1

Log2SharedMrgThdMinus6:    2

Log2ParallelMergeLevel:      2

Test2:

MER size = 128

SEIDecodedPictureHash: 	1

SharedMrg :                0

Log2SharedMrgThdMinus6:   0

Log2ParallelMergeLevel:     7

Test3:

MER size = 128, share list size = 32

SEIDecodedPictureHash: 	 1

SharedMrg :                 1

Log2SharedMrgThdMinus6:    0

Log2ParallelMergeLevel:      7

The testing conditions defined in JVET-L1024 [1] were used. The VTM3.0 is set as anchor. Two configurations were tested: RA and LB. The source code was compiled using GCC v4.8.5, and the test jobs ran on an Intel(R) Xeon E5-2680 v4 server.

The available results are shown in the following tables, and detailed data can be found in the attached spreadsheet.

- Experimental results

Table 1 - Results of Test1

Table 2 - Results of Test2

Table 3 - Results of Test3

- References

[1] Haitao Yang, Shan Liu, Kai Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-L1024, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.