| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0725-v1 |
| --- | --- |

| Title: | CE5: Results of tests CE5.1.1 and CE5.1.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jan Stegemann, Heiner Kirchhoffer, Heiko Schwarz, Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-183 jan.stegemann@hhi-extern.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the results of the tests CE5.1.1, and CE5.1.2. These tools include counter-based probability update with fixed and variable window sizes and a 32x8x8-bit table containing the pre-calculated values of the LPS range table. The average coding gains are 0.81% for AI, 0.71% for RA, 0.60% for LB and 0,60% for LP (CE5.1.1) and 0.97% for AI, 0.97% for RA, 0.89% for LB and 0,83% for LP (CE5.1.2)

# Introduction

The performance of the counter-based probability estimation using two counter variables is beneficial in terms of coding efficiency compared to the CABAC state machine of HEVC. While the window sizes for all context models are set to the binary logarithm na=4 and nb=7 (fixed) in test CE5.1.1, test CE5.1.2 uses custom window sizes in the range of the binary logarithm of 2 and 9 (variable). A 4-bit for each context model is specified for the derivation of na and nb as follows:

na=2+x>>2 & 3

nb=3+ na+x & 3.

Both tests use a retrained set of context model initialization values, a 10-bit and A 14-bit counter variables and a 32x8x8bit table for the interval subdivision as specified in [1].

# Results of experiments

The proposed changes are integrated into vtm-3.0. The simulations are performed using the common test conditions.

Table 1 - CE5.1.1 Fixed window sizes counter-based probability estimator

Table 2 - CE5.1.2 Custom window sizes (variable) probability estimator

# Reference

- H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine,” JVET-L1025, 12th Meeting, Macao, China, October 2018.

# Patent rights declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).