| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0737-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0432 (CE2-related: Alignment of affine control-point motion vector and subblock motion vector) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Anish Tamse Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the cross check results of “CE2-related: Alignment of affine control-point motion vector and subblock motion vector”. The simulation was carried out under CTC. The results provided by the proponent were verified.

# Cross Check Results

The simulation was carried out under the JVET common test condition [1]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Proponent provided the source code of their proposal based on the VTM-3.0 software and results. The results match with those provided by the proponent.

In a JVET-M0432, a combined method is proposed to keep affine inheritance from affine HMVP as in CE2.2.3.d, and the original affine inheritance from motion data line buffer when neighboring candidate is above the current CTU boundary. Table 1 shows the results of the proposed method.

Table 1: Summary of the BD-rates, EncT, and DecT for Test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | -0.03% | 94% | 93% |
| Class A2 | 0.03% | 0.14% | 0.13% | 93% | 93% |
| Class B | 0.04% | 0.09% | 0.12% | 97% | 92% |
| Class C | 0.02% | 0.08% | 0.09% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.09% | 0.08% | 96% | 95% |
| Class D | 0.12% | 0.12% | 0.06% | 100% | 104% |
| Class F (optional) | 0.02% | 0.02% | -0.06% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | -0.03% | 0.06% | 90% | 87% |
| Class C | 0.09% | 0.25% | 0.05% | 95% | 87% |
| Class E | -0.03% | 0.24% | -0.62% | 96% | 83% |
| Overall | 0.07% | 0.13% | -0.11% | 93% | 86% |
| Class D | 0.20% | 0.15% | 0.77% | 99% | 93% |
| Class F (optional) | 0.03% | -0.20% | 0.32% | 96% | 90% |

# Reference

[1] “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao,  October 2018.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).