| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MO, 9–18 Jan. 2019 | Document: JVET-M0756 v2 |
| --- | --- |

| Title: | CE2.2.7 related: Affine temporal constructed candidates without pruning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, A. Robert, F. Le Léannec, T. Poirier | Email: | franck.galpin@technicolor.com fabrice.leleannec@technicolor.com antoine.robert@technicolor.com tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution is similar to test A of CE2.2.7 removing the additional pruning introduced in CE2.2.7. The overall complexity of the constructed affine candidates both in terms of comparisons and CPMV constructions is reported to be reduced compared to VTM-3.0. The simulation results reportedly show slight gains for CTC configurations with higher gains for sequences containing affine motion.

# Introduction

In the current design of Affine Merge motion predictor, 6 constructed candidates are added after the inherited candidates. These candidates are built using 3 spatial and 1 temporal motion vectors.

This contribution proposes to replace some constructed spatio-temporal candidates by purely temporal affine candidates.

# Description

The constructed temporal candidates are the same as the ones proposed in JVET-L0156. The temporal motion vector process is simplified by taking the motion vectors inside the CU in order to avoid complex processing for CTU boundaries cases.

The candidate list length is kept the same (5 candidates). The simulations are conducted using the VTM configuration as described in JVET-L1010 [1]. VTM-3.0 reference software is used with VTM configuration files.

The number of tested candidates on encoder side is changed to have the same number as in regular merge list (4 SATD candidates) in order to show the impact of added candidates at the bottom of the list.

The VTM-3.0 complexity is assumed as follow:

| Merge list size | Max number of potential candidates | Max number of mv comparisons | Max number of ref idx comparisons | Max mv scaling |
| --- | --- | --- | --- | --- |
| 5 | 1 (ATMVP) + 2 (affine inherited) + 6 (affine constructed) = 9 | (3 + 0 + (4*2+2*1))*2 = 26 | (3 + 0 + (4*2+2*1))*2 = 26 | 1*2 |

Note that encoding/decoding time are not fully reliable, and complexity is better analyzed through complexity tables provided.

Figure 1 CPMV location

Figure 1 shows the location of the motion vectors used to compute the constructed model in VTM-3.0. The table below gives the constructed candidates list:

| model | corners used | CPMV computation |
| --- | --- | --- |
| 0 | LT, RT, LB | 0 |
| 1 | LT, RT, RB | 2 (LB) |
| 2 | LT, LB, RB | 2 (RT) |
| 3 | RT, LB, RB | 2 (LT) |
| 4 | LT, RT | 0 |
| 5 | LT, LB | 2 (RT) |

In model 1, 2 and 3, the constructed model used a mix of spatial and temporal motion vectors.

Moreover, in order to construct these models, additional computations are needed to compute the missing CPMV for both lists.

# Proposal description

In the proposed solution, 3 constructed CPMVs are removed (the one mixing spatial and temporal motion vectors, i.e. models 1, 2 and 3) and replaced by 3 purely temporal constructed affine candidates. The table below shows the CPMV used in the proposal:

| model | CPMV used | CPMV computation |
| --- | --- | --- |
| 0 | LT, RT, LB spatial | 0 |
| 1 | LT, RT, LB temporal | 0 |
| 2 | LT, RT spatial | 0 |
| 3 | LT, RT temporal | 0 |
| 4 | LT, LB spatial | 2 |
| 5 | LT, LB temporal | 2 |

## Complexity

The table below show the complexity changes compared to VTM-3.0.

| Merge list size | Max number of potential candidates | Max number of mv comparisons | Max number of ref idx comparisons | Max mv scaling |
| --- | --- | --- | --- | --- |
| 5 | 1 (ATMVP) + 2 (affine inherited) + 3*2 (affine constructed) = 9 | (3 + 0 + (1*2+2*1)*2)*2 = 22 | (3 + 0 + (1*2+2*1)*1)*2 = 14 | 3*2 |

The overall complexity is as follow:

- reduced number of mv comparisons: from 26 to 22 (-4)

- reduced number of refidx comparisons: from 26 to 14 (-12)

- reduced CPMV computation: from 8 to 4 (-4)

- increased mv scaling: from 2 to 6 (+4)

- temporal motion vectors extraction simplified

# Simulation results using 4 SATD candidates

## Results against VTM-3.0

Tables below show the simulation results of the proposed method in random access and low delay B configurations. In both configurations, the proposed method allows improvements of the BD-rate performances.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.19% | -0.09% | 102% | 100% |
| Class A2 | -0.14% | -0.11% | -0.09% | 102% | 101% |
| Class B | -0.09% | -0.05% | -0.10% | 102% | 99% |
| Class C | -0.02% | -0.01% | -0.11% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.08% | -0.10% | 102% | 100% |
| Class D | -0.03% | -0.09% | -0.02% | 103% | 102% |
| Class F | -0.02% | -0.04% | -0.04% | 102% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.30% | -0.07% | 111% | 106% |
| Class C | -0.02% | 0.12% | -0.07% | 111% | 117% |
| Class E | -0.09% | 0.29% | 0.16% | 111% | 117% |
| Overall | -0.05% | -0.01% | -0.01% | 111% | 112% |
| Class D | -0.05% | -0.61% | 0.16% | 107% | 107% |
| Class F | 0.11% | 0.05% | -0.28% | 110% | 113% |

## Results against VTM-3.0 with 4 SATD candidates

We also show the results compared to a VTM-3.0 where the number of SATD candidates is set to 4 as in merge for the anchor.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 + 4SATD candidates |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | -0.02% | 101% | 99% |
| Class A2 | -0.05% | -0.01% | -0.03% | 101% | 101% |
| Class B | -0.01% | -0.04% | -0.02% | 100% | 98% |
| Class C | -0.01% | 0.03% | -0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.03% | 100% | 100% |
| Class D | 0.01% | -0.09% | -0.03% | 101% | 103% |
| Class F | 0.02% | -0.02% | 0.03% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0+4SATD candidate |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.11% | -0.18% | 100% | 103% |
| Class C | 0.05% | 0.19% | -0.11% | 100% | 100% |
| Class E | -0.04% | 0.73% | 0.45% | 99% | 101% |
| Overall | 0.01% | 0.20% | 0.00% | 100% | 101% |
| Class D | 0.05% | -0.61% | -0.05% | 100% | 100% |
| Class F | 0.14% | -0.41% | 0.11% | 101% | 102% |

We want to kindly thank Qualcomm for their cross-check.

# Additional simulation results using 1 SATD candidate

Results with one SATD candidates for both anchor and proposal.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.04% | 0.04% | 99% | 100% |
| Class A2 | -0.03% | 0.00% | 0.05% | 99% | 99% |
| Class B | -0.01% | 0.05% | -0.03% | 99% | 99% |
| Class C | -0.01% | 0.01% | 0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.02% | 99% | 99% |
| Class D | -0.02% | -0.03% | -0.03% | 101% | 100% |
| Class F | 0.02% | -0.02% | 0.01% | 101% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.29% | -0.25% | 107% | 107% |
| Class C | 0.01% | 0.18% | 0.29% | 106% | 116% |
| Class E | 0.09% | -0.04% | -0.02% | 105% | 116% |
| Overall | 0.04% | -0.07% | -0.01% | 106% | 112% |
| Class D | 0.03% | 0.53% | 0.42% | 104% | 108% |
| Class F | -0.01% | -0.34% | -0.19% | 106% | 111% |

# Conclusion

This contribution proposes to use purely temporal constructed affine models instead of mixed spatio-temporal ones and reduces the complexity. Results show a slight gain for CTC configurations while complexity is reduced both in terms of comparison and CPMV construction.
It is suggested that the new candidates proposed in this contribution to be adopted into the reference test model.

# References

[1] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, 12th Meeting, Macao, CN, October 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).