| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0772 |
| --- | --- |

| Title: | CE5-related: Clean up of the context model initialization process for CE5.1.5 and CE5.1.6 |  |  |
| --- | --- | --- | --- |
| Status: | Input documentto JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Stegemann, H. Kirchhoffer, H. Schwarz, D. Marpe T. Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-183 jan.stegemann@hhi-extern.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This documents presents a clean up of the context model initialization process of the state-based probability estimator from tests CE5.1.5 and CE5.1.6 of CE5. Preliminary average coding gains of the proposed method are -0.01%(AI), x.x%(RA), -0.10%(LB) and -0.11%(LP) relative to CE5.1.5 and -0.01%(AI), -0.03%(RA), -0.14%(LB) and 0.01%(LP) relative to CE5.1.6.

# Introduction

All probability estimators tested in CE5 use a mapping table during the initialization process of the context models to translate from a HEVC probability state to the required format. This mapping table consists of 128x16bit values. The proposed clean up suggests initializing the context models directly without the usage of a mapping table.

# Proposed approach

The presented approach is similar to the HEVC initialization process originally described as “Formula 2” in [1]. For each slice type one 8-bit initialization value initVal per context model is used to derive the initial state pState by linear interpolation using the quantization parameter QP:

slope=initVal≫4*5-45,

offset=initVal&15 *16-32,

pState=Clip1,255,slope*QP≫3+offset.

Using pState the two state variables S0 and S1 from [2] are set according to their precision bitsS0 and bitsS1 as follows:

S0=pState-128≪bitsS0-8,

S1=pState-128≪bitsS1-8.

# Results

The proposed clean up was implemented on top of VTM 3.0. Tests were conducted under VTM common test condition [3]. The results show the impact of the proposed clean up relative to the results of the CE tests CE5.1.5 and CE5.1.6 in [4]. Because the initialization process is modified, the initialization values are retrained. Note, the processing times are inaccurate because of the use of mixed architectures. The results show the performance of the preliminary simulations with 49 frames.

Table 3-1 - CE5.1.5 init clean up, 8-bit and 12-bit states

Table 3-2 - CE5.1.6 init clean up, variable window size, 10-bit and 14-bit states

# Reference

- L.Guo et al., CE1 B3: 8-bit Linear Initialization for CABAC, JCTVC-H0535, San Jose, USA, Feb. 2012

- H.Kirchhoffer et al., CE5: State-based probability estimation (CE5.1.4, CE5.1.5) and coding interval subdivision (CE5.3.1), JVET-L0462, Macao, China, Oct. 2018

- JVET, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, China, Oct. 2018

- H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine,” JVET-L1025, Macao, China, Oct. 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).