| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0798 |
| --- | --- |

| Title: | Crosscheck of supplementary results of JVET-M0477 (CE2: Simplification of Affine constructed merge candidates (Test 2.2.6) and supplementary results) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hongbin Liu Yingdu Mansion F11, 48 Zhichun Road, Building B, Haidian District, 100086 Beijing, China | Tel: Email: | liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results of the supplementary results, i.e., test c and d, of JVET-M0477 [1] which proposes simplification of affine constructed merge candidates. The results match with that provided in JVET-M0477.

The minor differences in some cases are believed to be caused by the known bug in VTM-3.0 which may lead to minor difference in different platforms. Note that the proponents run the simulation in Windows platform, and the crosschecker run the simulation in Linux platform.

# Experimental results

JVET-M0477 were implemented on top of reference software VTM-3.0 [2], and simulations were performed following the common test conditions [3]. Two tests are conducted:

- Test#1: test c in JVET-M0477.

- Test#2: test d in JVET-M0477.

The results are shown in Table 1 and Table 2.

Table 1: Coding performance of Test#1.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.01% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.01% | 0.06% | 0.09% | 100% | 100% |  |  |  |  |  |
| Class B | 0.02% | 0.13% | -0.07% | 99% | 100% | 0.06% | -0.27% | -0.15% | 101% | 100% |
| Class C | 0.02% | 0.08% | 0.02% | 99% | 98% | 0.04% | 0.04% | 0.20% | 98% | 102% |
| Class E |  |  |  |  |  | -0.06% | -0.06% | -0.10% | 99% | 101% |
| Overall | 0.01% | 0.07% | 0.00% | 100% | 99% | 0.02% | -0.12% | -0.02% | 100% | 101% |
| Class D | 0.04% | -0.17% | -0.06% | 101% | 102% | 0.08% | 0.05% | 0.30% | 97% | 98% |
| Class F | -0.02% | -0.04% | -0.05% | 99% | 99% | 0.05% | 0.26% | -0.39% | 99% | 99% |

Table 2: Coding performance of Test#2.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.07% | 0.00% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.02% | 0.12% | 0.09% | 100% | 100% |  |  |  |  |  |
| Class B | 0.02% | 0.03% | -0.04% | 99% | 100% | 0.06% | -0.06% | 0.15% | 100% | 98% |
| Class C | 0.06% | 0.08% | 0.04% | 98% | 97% | 0.05% | 0.31% | 0.09% | 99% | 102% |
| Class E |  |  |  |  |  | 0.08% | 0.24% | -0.23% | 100% | 101% |
| Overall | 0.04% | 0.07% | 0.01% | 99% | 99% | 0.06% | 0.13% | 0.03% | 100% | 100% |
| Class D | 0.04% | -0.01% | 0.10% | 101% | 101% | 0.14% | 0.09% | -0.02% | 97% | 98% |
| Class F | 0.00% | 0.03% | 0.04% | 99% | 98% | 0.05% | 0.38% | 0.38% | 99% | 100% |

# References

- “CE2: Simplification of Affine constructed merge candidates (Test 2.2.6) and supplementary results”, Y.-C. Yang, Y.-J. Chang, Doc. JVET-M0477, Jan 2019.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.