| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0814-v3 |
| --- | --- |

| Title: | Non-CE3: block size restriction on PDPC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Jin Heo Jangwon Choi Jungah Choi Sunmi Yoo Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 {l.li, jin78.heo, jangwon84.choi,sunmi.yoo, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

PDPC as a post filter is applied to all block size both luma and chroma on Planar, DC, Horizontal, Vertical and some neighboring intra modes of diagonal directions. For the worst case, chroma 2x2 intra prediction, PDPC would not be desirable in terms of throughput. Therefore, block size restriction is proposed on the luma and chroma blocks which have less or equal to 16 pixels. Furthermore, PDPC is restricted for large blocks, so that only applies on luma blocks which size is up to 32x32 and chroma blocks which size is up to 16x16. The experiment result reportedly shows 0.05% and x.xx% bit rate changes for AI and RA of Test 1, 0.06% and x.xx% bit rate changes and 2% decoding time decrease for AI and RA of Test 2.

Proposed Method

Following block size restriction is applied.

- Test 1: PDPC is not applied to luma and chroma blocks which have less or equal to 16 pixels (W*H<=16)

- Test 2: On top of test 1, the maximum luma and chroma block size for PDPC is restricted as 32x32 and 16x16, respectively. (max(W, H)<=32 for luma, max(W,H)<=16 for chroma)

Experimental Results

The proposed method is implemented on top of VTM 3.0, and evaluated for VTM configuration according to the common test conditions [2].

Table 1 Experiment result for test 1 on all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | 0.05% | 100% | 102% |
| Class A2 | 0.04% | 0.06% | 0.00% | 98% | 99% |
| Class B | 0.05% | 0.04% | 0.00% | 100% | 100% |
| Class C | 0.06% | -0.10% | -0.01% | 100% | 100% |
| Class E | 0.12% | 0.10% | 0.11% | 100% | 101% |
| Overall | 0.05% | 0.01% | 0.03% | 100% | 100% |
| Class D | 0.10% | -0.06% | -0.04% | 100% | 99% |
| Class F | -0.14% | -0.19% | -0.09% | 99% | 99% |

Table 2 Experiment result for test 1 on all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.10% | -0.16% | 100% | 98% |
| Class A2 | 0.07% | -0.11% | -0.10% | 100% | 99% |
| Class B | 0.06% | -0.06% | -0.13% | 101% | 98% |
| Class C | 0.06% | -0.05% | -0.05% | 101% | 99% |
| Class E | 0.13% | -0.12% | 0.09% | 101% | 97% |
| Overall | 0.06% | -0.08% | -0.08% | 101% | 98% |
| Class D | 0.10% | -0.09% | -0.01% | 103% | 99% |
| Class F | -0.14% | -0.24% | -0.24% | 101% | 98% |

Summary

The proposed block size restriction on PDPC shows 0.05% and 0.06% bit rate changes for AI of Test 1 and Test 2. It is recommended to adopt this proposal to VVC.

References

- VTM 3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).