| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0817 |
| --- | --- |

| Title: | Cross-check of JVET-M0528 (Non-CE3: A unified luma intra mode list construction process) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Liang Zhao and Xin Zhao | Tel: Email: | leolzhao@tencent.com xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports a cross-checking of contribution JVET-M0528 related to unification of MPM list construction process.

The BD-rate reported by the proponents and the cross-checker match. The cross-checker received the source code and performed the cross-check during the meeting.

- Introduction

In JVET-M0528, the contribution proposes the use of a single list construction process comprising solely of angular modes. This process is the same as the one currently being used for non-zero reference line index. Since the list no longer contains non-angular modes (PLANAR, DC modes) the luma intra mode signaling is modified to first signal a flag indicating if the selected mode is angular, namely isAngular flag. For non-angular mode a further flag is received indicating if the mode is PLANAR or DC, namely isPlanarFlag. Six contexts are used in total to signal the isAngular Flag and isPlanar flag.

Test Result

The cross-check test is performed on top of VTM-3.0 with VTM-3.0 configurations. The results are shown in the following tables, and they match the results shown in JVET-M0528. Tables 1 and Table 2 show the results of the test in AI and RA configuration.

Table 1 Result of JVET-M0296 for VTM AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-3.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | -0.48% | -0.35% | 102% | 99% |
| Class A2 | -0.20% | -0.12% | -0.17% | 104% | 100% |
| Class B | -0.25% | -0.20% | -0.15% | 105% | 97% |
| Class C | -0.16% | -0.24% | -0.15% | 108% | 100% |
| Class E | -0.21% | -0.14% | -0.02% | 107% | 100% |
| Overall | -0.25% | -0.23% | -0.16% | 105% | 99% |
| Class D | -0.18% | 0.01% | -0.12% | 111% | 104% |
| Class F | 0.01% | 0.03% | -0.03% | 105% | 96% |

Table 2 Result of JVET-M0296 for VTM RA configuration

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.13% | 0.00% | 99% | 98% |
| Class A2 | -0.05% | 0.17% | 0.18% | 99% | 98% |
| Class B | -0.11% | 0.22% | 0.02% | 101% | 97% |
| Class C | -0.04% | -0.09% | 0.17% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.06% | 0.09% | 100% | 99% |
| Class D | -0.06% | -0.41% | -0.15% | 102% | 100% |
| Class F | -0.01% | -0.03% | 0.02% | 102% | 101% |

Reference

[1]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

[2]	F. Bossen et al., “Non-CE3: A unified luma intra mode list construction process”, Document of Joint Video Experts Team (JVET), JVET-M0528, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019