| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0830 |
| --- | --- |

| Title: | Crosscheck of JVET-M0736 (CE10-related: Triangular prediction with MMVD) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao | Tel: Email: | han.gao@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This document reports the cross-check results for the results reported in input document JVET-M0736 CE10-related: Triangular prediction with MMVD. This crosschecked contribution proposes the combination of triangular prediction with merge with motion vector difference (MMVD) techniques.

# Cross-checked results

The crosschecked method is implemented on top of VTM-3.0 [8]. For the combined approach, three syntax changes are necessary compared to the current VVC draft:

- If the triangle merge mode is enabled and if the MMVD mode is enabled, the decoding of an additional triangle_direction_flag syntax element is necessary, to specify the direction of the diagonal split (top-left to bottom-right or bottom-left to top-right). This syntax element is necessary as the direction of the diagonal split is otherwise inferred from the triangle_merge_index and a look-up table.

- If the triangle merge mode is enabled and if the MMVD mode is enabled, the syntax decoding processes associated with MMVD are invoked a second time, to derive the motion information of the second triangle.

- If the triangle merge mode is enabled and if the MMVD mode is disabled, the triangle_merge_index is decoded as currently specified.

The simulation results of the crosschecked proposal is shown as follow, which matched the results proposed by JVET-MM0763.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.17% | -0.22% | 151% | 100% |
| Class A2 | -0.19% | -0.26% | -0.39% | 165% | 100% |
| Class B | -0.14% | -0.19% | -0.07% | 168% | 100% |
| Class C | -0.19% | -0.30% | -0.31% | 185% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.23% | -0.23% | 168% | 100% |
| Class D | -0.30% | -0.41% | -0.52% | 191% | 102% |
| Class F | -0.08% | -0.13% | -0.17% | 170% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.44% | -0.89% | -0.57% | 203% | 100% |
| Class E | -0.30% | -0.32% | -0.69% | 180% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.65% | -0.26% | -0.58% | 210% | 99% |
| Class F | -0.37% | -0.61% | 0.71% | 186% | 100% |

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. does not have any current or pending patent rights relating to the technology described in this contribution.