| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0843-v3 |
| --- | --- |

| Title: | BoG report on CE4 related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kai Zhang | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com |
| Source: | BoG |  |  |

_____________________________

# Summary

XX sessions were held, 1540 ~ 2020 on Jan. 11, 0900 ~ 1045 on Jan. 12, 1830 ~ 2000 on Jan. 12, 1945~2300 on Jan. 13 for discussing 47 technical contributions in five categories:

1) Merge Modifications (18)

2) MMVD Modifications (13)

3) AMVP Modifications (7)

4) Weighted-Prediction Modifications (2)

5) Complexity Reduction (8)

## Recommendations

### Recommended adoptions to VTM (10)

Normative changes

Complexity Reduction

JVET-M0193 CE4-related: Pairwise Average Candidate Reduction

JVET-M0300 CE4-related: HMVP and parallel processing with tiles and tile groups

JVET-M0117 CE4-related: On MVP candidate list generation for AMVP

Bug Fix/Cleanup/Harmonization:

JVET-M0436 AHG2: Regarding HMVP Table Size

JVET-M0264 Non-CE4: Harmonization between HMVP and GBi

JVET-M0068 Non-CE4: MMVD scaling fix

JVET-M0171 CE4-related: MMVD cleanups

JVET-M0111 AHG13: On bi-prediction with weighted averaging and weighted prediction (the same syntax design to support WP is also proposed in JVET-M0067)

JVET-M0479 Non-CE4: On clipping of scaled motion vectors

Coding Efficiency:

JVET-M0255 AHG11: MMVD without Fractional Distances for SCC

JVET-M0444 CE4-related: Simplified symmetric MVD based on CE4.4.3

### Recommended CE4 tests from following contributions (16)

Merge-related

Syntax modification

JVET-M0069 Non-CE4: Syntax change of MMVD

JVET-M0359 Non-CE4: Modification of merge data syntax

JVET-M0369 CE4-related: Syntax changes of merge data

Merge list simplification

JVET-M0405 CE4-related: Simplified merge candidate list for small blocks

STMVP

JVET-M0518 CE4-related: Supplemental results on STMVP design of CE4.2.3.a and combination with methods of JVET-M0127 (CE4.1.2.a) and JVET-M0127

JVET-M0713 CE4-related: simplification of CE4.2.2

MMVD Signaling

JVET-M0206 CE4-related: MMVD improvements

JVET-M0267 Non-CE4: Harmonization of MMVD and AMVR

JVET-M0307 CE4-related: Candidates optimization on MMVD

JVET-M0308 Non-CE4: MMVD simplification

JVET-M0314 CE4-related: MMVD improving with signaling distance table

JVET-M0315 Non-CE4: MMVD scaling simplification

JVET-M0435 CE4-related: MMVD offset table signaling

TMVP Storage Reduction

JVET-M0230 CE4-related: Temporal MV buffer reduction

JVET-M0346 CE4-related: Non-square compression grid for temporal motion data storage

JVET-M0512 Non-CE4: On Temporal Motion Buffer Compression

## Open issues

### Revisit in track (10)

Depending on CE decision

Pending on JVET-M0170

JVET-M0272 CE4-related: Restrictions on History-based Motion Vector Prediction

JVET-M0345 CE4-related: Remove redundancy between TMVP and ATMVP

JVET-M0473 Simplified HMVP

JVET-M0350 CE4-related: Quadtree-based Merge Estimation Region for VVC

Related to JVET-M0281

JVET-M0081 Non-CE4: Simplification of AMVP list generation in AMVR

Related to JVET-M0403

JVET-M0422 CE4-related: Simplified MVD coding

No Consensus

JVET-M0406 CE2/4-related: Unified merge list size for block and sub-block merge modes

JVET-M0661 AHG-13: On Merge List Size

JVET-M0330 CE4-related: Simplification of MMVD scheme

# Merge Modifications (18)

JVET-M0069 Non-CE4: Syntax change of MMVD [E. Sasaki, T. Chujoh, T. Ikai (Sharp)]

In the current VTM-3.0, merge with MVD (MMVD)[1] shows gains in common test conditions (CTC)[2]. However the authors found that MMVD causes luma BD-rate loss of 0.21% / 0.95% in RA and LB conditions in high QP (42-57) range. The authors concluded this issue was caused by the current syntax structure of MMVD, which needs relatively large number of bins (5-bins) for the skip mode and merge mode.

This contribution proposes alternative syntax structure where the syntax elements of MMVD are signaled when skip_flag is equal to 0 and merge_flag is equal to 0, before the conventional AMVP motion vector difference. Experimental results shows -0.36%/-0.74% luma BD-rate change in RA and LB under high QP condition and -0.11%/-0.23% luma BD-rate change in RA and LB under CTC.

The syntax elements of MMVD are signaled when skip_flag is equal to 0 and merge_flag is equal to 0. Make MMVD a separate mode, not a special merge mode.

RA: -0.11%, LB: -0.23%. Higher gain is reported on the high QP configuration.

Discussion:

Q: Why there is a loss on chroma?

A: Maybe there is no MMVD skip.

Q: Where is the gain coming from?

A: Because MMVD is more similar to AMVP instead of merge.

Q: Are there results on low QP?

A: No.

It is asked the results for removing MMVD skip only.

It is commented that the gain may be overlapped with the bug fix.

Recommendation: Further study in CE

JVET-M0127 CE4-related: Modification on Merge List [Y. Han, W.-J. Chien, D. Rusanovskyy, Y.-H. Chao, C.-C. Chen, H. Wang, H. Huang, C.-H. Hung, Y. Zhang, M. Karczewicz (Qualcomm)]

This contribution presents a change on merge list which reduces the number of pairwise candidates from 6 to 1 and remove the dependency on HMVP candidates. For the “VTM3.0 as reference” tests in RA case with the size of merge list equal to 6, the presented simplification method achieved -0.02% coding gain.

Related to JVET-M0518 from Qualcomm.

Reduce pairwise candidates from 6 to 1, and move it before HMVP.

Discussion:

Q: How many HMVP candidates are checked? And how many pruning?

A: The pruning number is increased from 10 to 15.

Recommendation: No Action.

JVET-M0193 CE4-related: Pairwise Average Candidate Reduction [A. Tamse, M. W. Park, K. Choi (Samsung)]

In this contribution, it is proposed to reduce the number of pairwise average candidates added in the merge list. There are up to six pairwise average candidates defined during the generation of merge candidate list. However as shown in this proposal, it is never possible for some of the candidates to be used in the list generation. Also, other candidates that are used in the list generation, don’t improve the performance but add additional complexity for generation of the list. This contribution proposes several options to reduce the number of pairwise average candidates. The performance change by just using only one pairwise average candidate when tested on top of VTM3.0 under RA conditions is 0.01% for Y, 0.03% for U and 0.02% for V with no change in runtime.

If two pair-wise candidates are allowed, -0.01% for Y.

Discussion: Several experts recommend to adopt method 1 (only one pairwise candidate).

Recommendation: Adopt method 1.

JVET-M0220 Non-CE4: Subjective quality analysis of non-sub-block ATMVP [Y.-H Chao, W.-J Chien, M. Karczewicz (Qualcomm)]

This contribution investigates the subjective quality of non-sub-block ATMVP and the sub-block based ATMVP (fixed 8x8 blocks) adopted in the JVET meeting in Macao (October, 2018).

Just Information.

JVET-M0231 CE4-related: Regular merge flag coding [X. Wang, Y.-W. Chen (Kwai Inc.)]

In this contribution, modification on the signaling of the regular merge flag under both merge and skip modes is proposed. It is observed that regular merge/skip is the most frequently used mode but is signaled with the longest codeword in the current VVC. In this proposal, we propose to modify the flag signaling for merge and skip modes. In the proposed scheme, the codeword length for regular merge mode becomes the shortest one and the results reportedly show 0.00% and -0.20% BD-rate improvements under RA and LDB, respectively.

It is claimed that the codeword representing regular merge are too long. So it is proposed to change the binarization method and put regular merge first in the codeword. The proponent claimed that major benefits is higher parsing efficiency.

No coding gain in RA test.

Discussion:

Add one new context.

Remove the triangular mode flag since the binarization is changed. So three contexts are removed.

Recommendation: Revisit

Revisited on Jan. 12

Recommnedation: Study in CE

JVET-M0264 Non-CE4: Harmonization between HMVP and GBi [J. Li, S. Wang, W. Gao (Peking University), L. Zhang, K. Zhang, H. Liu, J. Xu (Bytedance), X. Xiu, D. Luo, Y. He (InterDigital)] (Chaired by X. Li)

In current VVC, History-based Motion Vector Prediction (HMVP) and Generalized Bi-Prediction (GBi) are both employed. With GBi, each inter-coded block is further assigned one bi-prediction weight index in addition to the motion information. The bi-prediction weight index is generally carried through the merge motion vector prediction process, but it is not carried through the HMVP table. To align the design, it is proposed to store the coded bi-prediction weight index in addition to the motion information for all HMVP candidates. Simulation results reportedly show that the harmonization achieves 0.01% and 0.01% BD rate reduction for RA Main10 and LDB Main 10 configurations, respectively.

GBI index is not included in HMVP table in VTM-3.0. Proposed to include GBI index in HMVP table. 0.01% coding gain is achieved for both RA and LDB. 0.05% and 0.06% gain for LDB class D and class F.

Discussion:

The table size is increased by 2*5 = 10 bits.

The pruning process is unchanged.

Two experts express support.

Recommendation: Adopt

JVET-M0272 CE4-related: Restrictions on History-based Motion Vector Prediction [L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)] (Chaired by Y. He)

Two restrictions are proposed for the History-based Motion Vector Prediction (HMVP) design. Firstly, for 4×4 blocks, HMVP candidates are excluded from the regular merge list and the associated motion information is disallowed to update the HMVP table. Secondly, whether motion information of one block could be added to the HMVP table further depends on the category of decoded merge candidate. If the merge candidate is a virtual merge candidate (i.e., candidates inserted after the HMVP candidates), such motion information could be utilized to update the HMVP table. With proposed restrictions, it is asserted that the worst case complexity of regular merge list construction process could be reduced and the bottleneck is changed from 4 × 4 blocks to larger blocks in terms of MV throughput. Simulation results reportedly show that the restricted HMVP achieves 0.00% and 0.02% BD rate reduction for RA Main10 and LDB Main 10 configurations, respectively.

Aspect 1: disable HMVP usage for merge list construction and HMVP table updating for 4x4 blocks

Aspect 2: disable using virtual merge candidate for HMVP update

Aspect 3: HMVP candidate is only pruned to the first two spatial/temporal merge candidates

Comment: The proposal for the shared merge list (JVET-M0170) will be revisited, but it is related to aspect 1 of this proposal.

Recommendation: Revisit in Track B after JVET-M0170 is revisited

JVET-M0300 CE4-related: HMVP and parallel processing with tiles and tile groups [A. M. Kotra, J. Chen, B. Wang, S. Esenlik, H. Gao (Huawei)]

In VTM-3.0, History-based motion vector prediction (HMVP) FIFO table is reset for each CTU Row to enable parallel processing for tools such as Wavefronts (WPP). From VTM-3.1 onwards the concept of slices is replaced with tile groups. Therefore this contribution extends the technique of re-setting the HMVP table for Tiles and Tile groups. Specifically, the HMVP table is re-set at the beginning of a new Tile group or a new Tile and also at the beginning of each CTU row inside a Tile.

Refresh HMVP table at the beginning of each tile/tile group.

Discussion:

It makes sense. The SW for tile groups is still not available.

Recommendation: Adopt

JVET-M0345 CE4-related: Remove redundancy between TMVP and ATMVP [S. H. Wang (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma (Peking University)]

At the 11th JVET meeting, the granularity of temporal MV compression is set to 8x8 and at the 12th JVET meeting, sub-block size of ATMVP have been fixed to 8x8. In specific cases, there are some redundancy operations between TMVP and ATMVP. Besides, TMVP with smaller block size becomes unnecessary in some cases due to the storage granularity. In order to remove the redundant process of deriving TMVP in merge mode, this contribution reports the experimental results on two cases as follows:

- Test(a): Skip TMVP process in merge mode when CU size is equal to 8x8

- Test(b): Skip TMVP process in merge mode when the width or height of CU is 4 + Test(a)

Simulation results for Test (a), Test (b) show minor coding gain compared to VTM-3.0 and observe minor encoding and decoding time decrease.

Test a: RA: -0.02%, LDB: 0.00%.

Test b: RA: -0.04%, LDB: 0.00%. (recommended by the proponent)

Discussion:

Remove some redundancy between TMVP and ATMVP (especially for 8*8 blocks).

For small blocks, TMVP is not needed.

Class D has coding loss in Test b. RA 0.11%, LDB: 0.19%.

One expert claimed that it’s related to shared merge list (JVET-M0170) and suggested to be tested on that. The results may likely be as Test a.

Recommendation: Revisit in Track B after JVET-M0170 is revisited

JVET-M0359 Non-CE4: Modification of merge data syntax [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

This contribution presents a modification of merge data syntax. The current VTM-3.0 has many merge schemes such as subblock merge modes, merge mode with motion vector difference (MMVD), the combined inter-picture merge and intra-picture prediction, and triangular shape-based motion compensation. When merge_flag is equal to 1, mmvd_flag is always parsed since it is the first syntax in merge data syntax. However, the current MMVD uses only the first or second merge candidate as a base candidate. Therefore, in the proposed syntax structure, mmvd_flag is parsed only when the merge candidate index is equal to 0 or 1. It reportedly shows -0.06%/-0.14%/-0.15% (Y/U/V) average BD-rate change with RA configuration and -0.17%/-0.22%/-0.30% (Y/U/V) average BD-rate change with LDB configuration. Also the proposed method could remove the mmvd_merge_flag and its context model.

Move MMVD syntax elements after merge_idx. MMVD flag is signaled only when merge_idx < 2. mmvd_merge_flag is removed.

Change the CABAC context initial value.

RA: -0.06%, LDB: -0.17%.

Discussion:

There is a bug on MMVD signaling in VTM-3.0 in the proposal. If the bug is fixed, the gain may be decreased.

There are related proposals such as JVET-M0369.

Recommendation: Study in a CE.

JVET-M0369 CE4-related: Syntax changes of merge data [Y. Ahn, D. Sim (Digital Insights)] [late]

This contribution proposes a clean-up of merge data syntaxes in order to maintain consistency according to prediction partitions. In the previous meeting, three kinds of merge list were adopted such as CU merge list, subblock merge list, and triangle merge list. More specifically, there are MMVD list and general CU merge list for CU-level. In the current draft, the order of syntax parsing for merge data is mmvd, subblock merge, multi hypothesis prediction, triangle prediction, and general CU-level merge. This contribution changes the order of merge data syntaxes in order to maintain consistency according to the type of prediction partitions. The proposed changes can achieve 0.17% and 0.14% BD-rate gain in random access configuration and low-delay B configuration, respectively.

It is proposed to re-arrange order of the syntax elements for varieties of merge modes.

RA: -0.07%, LDB: -0.14%.

Most gain comes from the triangular flag signaling bug fix.

It is claimed that the major benefit is to make the syntax design more neat.

Discussion:

Q: Is CABAC initial value changed?

A: No.

Recommendation: Study in CE.

JVET-M0405 CE4-related: Simplified merge candidate list for small blocks [X. Xu, X. Li, S. Liu (Tencent)]

It was asserted during Macao meeting that there would be no enough cycles to construct merge list for small blocks when resolution is 4K or above. This contribution proposes to simplify the merge candidate list construction process for small blocks (width*height <= threshold). The simplification is implemented by allowing only 1 spatial or temporal candidate before the history based candidates. In addition, the pruning operations are removed. It is reported that when a threshold for small blocks is set to 32 luma samples, the BD rate changes comparing to VTM-3.0 is 0.09% (0.00% and 0.02% for A1 and A2) and 0.14% in RA and LB condition, respectively; when a threshold for small blocks is set to 64 samples, the BD rate change comparing to VTM-3.0 is 0.21% (0.04% and 0.14% for A1 and A2) and 0.33% in RA and LB condition, respectively.

When width*height <= threshold, only 1 spatial or temporal candidate before the history based candidates is allowed. In addition, the pruning operations are removed.

RA 0.09% LB 0.14% when T = 32. (Recommended)

Discussion:

It is commented that it is aligned with ATMVP design.

All prunings are removed for small blocks.

Related to JVET-M0170.

Recommendation: Study in CE.

JVET-M0406 CE2/4-related: Unified merge list size for block and sub-block merge modes [X. Xu, X. Li, S. Liu (Tencent)]

This contribution proposes to signal one merge size for both block based and sub-block based merge modes, instead of separate signaling flags for individual lists with different maximum capacity. It is reported that when a unified merge size is set to 6, the BD rate changes comparing to VTM-3.0 is -0.02 % and 0.00 % in RA and LB condition, respectively; when a unified merge size is set to 5, the BD rate change comparing to VTM-3.0 is 0.06 % and 0.10 % in RA and LB condition, respectively.

Unified merge list size signaling.

Discussion:

Recommendation: Revisit with JVET-M0661 in track

JVET-M0433 CE4-related: Constraint on GBi index inheritance in Merge Mode [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

In this contribution, it is proposed to limit the GBi index inheritance to only allow the inheritance from regular spatial merge candidates. The proposed method was implemented on VTM-3.0. It was reported that by applying the proposed constraint, the overall BD-rate impact is 0.03% for both Random Access and Low Delay B configurations.

Remove GBI inheritance for affine merge.

RA/LB: 0.03%/0.03%.

Proponents claimed that this can reduce average complexity.

It should be CE2-related.

Discussion:

No impact on the worst case complexity.

Proponents reported that there is 0.1% RA loss when totally disabling GBI on affine.

Recommendation: Further study in CE2.

JVET-M0436 AHG2: Regarding HMVP Table Size [J. Zhao, S. Kim (LGE)]

In VTM3, History-based Motion Vector Prediction (HMVP) allows to store up to 6 HMVP candidates for HMVP table. When updating the HMVP table, redundancy check is applied to prune HMVP candidates in the table. When HMVP candidates are added to merge list, maximum 5 HMVP candidates are allowed. Hence, the oldest entry of HMVP table is never used in the list construction. Therefore, it is proposed to change HMVP table size from 6 to 5. Experiments confirmed that with this change, there is no RD performance change. Results are identical.

Recommendation: Adopt

JVET-M0518 CE4-related: Supplemental results on STMVP design of CE4.2.3.a and combination with methods of JVET-M0127 (CE4.1.2.a) and JVET-M0127 [D. Rusanovskyy, Y.-H. Chao, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution presents a supplemental result on proposed STMVP design of JVET-M0221 (CE4.2.3.a) and its combination with methods of JVET-M0126 (CE4.1.2.a) and JVET-M0127 (CE4-related).

Reported results show that in Test 1, proposed STMVP being integrated in the VTM3.0 provides 0.1% of BD-rate gain in luma on average for RA against the VTM3.0 anchor. No changes in encoder/decoder run time against the VTM3.0 is reported.

In the Test 2, STMVP design being integrated with complexity reduction methods of M0126 and M0127 provide 0.07% of BD-rate gain in luma on average for RA against the VTM3.0 anchor. Reported decoding run time is reduced to 96% comparing to the VTM3.0 anchor.

Test 1: Only the STMVP feature of CE4.1.2.a. (RA -0.1%, LDB 0.06%).

Test 1*: Test 1 +Move STMVP from “after spatial candidates” to after “TMVP candidate” (RA: -0.07%, LDB: 0.02%)

Test 2: Test 1 + M0127. (RA: -0.07%, LDB:0.11%)

Test 2*: Test 2 +Move STMVP from “after spatial candidates” to after “TMVP candidate”. (RA: -0.05%, LDB: missing).

Discussion:

One expert suggests to be studied in CE.

Recommendation: Further study STMVP (Test 1 and Test 1* in JVET-M0518) in CE.

JVET-M0473 Simplified HMVP [W. Zhu, A. Segall (Sharp)]

A history-based motion vector prediction (HMVP) method was adopted into the VVC draft in the 12th JVET meeting. In the HMVP scheme, a HMVP table is maintained and updated on-the-fly with a pruning process. As part of this pruning process, the motion information candidate is compared with all candidates in the HMVP table to determine if there is a redundancy. It is asserted here that this pruning process increases complexity. In this proposal, two modifications to the HMVP scheme are proposed to reduce the complexity of the pruning process. First, it is proposed to update the HMVP table using a 16x16 grid, and so only inter block located at these gird point will be added to the HMVP list. This is asserted to reduce the complexity of the HMVP operation in the worst case.  Second, it is proposed to remove the reference index comparison during the HMVP pruning process. Simulation results show that the combination of these wo methods results in a 0.02% luma BD-Rate change compared to VTM 3 for both the random-access low-delay B configurations.

#1: HMVP table is updated at 16x16 grid. RA/LB 0.02%/0.02%

#2: Remove reference checking in merge list pruning on HMVP candidates. RA/LB 0.00%/0.00%

#3: Remove reference checking when updating HMVP table. RA/LB 0.01%/0.06%.

#1+#2: RA/LB 0.02%/0.02%

Discussion:

Q: Are there results with a larger grid?

A: Not finished

It is commented that a counter may be safer than grid.

Recommendation: Revisit in Track B after revisiting JVET-M0272

JVET-M0661 AhG-13: On Merge List Size [X. Li, X. Xu, S. Liu (Tencent)] [late]

In CTC, by default regular merge list size is set to 6 while sub-block merge list size is set to 5. It is reported that regular merge list size 6 brings minor coding benefit over size 5, i.e., 0.06%, 0.10% and 0.09% luma BD-rate reduction. It is proposed to use size 5 in CTC.

Unify the merge list size.

Discussion:

Q: What’s the coding gain if increase regular merge list size to 6?

A: About 0.02% (RA).

Save one merge STAD test at encoder.

Regular merge list size is 6 because of ATMVP, but ATMVP is moved to sub-block merge list now.

Some experts supported it, but no consensus.

Recommendation: Revisit in the Track

JVET-M0713 CE4-related: simplification of CE4.2.2 [F. Le Léannec, A. Robert, T. Poirier (Technicolor)] [late]

In CE4 tests 4.2.2.a and 4.2.2.b, reported in JVET-M0106, respectively one and two spatial temporal motion vector prediction (STMVP) candidates are proposed. The STMVP candidate basically consists in a averaged motion vector between 2 spatial and 1 temporal motion vector predictors, or between 2 spatial motion vector predictors.

In this contribution, a simplified computation of the average motion vector compared to JVET-M0106 is employed, when computing the average between 3 motion vectors. The same STMVP candidates are used as in JVET-M0106. The simplified test 4.2.2.a reportedly brings 0.11% BD-rate gain over VTM-3.0 (same as JVET-M0106). The simplified test 4.2.2.b reportedly brings 0.14% BD-rate gain over VTM-3.0.  LDB results are unchanged compared to JVET-M0106.

Recommendation: Further study in CE

# MMVD Modifications (13)

JVET-M0068 Non-CE4: MMVD scaling fix [E. Sasaki, T. Ikai (Sharp)]

This contribution proposes a fix on MMVD motion vector calculation. VTM3.0 MMVD flips / scales the motion vector difference (MVD) two times, i.e. with POC direction and with POC distance between L0 and L1 reference picture. This two time scaling creates the case where not mirrored motion vector is used (when the POC direction is mirrored and POC distance is different) while MMVD uses mirrored motion vectors in other most cases. This two time scaling causes confusions and it is reported this non mirrored motion usage is not efficient.

Experimental results shows -0.02% luma BD-rate change in RA and there is no change in LB.

Discussion:

Confirmed as a bug-fix by other experts

Recommendation: Adopt

JVET-M0171 CE4-related: MMVD cleanups [C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes three cleanups for merge mode with motion vector difference (MMVD).

In the first cleanup, MMVD can generate a bi-prediction candidate in VTM3.0 regardless of the slice type and is modified to allow only uni-prediction candidates when the current slice is a P-slice.

In the second cleanup, the MVD derivation process of bi-prediction MMVD is modified. There are two steps in the original MVD derivation process of bi-prediction MMVD. The first step is to decide the signs of MVDs according to whether the true bi-prediction (one reference picture is before the current picture in display order while the other reference picture is after the current picture in display order) is satisfied. Next, MVDs with the assigned signs are further scaled according to the temporal distances between the current picture and the two reference pictures. To simplify the two-step process into one-step, it is proposed to directly apply the existing motion vector (MV) scaling method for MVD scaling to derive the final MVDs of the two reference pictures according to the temporal distances between the current picture and the two reference pictures. Besides, the mismatch between JVET-L1001-v7.docx and VTM3.0 caused by the two-step MVD derivation process is also removed.

In the third cleanup, MMVD candidates are constrained to allow uni-prediction only when the current coding unit (CU) is 4x4 since 4x4 bi-prediction is intended to be prohibited in VTM3.0.

The combined test results including the three cleanups are as follows.
VTM3.0-RA: {Y-BD-rate = -0.03%, EncT=100%, DecT=101%}
VTM3.0-LB: {Y-BD-rate = 0.02%, EncT=102%, DecT=103%}
VTM3.0-LP: {Y-BD-rate = 0.00%, EncT=101%, DecT=100%}

Discussion:

Method 1 is a bug fix.

Method 2 is the same as JVET-M0068.

Method 3 is to align SW with the WD. The coordinator of SW (X. Li) commented that this has not been fixed in the SW.

Recommendation: Adopt Method 1 and Method 3 (bug fix). Method 2 the same as JVET-M0068

JVET-M0206 CE4-related: MMVD improvements [S. Jeong, M. W. Park, K. Choi (Samsung)]

This contribution proposes a method for improving the binarization of MMVD signalling. This includes three tests. The first test is the binarization method change of distance list index. There is no additional CTX model and other change. Experimental results shows -0.XX% luma BD-rate change in RA and 0.XX% luma BD-rate change in LDB. The second test is adding new distance group syntax with CTX model. Experimental results shows -0.10% luma BD-rate change in RA and 0.01% luma BD-rate change in LDB. The third test is context initial value change test of Test 1. Experimental results shows -0.XX% luma BD-rate change in RA and 0.XX% luma BD-rate change in LDB.

#1: Change the binarization method for MMVD distance index. (RA/LB: -0.08%/0.01%)

#2: #1+One new context model. (RA/LB: -0.10%/0.01%)

#3: #1+change context initial value. (RA/LB: -0.08%/0.02%)

Discussion:

Better gain on class A1/A2 (-0.18%/-0.11%).

Q: Can this also provide gains when there are two tables?

A: No results.

It is commented that the proposed method is less efficient on low resolution sequences with the current single table design.

Recommendation: Study in CE.

JVET-M0267 Non-CE4: Harmonization of MMVD and AMVR [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)] (Chaired by Y. He)

In this contribution, it is proposed to harmonize MMVD and AMVR. To signal the MVD in MMVD mode, The AMVR information is signaled first, followed by a MVD signaled with the specified MV resolution. Simulation results reportedly show 0.14% BD-rate savings under Random Access (RA) configurations.

Discussion:

Q: If AMVR is off, what’s the performance?

A: The syntax “mmvd_amvr_mode” signaled in this approach does not need to depend on the AMVR control in SPS. But this syntax element still shares the context of AMVR precision signaling.

Q: Why does not use separate contexts for “mmvd_amvr_mode”?

It is commented that it might be desirable to use separate context for “mmvd_amvr_mode”. The expert suggested to change the name of this syntax element.

Recommendation: Further study in CE

JVET-M0307 CE4-related: Candidates optimization on MMVD [N. Park, H. Jang, J. Nam, J. Lim, S. Kim (LGE)]

In this contribution, MMVD optimization scheme is proposed. Specifically, instead of 8 motion distance indices of MMVD in VTM3.0, 4 motion distance indices are used, and the actual motion distance from the given motion distance index is further adjusted considering 1) the magnitude of the base motion vector and block size or 2) mv resolution of the base motion vector. The experimental results for Test #1 reportedly show 0.08% (0.xx%) BD-rate savings with 97% (xx%) and 100% (xx%) encoding and decoding run-time compared to VTM3.0 as anchor in RA(LB) configurations, respectively. The experimental results for Test #2 reportedly show 0.05% (0.xx%) BD-rate savings with 97% (xx%) and 100% (xx%) encoding and decoding run-time compared to VTM3.0 as anchor in RA(LB) configurations, respectively.

The distance table size is reduced from 8 to 4.

#1: distance is further refined based on block dimensions (RA: -0.05%, LB: 0.01%)

#2: distance is further refined based on MV resolution of the base merge candidate. ( RA: -0.08%, LB: -0.02%)

Discussion:

More details should be provided on how to refine the distance.

No need to check the reference index, but check the prediction direction.

Recommendation: Study in CE

JVET-M0308 Non-CE4: MMVD simplification [X. Chen, J. Zheng (HiSilicon)]

This contribution presents simplification method for MMVDbased on VTM-3.0 that remove scaling process. It has no performance impact to VTM-3.0 anchor.

Remove MVD scaling in MMVD.

Discussion:

It is commented that the scaling should be kept for the LDB case.

Recommendation:

Study in CE.

JVET-M0314 CE4-related: MMVD improving with signaling distance table [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

This contribution proposes to signal adaptive distance table instead of deriving at decoder side to reduce complexity based on CE4.4.4e and CE4.4.5c. Experimental results show -0.2% (RA) and 0.02% (LDB) luma BD-rate change and 0.33% (RA) and 0.01% (LDB) luma BD-rate change respectively. There is no much change in encoding and decoding time change is observed compared with CE4.4.4e and CE4.4.5c.

Test 1: CE4.4.4.c but derive the distance table at encoder and signal the information in slice header. RA/LB -0.20%/0.02%

Test 2: CE4.4.5.e but derive the distance table at encoder and signal the information in slice header. RA/LB -0.33%, 0.01%.

Signaling in slice: One flag to signal whether change the table index order, 24 bits to signal the table indices.

The encoder strategy is as the same in CE test but only applied at encoder.

Whether the distance is left shift is based on whether it is a 4K sequence at decoder;

The index order is derived based on the best-gain algorithm.

Also include the encoder optimization (but not in anchor).

Discussion:

It is commented that decoding time may be decreased because more MMVD mode is chosen. But it is also commented that the decoding time may be not reliable.

Q: How to get the table index order?

A: Based on counting.

Q: How to get the weighting table?

A: Theoretically analysis to get the bits we can save.

Recommendation:

Further study in CE

JVET-M0315 Non-CE4: MMVD scaling simplification [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

This contribution proposes to simplify MV offset calculation process for MMVD. In this contribution, scaling process of generating MMVD MV offset is removed and flipping process is kept. In addition, a flag is added to SPS to indicate whether MMVD tool is used.

Experimental results show -0.01% (RA) and 0.08% (LDB) luma BD-rate change Ignorable change in encoding and decoding time is observed.

Same as JVET-M0308, but an additional SPS flag.

Discussion:

Recommendation: Further study in CE with JVET-M0308

JVET-M0330 CE4-related: Simplification of MMVD scheme [L. Xu, F. Chen, L. Wang (Hikvision)]

This contribution presents a simplified MMVD (Merge with MVD) scheme with one base candidate. MMVD is a motion vector expression method for skip and merge modes which has been adopted. Based on the MMVD scheme, in this contribution, three aspects of tests are conducted. (1) Reduce the base candidate from two to one, thus the base candidate index is not signaled. (2) Use a reordered merge candidate list to provide a better base candidate. (3) Combination of the CE4.4.4 which tries to optimize the MVD coding. Here, 5 tests are conducted:

Test a: 1 base candidate

RA: 0.25%	0.20%	0.22%	93%	98%

LB: 0.07%	0.01%	0.04%	98%	100%

Test b: 1 base candidate and reordered merge candidate list

RA: 0.07%	0.05%	0.05%	94%	98%

LB: -0.11%	-0.18%	-0.21%	98%	100%

Test c: 2 base candidate and reordered merge candidate list

RA: -0.06%	-0.05%	-0.04%	100%	101%

LB: -0.12%	-0.32%	-0.14%	101%	100%

Test d: 1 base candidate and CE4.4.4 Test c

RA: 0.04%	0.05%	0.03%	99%	100%

LB: 0.11%	0.18%	0.09%	101%	100%

Test e: 1 base candidate and CE4.4.4 Test c and reordered merge candidate

RA: -0.19%	-0.12%	-0.14%	99%	101%

LB: -0.12%	-0.09%	0.07%	102%	100%

Test a: Reduce base merge candidate from 2 to 1;

Test b: Test a + Test c;

Test c: Merge list reordering for MMVD merge and regular merge based on block height and width;

Does not change the merge list for triangular prediction.

It was in CE at previous meetings.

Discussion:

It is commented by the crosschecker that test a can be done as an encoder-only method.

Recommendation: Revisit in Track B

JVET-M0435 CE4-related: MMVD offset table signaling [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

An MMVD offset table signaling method is proposed based on the MMVD offset table reordering method in CE4.4.5, to resolve the inter picture dependency and error resilience issue. The proposed method was implemented on top of CE4.4.5 software with signaling in slice header. It is reported 0.05% （RA） and 0.14% （LDB）luma BD-rate increase over CE4.4.5.c. Compared to VTM-3, 0.19% luma BD-rate reduction and 0.01% BD-rate increase are reported for RA and LDB, respectively.

It is reported that there is an error resilience issue if we derive the table index order at decoder.

It is proposed to signal the index table at slice level.

The signaling causes loss RA/LB 0.05%/0.14%

Discussion:

What’s the difference between this contribution and JVET-M0314?

This contribution codes the difference between the default order and the new order.

Recommendation: Study in CE.

JVET-M0437 Non-CE4: Size constraint on MMVD [J. Zhao, S. Kim (LGE)]

In this document, it is proposed to disable MMVD for 4x4 blocks to reduce the MMVD complexity. Experimental result show that the coding impact is 0.01%, 0.03%, 0.01% for Y, U, V, components respectively in RA case, and 0.02%,0.08%,0.08% for Y, U,V, component respectively in LD case.

It is reported that MMVD benefit may not justify the parsing burden for small blocks. So it is proposed to disable MMVD for 4x4 blocks.

Discussion:

It is commented that MMVD complexity may not be a big deal at decoder. So the constrain is not necessary.

The proponents claimed that MMVD introduces more parsing burden.

One expert suggested that we may have an encoder-only constrain on that.

It is commented that more tools may be disabled on 4x4 blocks.

Recommendation: No action

JVET-M0627 Non-CE4: Supplementary results of combined solution of JVET-M0255, JVET-M0267 and JVET-M0069 [H. Liu, K. Zhang, L. Zhang (Bytedance), E. Sasaki, T. Chujoh, T. Ikai (Sharp)] [late] [miss] (Chaired by Y. He)

This contribution reports results of combination solution of JVET-M0255, JVET-M0267 and JVET-M0069. Three modifications on MMVD signalling are presented. First, fractional distances in merge with MVD (MMVD) are disabled for screen content videos. Second, distance index signalling in MMVD is harmonized with adaptive motion vector resolution (AMVR). Third, MMVD related syntax elements are signaled only for coding units that are coded in neither merge mode nor skip mode. Simulation results reportedly show 0.23% and 1.29% luma BD-rate saving for non-SCC sequences and SCC sequences respectively, in Random Access configuration.

Discussion:

Recommendation:

JVET-M0255:

Discussion:

It is commented that the idea is straightforward. It shows big coding gain for screen content (-1.56% for RA, -2.74% for LDB).

Cross-checker supported this approach.

The encoding time is reduced because MMVD mode (merge) will be selected more with this change.

Recommendation: adopt

# AMVP Modifications (5)

JVET-M0081 Non-CE4: Simplification of AMVP list generation in AMVR [Y. Kato, K. Abe, T. Toma (Panasonic)]

This contribution provides a simplification of AMVP list generation in AMVR. In AMVP list generation process of current AMVR, there are three intermediate rounding operations of AMVP candidates. In this proposal, intermediate rounding operations are all removed. This modification reduces the complexity of AMVR and enables encoder to share the AMVP list which is obtained for the first precision with other precisions. Simulation results reportedly show that the proposed method provides 0.01% BD-rate loss for RA, 0.03% BD-rate loss for LDB.

Remove all intermedia rounding for AMVR mode, only keep the final rounding.

Discussion:

It is an alternative way to unify the rounding issue for AMVR and non-AMVR MVP list construction. Related to CE proposal M0281: test 4.1.5.

Recommendation: Revisit at Track B

JVET-M0117 CE4-related: On MVP candidate list generation for AMVP [R. Yu, D. Liu, K. Andersson, P. Wennersten, J. Ström, R. Sjöberg (Ericsson)]

This contribution proposes a motion vector predictor (MVP) candidate list generation method for the advanced motion vector prediction (AMVP) mode in VTM-3.0. The method is claimed to simplify the current VTM-3.0 method by reducing the number of MV comparisons as well as reducing the number of MV rounding operations. It is stated that the number of MV comparisons in the worst case is reduced from 10 to 1. The number of MV rounding operations in the worst case is reduced from 13 to 3. The method was reportedly tested under VTM-3 random access (RA) and low delay B (LDB) common test configurations. The results are reported to be 0.01% RA and 0.01% LDB. It is asserted by the proponent that the method has marginal impact on the BD-rate performance while a simplification of the MVP list generation is achieved.

Reduce the number of pruning and rounding operations in AMVP candidate list construction.

Change the scanning order of HMVP candidates.

All rounding are done before pruning (as the adopted CE proposal M0281: test 4.1.5).

Recommendation: Revisit after crosschecker (LGE) ‘s report.

Revisit: Crosschecker confirmed the algorithm in SW matches with the proposal.

Recommendation: Adopt

JVET-M0422 CE4-related: Simplified MVD coding [X. Li, X. Xu, X. Zhao, S. Liu (Tencent)]

In MVD coding, abs_mvd_greater0_flag and abs_mvd_greater1_flag are context coded. In this contribution, it is proposed to use equal probability to code these two syntax elements to simplify MVD coding process. It is reported that bypass coding abs_mvd_greater0_flag leads to 0.12% (RA) and 0.09% (LDB) luma BD-rate increase, while bypass coding abs_mvd_greater1_flag results in 0.03% (RA) and -0.02% (LDB) luma BD-rate increase.

Discussion:

The crosschecker supports to adopt it.

Related to adopted CE proposal JVET-M0403

Recommendation: Revisit at Track B

JVET-M0444 CE4-related: Simplified symmetric MVD based on CE4.4.3 [J. Luo, Y. He (InterDigital)]

This contribution proposes a simplified symmetric MVD mode for bi-prediction based on the corresponding CE test CE4.4.3. With the symmetric MVD mode, MVD of list 1 is not signaled but derived from the signaled MVD of list 0, and reference picture indices of both list 0 and list 1 are not signaled but derived based on the POC of reference pictures and the current picture. Two items are proposed in this contribution: (1) Disabling bi-directional optical flow (BDOF) for symmetric MVD mode;(2) encoder optimizations to improve the performance. The simulation results reportedly show on average 0.33% BD rate saving with 105% encoding time for RA configuration over VTM 3.0.

Based on CE4.4.4. Two changes: 1) Disable BDOF on sMVD mode; 2) encoder optimization.

RA (-0.33% 105%). Not applied on LDB. Compared to CE results RA(-0.17%, 101%), the additional gain is 0.16%.

Discussion:

Better tradeoff than the CE test.

One expert supports to adopt it. And the crosschecker also supports.

Q: Why encoding time is increased?

A: More initial MVs are searched. The encoder optimization targets at coding gains.

Q: Can the encoder optimization be applied on the anchor?

A: No. It can only be applied when sMVD is used.

Recommendation: Adopt

JVET-M0502 CE4-related: Improved context for prediction mode flag [X. Zhao, X. Li, S. Liu (Tencent)]

This contribution proposes an improved context design for signaling the pred_mode flag. One context is added, and the context value is derived by whether neighboring blocks are coded by intra prediction, inter prediction or inter-intra prediction mode. It is reportedly shown that, on top of VTM-3.0, average -0.09%, and -0.02% luma BD-Rate is achieved under Random Access (RA) and Low Delay B (LDB) configurations, respectively.

Contexts are based on neighbouring block is inter, intra or inter-intra.

+2 context: RA/LB -0.09% /-0.02%,

+1 context: RA/LB -0.09% /-0.02%

Related proposals:

JVET-M0519 Non-CE: Context modeling for coding the prediction mode flag [S.-T. Hsiang, S.-M. Lei (MediaTek)]

+3 contexts

Contexts are based on neighbouring block is inter, intra and CU size.

One more context is added when CPR and dual-tree are enabled to code pred_mode for chroma.

RA/LB -0.11%/-0.07%,

JVET-M0513 CE7-related: Context modeling of pred_mode_flag

Not all companies proposing related contributions are at the BoG meeting.

Recommendation: Revisit

# Weighted-Prediction Modification (2)

JVET-M0067 Non-CE4: Weighted prediction with BDOF and bi-prediction with CU weights harmonization [T. Hashimoto, T. Chujoh, T. Ikai, E. Sasaki (Sharp)]

Weighted prediction (WP) which has been included in the conventional international video coding standards which are AVC and HEVC, is a well-known coding for illumination variations, for example black/white fade scenes. Although WP has not been included in the draft specification of VVC, it has been already implemented on current VTM-3.0 software. Since bi-directional optical flow (BDOF) and bi-prediction with CU weights called GBI which are new bi-directional prediction tools for VVC, are disabled with WP, in this contribution, how to perform those two tools is proposed. As experimental results using black and white fade sequences proposed at the Macao meeting, compared to VTM-3.0, average of 10.12%/28.79% improvements on RA/LB configurations for black fade and average of 11.71%/29.75% improvements on RA/LB configurations for white fade have been achieved. In the case of comparison with VTM-3.0 when WP is applied, average of 1.26%/0.10% improvements on RA/LB configurations for black fade and average of 1.00%/0.08% improvements on RA/LB configurations for white fade have been achieved.

It is proposed to apply GBI independently of WP. When both BDOF and WP are applied, weighted BDOF is introduced.

If gbiIdx == 0, WP and BDOF can be applied.

When WP is applied, a pre-processing is conducted before BDOF.

Check SAD at decoder, if it is small enough, weighted samples are applied.

Then BDOF is performed.

Discussion:

The semantics of gbi_idx equal to 0 is changed.

It is commented that the proposed method may cause loss in CTC, but the proponent has different opinion.

It is commented that the decoding process is changed a lot.

The logic is using a CU level syntax to revert a higher level syntax, which is not desirable.

It is commented that the worst case computational complexity is increased.

It is commented that BDOF applied with WP may be too complex.

The test is conducted under the test condition provided by Technicolor.

Recommendation: The syntax to support WP in M0111 is recommended to be adopted. And the same syntax design is also proposed in JVET-M0067.

JVET-M0111 AHG13: On bi-prediction with weighted averaging and weighted prediction [Y. Ye, J. Chen, M. Yang (Alibaba), P. Bordes, E. François (Technicolor)]

At the 12th JVET meeting, bi-prediction with weighted averaging (BWA) (previously referred to as generalized bi-prediction or GBi in JVET-L0646) was adopted into the VVC draft 3. It was further agreed that “weighted prediction should also be put in the draft,” and that BWA “and weighted prediction would be mutually exclusive at the picture level.” As of VVC draft 3 version 7, weighted prediction was not yet incorporated into the draft text. Therefore, this contribution first proposes to add the HEVC-style weighted prediction into the VVC. Further, this contribution proposes three alternatives to ensure that BWA and weighted prediction are invoked in a “mutually exclusive” manner in the sense that the decoding process of both tools will not be invoked at the same time.

In v2, proponents from Sharp Corporation were added, and a presentation PPT file was added to the zip archive.

Two aspects:

1) Add the HEVC-style weighted prediction into the VVC

2) Three alternatives to ensure that BWA and weighted prediction are invoked in a “mutually exclusive” manner.

a) gbi_idx is not signaled if SPS WP flag is on.

b) gbi_idx is not signaled if weighted prediction weightings are not zero with the current reference picture.

c) move gbi flag from SPS to PPS. Gbi_flag is inferred to be 0 if SPS WP flag is on.

Discussion:

Option 2 is CU level, and Option 1/3 are picture level.

One expert prefers 1/3), but 2) is also acceptable.

Experts comment that option 2 is useful in real applications.

It is agreed that WP should be included in VVC.

Several experts comment that option 2 will provide benefits on coding gains.

It is agreed that option 2 has no complexity burden.

No additional syntax elements are sent in option 2.

Recommendation: Adopt JVET-M0111 with option 2.

# Complexity Reduction (8)

JVET-M0230 CE4-related: Temporal MV buffer reduction [Y.-W. Chen, X. Wang (Kwai Inc.)]

In VTM-3.0, for higher coding efficiency, 1/16-pel motion compensation is supported and all the MVs are stored in 1/16-pel precision in the temporal MV buffer. To reduce the size of temporal MV buffer, we propose to store these MVs in 1/4-pel precision in the temporal MV buffer. It is asserted that the proposed method can reduce the required temporal MV buffer size by about 12%. According to the results reported, the proposed simplifications incur 0.03% and 0.09% BD-rate increase under RA and LDB conditions respectively.

Reduce the bits number of a MV component from 18 to 16 for TMVP storage.

Recommendation: Study in CE with JVET-M0346 and JVET-M0512.

JVET-M0346 CE4-related: Non-square compression grid for temporal motion data storage [S. H. Wang (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma (Peking University)]

In this contribution, it is proposed to reduce the temporal motion vector storage with a factor of 2 and the coding performance with respect to the granularity of MV storage is reported in two tests:

- Test(a): 8x16 unit based temporal motion compression

- Test(b): 16x8 unit based temporal motion compression

Simulation results for Test (a) shows 0.06% BD-rate loss in VTM Random access configuration and 0.23% BD-rate loss in Low delay B configuration.

Store the center MV instead of the top-left MV in a grid.

Test a: RA/LB 0.06%/0.23%;

Test b: RA/LB 0.07%/0.30%;

Recommendation: Study in CE.

JVET-M0348 CE4-related: Improvement on 4x4 bi-prediction [X. W. Meng (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma (Peking University)]

In current VVC standard, 4x4 bi-prediction inter CUs are disallowed because of the memory bandwidth issue. [1] And 4x8/8x4 bi-prediction inter CUs are allowed, which causes worst-case memory bandwidth increase. This contribution proposes an improvement method that combines 4x4/4x8/8x4 bi-prediction inter CUs with full pixel and 4-pixel AMVR, which allows 4x4/4x8/8x4 bi-prediction and avoid the increase of memory bandwidth compare to HEVC. There are two test conditions conducted, (a) 4x4 bi-directional inter CUs + full pixel and 4-pixel AMVR mode; (b) 4x4/4x8/8x4 bi-directional inter CUs + full pixel and 4-pixel AMVR mode. The proposed method (a) can achieve 0.01% and 0.03% bit-rate reduction with no encoding and decoding time increase respectively under RA and LDB cases. The proposed method (b) can reduce the worst-case memory access bandwidth by about 32% compared to the current VVC, with 0.01% and 0.03% bit-rate reduction and xxx% encoding and xxx% decoding time reduction respectively under RA and LDB cases.

It has already been discussed in CE2.

JVET-M0350 CE4-related: Quadtree-based Merge Estimation Region for VVC [T. L. Fu (Peking University), X. Zheng (DJI), S.S Wang, S.W. Ma (Peking University)]

In this contribution, a method for parallel merge estimation of neighboring blocks is proposed, which can handle quadtree, binary tree and ternary tree partitions. In principal, the proposed method sets the leaf node of quad-tree as the size of merge estimation region (MER), which means candidate blocks in the same leaf node of quad-tree is not included in the merge candidate list construction. Simulation results for N equal to 8, 16, 32 and 64 are presented in the contribution document.

Two conditions:

1) Use QT leaf as MER if QT size >= Threshold

2) If current CTU’s QT size >= Threshold, use the minimum MER size.

8*8 MER: RA 0.49%

Recommendation: Revisit in Track B after JVET-M0170 is revisited

JVET-M0479 Non-CE4: On clipping of scaled motion vectors [K. Misra, F. Bossen (Sharp)]

This contribution proposes a simplification method of triangle prediction merge list construction in VTM-3.0[1]. The proposed method reusing inter merge list as triangle prediction’s merge list. It is reported that as compared to VTM-3.0, xxx% B-D rate saving with xxx% encoding time and xxx% decoding time is achieved in random access configuration. 0.09% B-D rate increase with xxx% encoding time and xxx% decoding time is achieved in low delay B configuration.

RA/LB 0.00%/0.00%

In some cases, MV is clipped to 16 bits instead of 18 bits (both in SW and WD). It is proposed to always clip MV to 18 bits.

Discussion:

All MV clips in SW and WD should be unified as clipped to 18 bits.

Recommendation: Adopt (BF)

JVET-M0484 Non-CE4: Line buffer size reduction method for generalized bi prediction [T. Solovyev, H. Gao, S. Esenlik, S. Ikonin, J. Chen (Huawei)]

This contribution describes modification of generalized bi prediction index derivation process for merge mode and for affine mode. In order to reduce the line buffer size, borrowing of generalized bi prediction index is restricted to be within the current CTU line. The simulations results reportedly show 0.02% and 0.06% luma BD-rate loss for RA and LDB configurations respectively, without significant change in encoding and decoding times.

Remove the gbi_index (3 bits per 4x4 block) stored in line-buffer.

RA/LB: (0.02% /0.06%)

Q: What’s the ratio of bits on line-buffer is saved?

A:Originally there are 84 bits per 4x4 block, so about 3.57% line buffer size can be saved.

One expert supported to adopt it. One expert commented that there is a loss so it may not be adopted.

It is commented that the benefit is small compared with the loss.

It is encouraged to reduce the line-buffer size, e.g. gbi_index.

Recommendation: Further study.

JVET-M0507 CE4-related: Hybrid Merge Estimation Region [H. Wang, V. Seregin, W.-J. Chien, T. Hsieh, Y. Han, M. Karczewicz (Qualcomm)]

In this contribution, a method of reducing the local inter-dependency as well as the number of merge candidate list generation operations is proposed. The method uses 2 parameters (N1 and N2) as 2 thresholds of coding splitting node area. (N1≤N2). For splitting nodes with the size of N2, all merge candidate list generation operations within the area can be performed in parallel. And for all blocks within a splitting area with the size of N1, a shared merge candidate list is used. Simulation results for multiple sets of N1 and N2 are presented.

Merge list sharing and MER are both supported.

It is reported that the proposed method may bring benefits on the merge sharing method in JVET-M0170.

Recommendation: Further study

JVET-M0512 Non-CE4: On Temporal Motion Buffer Compression [F. Bossen, K. Misra, A. Segall (Sharp Labs of America)]

This contribution proposes a method to reduce the number of bits stored in the temporal motion buffer.  It is reported that the current VVC WD requires 22.5 bits of storage per 4x4 luma sample block, and it is further asserted that the proposed approach reduces the amount requirement to 10.5 bits per 4x4 luma sample block with negligible impact on coding efficiency.  The storage reduction is achieved by first scaling the original temporal motion vector to a fixed POC distance, eliminating the need to store delta POC information. The approach then converts the scaled motion vector value to a 6-bit mantissa and 4-exponent representation prior to storage. The selected representation effectively quantizes larger motion vector values more coarsely while retaining higher precision for smaller motion vectors. The proposed method was implemented in VTM-3.0 and its performance evaluated using Random Access (RA) and Low Delay B (LDB) configurations from the Common Test Conditions. It is reported that the proposed method results in an average luma BD Rate of 0.00% (RA), 0.02% (LDB).

Two aspects:

1) Remove delta POC storage: Scale MV during storage to a fixed delta POC (=4)

2) Converts the scaled motion vector value to a 6-bit mantissa and 4-exponent representation prior to storage.

0.00% (RA), 0.02% (LDB)

Discussion:

It is commented that aspect 1) may be lossless and can be applied on other TMVP storage methods.

The proponent commented that aspect 1) may also introduce loss due to MV precision rounding.

It is asked the complexity of the conversion of MV?

The proponent answered that it can be provided later.

Recommendation: Study in CE.