| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0847 |
| --- | --- |

| Title: | Cross-check of JVET-M0524 (CE3/6-related: Unification of RQT-like transform partitioning for intra and inter blocks) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pierrick Philippe | Email: | pierrick.philippe@orange.com |
| Source: | Orange / bcom |  |  |

_____________________________

# Abstract

This document reports the crosscheck results of JVET-M0524. It is reported that the average BD-rates match those provided by the proponents. No discrepancies have been found between the provided software and the description of the tests.

# Description

Two methods are proposed in JVET-M0524:

- Method 1: The method tested in CE6.4.2 is extended to be applied for intra predicted blocks.

- Method 2: Compared to CE6-4.2 TU splits are further allowed to be partitioned if either width or height of a TU is larger than 4

# Experimental Results

Experiments were performed based on VTM3.0, using the common test conditions described in JVET-L1010. No discrepancies have been found between the provided software and the description of the tests. The execution times are within the measurement error range.

The methods are tested in AI and RA, for the tests the reference is VTM-3.0 with the CTC.

Inter MTS are enabled for Method 1 & 2 (in random access), while not activated for the reference VTM-3.0 encoder.

The coding results below match perfectly those presented in JVET-M0524, the runtimes are similar, within the error measurement range.

Note that for the RA cases, the relative timings reported are not perfectly reliable, due to heterogeneous hardware.

## Method 1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.08% | -0.05% | 125% | 98% |
| Class A2 | -0.14% | -0.10% | -0.12% | 122% | 99% |
| Class B | -0.19% | -0.13% | -0.15% | 123% | 98% |
| Class C | -0.22% | -0.23% | -0.31% | 122% | 99% |
| Class E | -0.32% | -0.28% | -0.08% | 125% | 98% |
| Overall | -0.19% | -0.16% | -0.15% | 123% | 99% |
| Class D | -0.10% | -0.15% | -0.10% | 120% | 99% |
| Class F | -0.63% | -0.34% | -0.28% | 121% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class B | -0.85% | 0.32% | 0.45% | 143% | 102% |
| Class C | -0.69% | 0.39% | 0.69% | 154% | 104% |
| Class E |  |  |  |  |  |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class D | -0.85% | 0.18% | 1.08% | 154% | 103% |
| Class F | -0.80% | -0.12% | -0.13% | 139% | 101% |

## Method 2:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.10% | -0.04% | 140% | 99% |
| Class A2 | -0.17% | -0.11% | -0.13% | 139% | 100% |
| Class B | -0.26% | -0.17% | -0.16% | 144% | 98% |
| Class C | -0.43% | -0.32% | -0.45% | 148% | 99% |
| Class E | -0.42% | -0.26% | -0.17% | 148% | 97% |
| Overall | -0.29% | -0.20% | -0.20% | 144% | 98% |
| Class D | -0.29% | -0.10% | -0.15% | 149% | 98% |
| Class F | -1.77% | -1.23% | -1.21% | 154% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class B | -0.94% | 0.26% | 0.38% | 150% | 102% |
| Class C | -0.89% | 0.15% | 0.42% | 164% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class D | -1.07% | 0.00% | 0.72% | 168% | 103% |
| Class F | -1.91% | -0.93% | -0.89% | 153% | 100% |

# Patent rights declaration

b<>com, Orange may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).