| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0852 |
| --- | --- |

| Title: | Crosscheck of JVET-M0397 (CE6-related: DST-3 based transform kernels derivation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jaeil Kim Kyunghwan Ko Jason Jung | Tel: Email: | +82-10-9396-2113 jaeilkim@sk.com khko@pixtree.com jason.jung@wilusgroup.com |
| Source: | SK telecom, PIXTREE, WILUS Inc. |  |  |

_____________________________

- Abstract

This document reports cross-check results of JVET-M0397 where an efficient and low complexity transform kernels is derived for a 64×64 DST-3 kernel, which can be partitioned into 32×32 DST-4, 16×16 DST-4, 8×8 DST-4, 4×4 DST-4, 2×2 DST-4 and 2×2 DST-3. The experiments were done under the common test conditions (CTC). It is obvious that the cross-check results are matched with the experimental results provided by the proponent of JVET-M0397 [1] with negligible EncT and DecT differences.

Cross-check results

The source code based on VTM-3.0 was delivered by the proponent provided by the proponent [1]. As same to the proponent, Tables show BD-rate results of [1].

BD-rate results of JVET-M0397

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.44% | 0.80% | 0.95% | 97% | 95% |
| Class A2 | 1.88% | 1.21% | 1.17% | 97% | 95% |
| Class B | 1.95% | 1.53% | 1.49% | 97% | 95% |
| Class C | 1.94% | 1.33% | 1.16% | 98% | 96% |
| Class E | 2.88% | 2.80% | 2.62% | 99% | 95% |
| Overall | 2.00% | 1.52% | 1.46% | 98% | 95% |
| Class D | 1.91% | 1.19% | 1.05% | 98% | 97% |
| Class F | 1.59% | 1.29% | 1.33% | 99% | 98% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.53% | 0.33% | 0.51% | 100% | 99% |
| Class A2 | 0.63% | 0.39% | 0.46% | 100% | 99% |
| Class B | 0.92% | 0.87% | 0.75% | 100% | 99% |
| Class C | 1.01% | 0.76% | 0.79% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.81% | 0.64% | 0.66% | 100% | 99% |
| Class D | 0.87% | 0.21% | 0.48% | 100% | 100% |
| Class F | 1.06% | 0.84% | 0.94% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.27% | 0.10% | 0.33% | 100% | 100% |
| Class C | 0.31% | 0.43% | 0.31% | 100% | 100% |
| Class E | 0.42% | 0.64% | 0.80% | 100% | 101% |
| Overall | 0.32% | 0.35% | 0.44% | 100% | 100% |
| Class D | 0.27% | -0.10% | 0.98% | 100% | 101% |
| Class F | 0.39% | 0.68% | 0.43% | 100% | 101% |

Conclusion

In this contribution, performance of JVET-M0397 [1] was verified. It is reported that the experimental results are matched with those of the proponent except for negligible differences in encoding and decoding time due to different experiment environments.

Reference

[1] S. Shrestha, A. Kumar, B. Lee, Y. Lee, J. Park, “CE6-related: DST-3 based transform kernels derivation,” JVET-M0397, 13th JVET meeting, Marrakech, MA, Jan. 2019

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th JVET meeting, Macau, CN, Oct. 2018.