| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0xxx-v1 | Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-M0xxx |
| --- | --- | --- | --- |

| Title: | Crosscheck of JVET-M0283 (CE10-related: Reduction of motion predictor pruning in Triangle Merge mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This document reports crosscheck results for the first two tests of JVET-M0283 (CE10-related: Reduction of motion predictor pruning in Triangle Merge mode). The crosscheck results match the ones provided by the proponent.

# Introduction

In JVET-M0187 [1], two simple motion predictor pruning processes to be applied in Triangle Merge modes are proposed. The proposed processes have been implemented in the JVET VTM3.0.

Test 1 proposes to:

- Harmonize first spatial candidates pruning as in Merge mode since these candidates are the same as in Merge,

- Perform a simple pruning of the temporal candidates together,

- Remove the pruning of uni-directional candidates since the pre-list is already (partially) pruned, and

- Remove the pruning of averaged candidates.

In Test 2 all pruning of the motion predictor list construction are removed, i.e. in addition to the first test the following modifications are done:

-	Remove the pruning of L0 parts of bi-directional candidates,

-	Remove the pruning of L1 parts of bi-directional candidates.

# Simulation results

Simulation results for both tests are given below.

Table1. Results for Test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | -0.01% | -0.03% | 0.03% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.03% | 0.08% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.02% | 0.01% | 0.15% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
| TGM | 0.05% | 0.07% | 0.08% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.03% | 0.13% | 0.04% | 100% | 99% |
| Class E | -0.04% | 0.60% | -0.18% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.01% | 0.32% | 0.26% | 100% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table2. Results for test 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.02% | 0.06% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.01% | 0.06% | -0.02% | 100% | 99% |
| Class C | 0.06% | 0.10% | 0.09% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.08% | 0.01% | 0.07% | 100% | 99% |
| Class F | 0.02% | 0.05% | 0.01% | 100% | 99% |

# References

- CE10-related: Reduction of motion predictor pruning in Triangle Merge mode, JVET-M0283

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th Meeting: Macau, CN, 3–12 Oct. 2018