| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0870 |
| --- | --- |

| Title: | JVET common test conditions and evaluation procedures for MCTS and sub-pictures with boundary padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | M. Coban R. Skupin | Email: | mcoban@qti.qualcomm.com robert.skupin@hhi.fraunhofer.de |
| Source: | JVET |  |  |

_____________________________

# Abstract

This document defines common test conditions (CTC), conversion practices, and software reference configurations to be used in evaluation of MCTS and sub-picture coding schemes. These common test conditions are also recommended for use in technical contributions to the following JVET meetings, if applicable.

# Introduction

Common test conditions are desirable to conduct experiments in a well-defined environment and ease the comparison of the outcome of experiments.

This document defines 2 test conditions, reflecting random-access and low-delay settings:

- Random access, 10 bit

- Low delay, 10 bit

Version 3.0 of the VTM software is expected to be used for most experiments. Version 8.0 of the 360Lib software should be used for conversions. More recent versions are encouraged where applicable.

The following sections define testing procedure, test sequences, coding projection formats, quantization parameter values, evaluation procedure, configuration files, and compile-time options to be used.

# Testing procedure

For a given input 360° video, conversion is applied first to convert source projection format into CMP format. The resulting CMP format YUV files will be used as input YUVs for MCTS anchor generation [3]. For sub-picture boundary padding evaluation, the sub-picture YUV regions will be cropped from CMP format YUV files. The resulting cropped YUV files will be used as input YUVs for sub-picture evaluation.

# Test sequences

Table 1 defines the set of test sequences to be used for random-access, and low-delay conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences and test cases described below.

Test sequences are available on ftp://jvet@ftp.ient.rwth-aachen.de/testsequences/testset360 and ftp://jvet@ftp.hhi.fraunhofer.de/testsequences/testset360

Test sequence are only available to qualified JVET participants. Qualified participants may contact the JCT-VC/JVET chairs for login information.

Table 1. Test sequences

| Class | Sequence name | Width | Height | Frame count | Frame rate | Bit depth | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | SkateboardInLot | 8192 | 4096 | 300 | 30 fps | 10 | M | O |
| S1 | ChairLift | 8192 | 4096 | 300 | 30 fps | 10 | M | O |
| S1 | KiteFlite | 8192 | 4096 | 300 | 30 fps | 8 | M | O |
| S1 | Harbor | 8192 | 4096 | 300 | 30 fps | 8 | M | O |
| S1 | Trolley | 8192 | 4096 | 300 | 30 fps | 8 | M | O |
| S1 | GasLamp | 8192 | 4096 | 300 | 30 fps | 8 | M | O |

Notes:

- When the encoder operates in 8-bit mode (InternalBitDepth=8) for a 10-bit source, each 10-bit source sample x is converted prior to encoding to an 8-bit value (x+2) / 4 clipped to the [0,255] range. Similarly, when the encoder operates in 10-bit mode (InternalBitDepth=10) for an 8-bit source, each 8-bit source sample x is converted prior to encoding to a 10-bit value 4*x. This behavior is built into the reference encoder and no external conversion program is required.

- M and O are for mandatory and optional, respectively.

Table 2. MD5 checksum of test sequences

| Sequence name | MD5 checksum |
| --- | --- |
| SkateboardInLot | e8eae04c43e959060f641fec4892fced |
| ChairLift | 9126f753bb216a73ec7573ecc4a280c3 |
| KiteFlite | 18c0ea199b143a2952cf5433e8199248 |
| Harbor | aa827fdd01a58d26904d1dbdbd91a105 |
| Trolley | 25c1082d1e572421da2b16530718156d |
| GasLamp | 3417d0b862ffb0fd34f65c3bc810d25c |
| Balboa | 1457bb109ae0d47265f5a589cb3464d7 |
| Broadway | 1baf6667ef84bc3534809463faee8e82 |
| Landing2 | d1b064b272d82d4de6f853a760c2f75e |
| BranCastle2 | 9b411717977f6c50aadb458f52b96951 |

# Coding projection format

Prior to encoding, the high resolution equi-rectangular projection (ERP) [1] input shall be converted to low resolution coding projection format at resolutions specified in Table 3. The conversion shall use 6-taps Lanczos filter for luma and 4-taps Lanczos filter for chroma with 10 bits internal bit-depth. Output bitdepth shall be equal to the input bitdepth.

Table 3. Frame sizes for different projections for 8K and 6K input resolution

| Format | 8K and 6K Input ERP Resolution |  |  |  |
| --- | --- | --- | --- | --- |
|  | faceW | faceH | W | H |
| CMP [1] | 1536 | 1536 | 4608 | 3072 |

# Quantization parameter values

The same quantization parameter values as in the SDR CTC [2] are to be used for each video sequence.

# Anchor

The anchor is provided for the test model (VTM) encoded in CMP format in MCTS configuration with tile configurations listed below. Config files are provided, as described in section 8.

Table 4: Tile configurations.

| Tile Configuration | Tile Width | Tile Height |
| --- | --- | --- |
| 6x4 | 768 | 768 |
| 12x8 | 384 | 384 |

# Evaluation

## Reported metrics

JVET CTC is used to for evaluation of coding methods. PSNR (Y, Cb, Cr) metric between the input and output of the encoder is used as distortion metric. For computation of bitrate for sub-picture configuration, the extra bitrate from having multiple bitstreams is accounted. Extra parameter set data will be deducted from the total bitrate. Tile entry point and configuration info shall be added to the total bitrate. The goal is to equalize the single stream MTCS overhead and the sub-picture configuration overhead. The average distortion shall be calculated by using hexPSNR values.

# Configuration files

The same encoder configuration files as in the SDR CTC [2] are to be used for each test case. Sequence-specific parameters are to be found in the cfg-360Lib/per-sequence/360/ folder. Projection format-specific parameters are to be found in the cfg-360Lib/ and cfg-360Lib/360Lib/ folders.

# Parallel encoding/decoding

The same parallel encoding/decoding as in the SDR CTC [2] may be applied for RA configurations.

# Compile-time settings

The same compile-time settings as in the SDR CTC [2] are to be used.

# References

- Y. Ye, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 8”, Joint Video Exploration Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1004, October 2018, Macau, CN.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1010, October 2018, Macau, CN.

- R. Skupin, V. George, K. Sühring, Y. Sanchez, T. Schierl, “AHG12: On motion constrained tiles for VVC”, Joint Video Exploration Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M0445, January 2019, Marrakech, MA.