| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0882-v2 |
| --- | --- |

| Title: | Cross check of CE7-related: Context modeling of pred_mode_flag (JVET-M0513) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Min Woo Park Heechul Yang Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | m.w.park@samsung.com h_c.yang@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This document reports the cross check results of “CE7-related: Context modeling of pred_mode_flag (JVET-M0513)”. The simulation was carried out under CTC. The results provided by the proponent were verified.

Cross Check Results

The simulation was carried out under the JVET common test condition [1]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Huawei provided the source code of their proposal based on the VTM-3.0 software and results. The results are the same with those provided by the proponents.

Table 1. Test 1 results (LDP)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.08% | -0.03% | 94% | 94% |
| Class A2 | -0.09% | 0.08% | 0.08% | 91% | 93% |
| Class B | -0.09% | 0.09% | -0.07% | 92% | 92% |
| Class C | -0.08% | -0.05% | -0.01% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.02% | -0.02% | 94% | 95% |
| Class D | -0.03% | -0.01% | -0.06% | 98% | 98% |
| Class F (optional) | -0.05% | -0.01% | -0.01% | 96% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | 0.29% | 0.14% | 90% | 94% |
| Class E | -0.03% | 0.41% | -0.13% | 88% | 83% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.19% | 0.16% | 90% | 90% |
| Class F (optional) | 0.08% | -0.16% | 0.24% | 93% | 92% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.15% | 0.42% | 90% | 98% |
| Class C | -0.08% | 0.08% | 0.08% | 90% | 89% |
| Class E | 0.00% | -0.22% | -0.64% | 90% | 90% |
| Overall | -0.05% | 0.03% | 0.04% | 90% | 93% |
| Class D | 0.01% | -0.27% | 0.30% | 93% | 94% |
| Class F (optional) | 0.00% | -0.08% | -0.30% | 91% | 87% |

Table 2. Test 2 results (RA, LDB and LDP)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | -0.10% | -0.06% |  | 0.01% |  | 93% |  | 95% |  |
| Class A2 | -0.08% | 0.03% |  | 0.09% |  | 96% |  | 96% |  |
| Class B | -0.10% | 0.16% |  | 0.03% |  | 93% |  | 92% |  |
| Class C | -0.08% | -0.01% |  | 0.12% |  | 99% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.09% | 0.04% |  | 0.06% |  | 95% |  | 95% |  |
| Class D | -0.02% | -0.02% |  | 0.04% |  | 96% |  | 100% |  |
| Class F (optional) | -0.07% | -0.06% |  | -0.03% |  | 95% |  | 95% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.24% |  | -0.10% |  | 92% |  | 101% |  |
| Class C | -0.06% | 0.25% |  | -0.04% |  | 91% |  | 97% |  |
| Class E | 0.06% | 0.05% |  | -0.29% |  | 89% |  | 86% |  |
| Overall | -0.03% | -0.01% |  | -0.13% |  | 91% |  | 96% |  |
| Class D | -0.04% | 0.00% |  | 0.30% |  | 94% |  | 92% |  |
| Class F (optional) | 0.03% | 0.06% |  | -0.04% |  | 94% |  | 96% |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.08% | 0.24% |  | 0.06% |  | 100% |  | 101% |  |
| Class C | -0.05% | 0.28% |  | 0.03% |  | 91% |  | 90% |  |
| Class E | -0.20% | -0.56% |  | -0.95% |  | 97% |  | 96% |  |
| Overall | -0.10% | 0.05% |  | -0.20% |  | 96% |  | 96% |  |
| Class D | -0.01% | -0.08% |  | 0.12% |  | 94% |  | 91% |  |
| Class F (optional) | -0.02% | 0.02% |  | -0.09% |  | 92% |  | 95% |  |

Table 3. Test 3 results (RA, LDB and LDP)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.19% | 0.04% | 92% | 94% |
| Class A2 | -0.02% | 0.19% | 0.15% | 94% | 94% |
| Class B | -0.02% | 0.21% | 0.06% | 91% | 90% |
| Class C | -0.04% | 0.15% | 0.12% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.19% | 0.09% | 93% | 93% |
| Class D | -0.06% | -0.11% | -0.01% | 98% | 99% |
| Class F (optional) | -0.02% | -0.01% | 0.03% | 95% | 92% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | 0.16% | 0.20% | 91% | 94% |
| Class E | 0.17% | -0.29% | 0.10% | 89% | 86% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.12% | 0.21% | 93% | 89% |
| Class F (optional) | 0.23% | 0.18% | 0.15% | 95% | 97% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.25% | 0.08% | 92% | 96% |
| Class C | -0.06% | 0.17% | 0.06% | 90% | 87% |
| Class E | 0.05% | -0.36% | -0.44% | 98% | 91% |
| Overall | -0.01% | 0.07% | -0.05% | 93% | 92% |
| Class D | 0.02% | -0.13% | 0.21% | 94% | 93% |
| Class F (optional) | 0.05% | -0.10% | -0.13% | 94% | 91% |

It should be noted that encoding and decoding times of the cross check results might be different from those provided by the proponent because the simulation was conducted in a different computing environment.

Reference

[1] “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, Oct. 2018.