| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0883 |
| --- | --- |

| Title: | CE10-related: Using regular merge index signaling for triangle mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang, Wei-Jung Chien, Vadim Seregin, Yung-Hsuan Chao, Han Huang, Marta Karczewicz (Qualcomm) Xianglin Wang, Yi-Wen Chen (Kwai) Timofey Solovyev, Semih Esenlik, Sergey Ikonin, Jianle Chen (Huawei) Meng Xu, Xiang Li, Shan Liu (Tencent) | Tel: Email: | 1-858-658-4535 hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated, Kwai Inc., Huawei Technologies. Co., Ltd, Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to replace look-up table based signaling in the triangular mode with a triangle split direction flag and two merge indices. Test results reportedly show -0.01% BD rate reduction for RA and LB configurations, respectively

# Introduction

In the triangle mode, a CU is split into 2 triangular PU by its diagonal or anti-diagonal line. The motion information of triangular PU mode (splitting direction, motion indexes of the 2 PUs) is combined into a value with a range of [0,39]. The mapping between the combined value and the motion information parameters is performed by a look-up table.

# Proposed method

In this contribution, the look-up table is removed and instead of combining the motion information into a single value, 3 values are encoded into bitstream:

- The splitting direction flag is encoded using bypass bin.

- The merge index of PU0 (Idx0) is encoded in the same way as the merge index of regular merge mode.

- Since the value of Idx0 and Idx1 cannot be equal. For PU1, instead of encoding the value of Idx1, the value of Idx1'=Idx1-(Idx1<Idx0 ?0 :1) is signalled.

Proposed syntax modification is provided in the table below.

| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| --- | --- |
| { |  |
| merge_triangle_split_dir[x0][y0] | u(1) |
| merge_triangle_idx0[x0][y0] | ae(v) |
| merge_triangle_idx1[x0][y0] | ae(v) |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

Table 8-8 – Specification of m and n using merge_triangle_idx[ xCb ][ yCb ]

| merge_triangle_idx[ xCb ][ yCb ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | 1 | 0 | 0 | 0 | 2 | 0 | 0 | 1 | 3 | 4 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 2 |
| n | 0 | 1 | 2 | 1 | 0 | 3 | 4 | 0 | 0 | 0 | 2 | 2 | 2 | 4 | 3 | 3 | 4 | 4 | 3 | 1 |
| merge_triangle_idx[ xCb ][ yCb ] | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| m | 2 | 2 | 4 | 3 | 3 | 3 | 4 | 3 | 2 | 4 | 4 | 2 | 4 | 3 | 4 | 3 | 2 | 2 | 4 | 3 |
| n | 0 | 1 | 3 | 0 | 2 | 4 | 0 | 1 | 3 | 1 | 1 | 3 | 2 | 2 | 3 | 1 | 4 | 4 | 2 | 4 |

# Test results

The proposed method was implemented on top of VTM-3.0 software and tested under common test conditions described in JVET-L1010.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.06% | 97% | 99% |
| Class A2 | -0.02% | -0.05% | 0.07% | 101% | 99% |
| Class B | -0.02% | 0.13% | 0.06% | 99% | 99% |
| Class C | -0.01% | 0.03% | 0.12% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | 0.08% | 99% | 99% |
| Class D | -0.02% | -0.12% | -0.09% | 100% | 99% |
| Class F | -0.01% | -0.03% | -0.01% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.24% | 0.24% | 99% | 95% |
| Class C | -0.01% | 0.09% | -0.03% | 102% | 101% |
| Class E | 0.00% | 0.11% | -0.41% | 100% | 99% |
| Overall | -0.01% | -0.04% | -0.01% | 100% | 98% |
| Class D | -0.03% | 0.05% | 0.70% | 100% | 101% |
| Class F | -0.01% | 0.28% | 0.29% | 99% | 97% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).