| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0895 |
| --- | --- |

| Title: | Cross-check result of JVET-M0627 (Non-CE4: Supplementary results of combined solution of JVET-M0255, JVET-M0267 and JVET-M0069) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research) |  |  |

_____________________________

- Abstract

This document is a cross-check report of JVET-M0627. The proposal modifies the MMVD signalling, which provides more gain for screen content than that for camera captured content. Bitrate and PSNR are matched with data provided by the proponents. Software implementation is matched with the proposal of the contribution.

Introduction

JVET-M0627 proposes to modify the MMBD signalling method by combining four methods [1]. The provided software implementation is match with that in the contribution.

The reference was generated by using the common test condition [2]. Provided software was compiled on 64-bit CentOS environment. The experiments were performed on the cluster PCs.

Table 1 Summary results of the random access condition by the JCTVC-M0627

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.27% | -0.26% | 101% | 101% |
| Class A2 | -0.23% | 0.02% | 0.10% | 103% | 101% |
| Class B | -0.21% | 0.03% | -0.08% | 102% | 101% |
| Class C | -0.19% | 0.00% | -0.02% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | -0.04% | -0.06% | 102% | 100% |
| Class D | -0.15% | 0.12% | 0.13% | 103% | 101% |
| Class F | -0.13% | -0.16% | -0.18% | 100% | 99% |
| Class SCC | -1.30% | -1.14% | -1.19% | 97% | 101% |

Table 2 Summary results of the low-delay B condition by the JCTVC-M0627

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.12% | -0.35% | 102% | 101% |
| Class C | -0.25% | -0.04% | 0.05% | 102% | 101% |
| Class E | -0.21% | -0.27% | -0.48% | 103% | 100% |
| Overall | -0.22% | -0.13% | -0.25% | 102% | 101% |
| Class D | -0.20% | 0.13% | -0.29% | 102% | 101% |
| Class F | -0.28% | -0.46% | -0.10% | 100% | 101% |
| Class SCC | -2.35% | -2.26% | -2.31% | 99% | 100% |

Conclusion

We had verified results reported in JCTVC-M0627. The results of the cross-check matched with proponent’s report. The encoding / decoding run-times also matched closely to those reported by the proponent.

References

- H. Liu, K. Zhang, L. Zhang, J. Xu, E. Sasaki, T. Chujoh, T. Ikai, “Non-CE4: Supplementary results of combined solution of JVET-M0255, JVET-M0267 and JVET-M0069”, JVET-M0627, Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

KDDI does not have any current or pending patent rights relating to the technology described in this contribution.