| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0904-v1 |
| --- | --- |

| Title: | BoG report on neural networks for video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yiming Li Shan Liu | Email: | userlym@whu.edu.cn shanl@tencent.com |
| Source: | BoG coordinators |  |  |

_____________________________

# Abstract

This contribution provides the report of the BoG on neural networks (NN) for video coding, especially for neural network based loop filter. An information report (JVET-M0691) is discussed in this BoG, and then the CE plan.

The BoG recommends the following plan of CE:

- Divide the 6 NN based loop filter methods into two category and build two sub CE test.

SubCE1 is the test for sequence-adaptive (two-pass) method, SubCE2 is the test for sequence-independent (one-pass) method.

- a) Investigate the impact of NN filter position in the filter chain.

Due to there are many test cases, only choose some cases to test, as defined below.

- For SubCE1:

anchor: DBF + SAO + ALF

case 1a : DBF + SAO + ALF + NN filter, which is the case currently subCE1 contributions used.

case 2a : NN filter.                   (optional test)

- For SubCE2:

anchor: DBF + SAO + ALF

case 1a : DBF + NN filter + SAO + ALF, which is the case that most of the contributions used.

case 2a : NN filter.                   (optional test)

case 3a : NN filter + ALF.             (optional test)

- b) Investigate the benefit of the CTU/block level NN filter adaptive on/off.

Define some cases to test. To test this part, other parts should be fixed, e.g., using case 1a as the position set and case 1c as the QP set.

case 1b: make the NN filter adaptive on/off at CTU level.

case 2b: always use NN filter in one slice. (optional test)

- c) Investigate the performance of the NN filter when the test QP is not the same as the training QP.

Define some cases to test. To test this part, one should fix using case 1a and case 1b.

Using CTC QP (22, 27, 32, 37) for training and get the NN parameter.

case 1c: based on the NN parameter, use CTC QP for testing

case 2c: based on the NN parameter, use CTC QP + 2 for testing (24, 29, 34, 39) (optional test)

case 3c: based on the NN parameter, use CTC QP - 2 for testing (20, 27, 30, 35) (optional test)

- More detailed assignment of CE can be seen in Table 3 and Table 4. Mandatory test and optional short test are marked in the Table 2.

The BoG recommends the following to be discussed.

- For subCE1, due to the online training, the NN parameter is not fixed. Thus, only providing the code in the inference stage is insufficient for others to use. (Although the parameter training/generation process is encoder only)

The proponent of Intel is willing to provide the training method, but not before the next meeting. The use of Tensorflow or other third-party framework in the common test conditions should be discuss in the track.

- For subCE1, training process need GPU (using CPU is 10x slower), but the inference stage/second pass only use CPU. The training process is separated from the second pass. Currently, the encoding time listed in the table only includes the coding time for the second pass, and the training time is reported separately. Does it need to add the training time and second coding time for reporting? Or whether there is another better solution?

- Information Document

JVET-M0691 AHG9: Complexity analysis about neural network video coding tools [Y. Li, Z. Chen(Wuhan Univ.), S. Liu(Tencent)]

This document provides a brief complexity analysis of the AHG9 related input documents for this meeting. Their information is collected and put in one table for comparison, which can also help to discuss and analysis these methods more efficiently in this meeting.

Encoding and decoding time is related to number of layers, size of convolution, and number of parameters.

How long does it take to train the NN parameters for the online (2pass) training methods? Taking JVET-M0566 as an example, it takes 15min (GTX1080ti) per RAS. Eight frames (I plus the previous seven frames) are used for training. JVET-M0159 takes a few hours (GTX1080ti) to train one RAS.

For online (2pass) training methods, the training time is not included in reported encoding time.

For online (2pass) training methods, GPU is preferred because (1) training has to be done each time together with encoding; (2) CPU is a lot slower - approximately 10 times slower, commented by proponent of JVET-M0566.

It is generally agreed that GFLOPs is not a very useful factor for measuring complexity, thus may be removed from the reporting template.

It is commented that it may be useful to include parameter precision (float or integer? Bit-depth?) in the reporting template.

- The list of discussion of CE plan

It is suggested to divide the 6 neural network (NN) based filter contributions into 2 categories and build two sub CE test. SubCE1 is the test for sequence-adaptive (two-pass) method, SubCE2 is the test for sequence-independent (one-pass) method.

For SubCE1, the discussions are listed below:

1. What’s position of NN filter in the current contributions?

Mediatek’s NN filter is after the ALF. Intel’s NN filter is after the ALF. Because for the online training process, the training data is the reconstruction is after ALF process.

2. How to investigate the impact of NN filter position in the filter chain?

Due to the online training, if it would like to put the NN filter in the middle of the filter chain, it is hard to get the training data. So, it is agreed that they only need to test to put the filter in the last position of the filter chain.

3. What’s the benefit of the CTU/block level NN filter adaptive on/off?

Both of the two contributions have used the NN filter block level on/off. They do not need to add test for this investigation.

4. Does the SubCE1 need to investigate the performance that the test QP is not the same as the training QP?

Since they have used online training, the process can guarantee the consistency of the QP in training and test process.

5. For subCE1, since it is online training, it needs the training process for each sequence. Although only the inference stage is available, how can other people use it since they do not have the code for training?

The proponent of Intel is willing to provide the training method, but not before the next meeting. The use of Tensorflow or other third-party framework in the common test conditions should be discuss in the track.

6. After the discussion of SubCE2, it is mentioned SubCE1 can also test using NN filter to replace all the traditional filter. It is an optional test, and the test condition is the same as what will be mentioned in SubCE2.

case 1a : DBF + SAO + ALF + NN filter, which is the case currently subCE1 contributions used.

case 2a : NN filter.                   (optional test)

For SubCE2, the discussions are listed below:

1. What’s position of NN filter in the current contributions?

In the Hikvision’s contribution, it has two methods, method1 is to replace all the traditional filters, and method 2 is to replace the traditional filters, except ALF.

For KDDI, USTC, and WHU, their filter are added between DBF and SAO. One expert mentioned ALF and SAO may correct the artifact of the NN network output, thus they add the filter between SAO and DBF.

2. How to investigate the impact of NN filter position in the filter chain?

It is mentioned that considering the number of traditional filters in the filter chain. There are many different combinations. To make the CE test more practical, several tests are selected, as listed:

anchor : DBF + SAO + ALF

case 1a : DBF + NN filter + SAO + ALF, which is the case that most of the contributions used.

case 2a : NN filter.                   (optional test)

case 3a : NN filter + ALF.             (optional test)

case 2a and case 3a are optional, but case 2a has high priority.

3. What’s the benefit of the CTU/block level NN filter adaptive on/off?

All of the methods in the related contributions have used the CTU basis switching. But it is suggest to add an optional test to investigate the benefit of the CTU basis switching.

case 1b: make the CNN filter adaptive on/off at CTU level.

case 2b: always use CNN filter in one slice. (optional test, KDDI may try this)

4. How to investigate the performance of the NN filter when the test QP is not the same as the training QP.

One expert suggests we can train the network in CTC QP, for the test phase, we can test in the non-CTC QP condition. Specifically:

Using CTC QP (22, 27, 32, 37) for training and get the NN parameter.

case 1c: based on the NN parameter, use CTC QP for testing

case 2c: based on the NN parameter, use CTC QP + 2 for testing (24, 29, 34, 39) (optional test, USTC and WHU may test it)

case 3c: based on the NN parameter, use CTC QP - 2 for testing (20, 27, 30, 35) (optional test, USTC and WHU may test it)

The proponent from Hikvison said they has trained the model for all the QP(0-63), and have proved it can be practical to train a model to handle all the QP values, so they do not need to test this subtest.

5. One expert ask if we open the rate control, the QP within CTU or the QP among CTUs are different, how to handle this condition?

It is suggested to research it and submit a contribution. Current CE will not include it as the subtest.

6. Should we care about the training stage?

At the current stage, we can just focus on the process in the inference stage. How to get the network parameter is not our concern, only if the training date is different as the test data (for SubCE2). But it is suggested to provide clear information of the training process.

7. Considering there are many subtests, it is suggested to define the test condition for this CE test. The test condition includes full test and short test, the full test follows the CTC for SDR as defined in 13th JVET meeting (Marrakech, MA).The short test is based on the CTC, but only need to test the first random access segment of the test sequences. More details of the description can be seen in Table 1, and Table 2. The assigned tests are shown in Table 3 and Table 4.

Table 1. The summary of different cases

| # | Description For SubCE1 | Description For SubCE2 |
| --- | --- | --- |
| anchor | DBF + SAO + ALF, no NN filter, use CTC QP to test | DBF + SAO + ALF, no NN filter, use CTC QP to test |
| 1a | DBF + SAO + ALF+ NN filter | DBF + NN filter + SAO + ALF |
| 2a | NN filter. | NN filter. |
| 3a | N/A | NN filter + ALF. |
| 1b | NN filter CTU switch | NN filter CTU switch |
| 2b | N/A | always use NN filter |
| 1c | use CTC QP | use CTC QP |
| 2c | N/A | use CTC QP + 2 to test |
| 3c | N/A | use CTC QP - 2 to test |

Table 2. The description of the test condition

| Purpose | # | CTC test(Full test) |  |  | Test First intra period (Short test) |
| --- | --- | --- | --- | --- | --- |
|  |  | All Intra | Random Access | Low delay | Random Access |
|  | anchor | M (For SubCE2) | M | O | O,S |
|  | 1a+1b+1c | M (For SubCE2) | M | O | O,S |
| Filter position | 2a+1b+1c | N/A | N/A | N/A | O,S |
|  | 3a+1b+1c | N/A | N/A | N/A | O,S |
| CTU switch | 1a+2b+1c | N/A | N/A | N/A | O,S |
| Generalization (QP) | 1a+1b+2c | N/A | N/A | N/A | O,S |
|  | 1a+1b+3c | N/A | N/A | N/A | O,S |

Note: M denotes mandatory test. O denotes the optional test. S denotes the short test of only first intra period. The full test which should follow the CTC. The short test should also follow the CTC, except the frame length, which only need to test the first intra period.

- Pre-CE description

  - Participant list

| # | Name | Affiliation | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1. | Jiabao Yao | Hikvision | yaojiabao@hikvision.com |  |  |
| 2. | Chaoyi Lin | Hikvision | linchaoyi@hikvision.com |  |  |
| 3. | Li Wang | Hikvision | wangli7@hikvision.com |  |  |
| 5. | Hujun Yin | Intel | hujun.yin@intel.com |  |  |
| 6. | Zhiping Deng | Intel | zhipin.deng@intel.com |  |  |
| 7. | Rongzhen Yang | Intel | rongzhen.yang@intel.com |  |  |
| 8. | Kei Kawamura | KDDI | ki-kawamura@kddi.com |  |  |
| 9. | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
| 10 | Yu-Ling Hsiao | MediaTek | Yuling.Hsiao@mediatek.com |  |  |
| 11 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com |  |  |
| 12 | Tzu-Der Chuang | MediaTek | Peter.Chuang@mediatek.com |  |  |
| 13 | Shan Liu | Tencent | shanl@tencent.com |  |  |
| 14 | Yue Li | USTC | lytt@mail.ustc.edu.cn |  |  |
| 15 | Ning Yan | USTC | nyan@mail.ustc.edu.cn |  |  |
| 16 | Yuanying Dai | USTC | daiyy@mail.ustc.edu.cn |  |  |
| 17 | Dong Liu | USTC | dongeliu@ustc.edu.cn |  |  |
| 18 | Yingbin Wang | WHU | wyb@whu.edu.cn |  |  |
| 19 | Yiming Li | WHU | userlym@whu.edu.cn |  |  |
| 20 | Zhenzhong Chen | WHU | zzchen@whu.edu.cn |  |  |
| 21 | Yoshitaka Kidani | KDDI | yo-kidani@kddi.com |  |  |

  - SubCE1: Sequence-adaptive (two-pass coding) test (2)

JVET-M0159 AHG9: Convolutional neural network loop filter [Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0566 Adaptive convolutional neural network loop filter [H. Yin, R. Yang, X. Fang, S. Ma, Y. Yu (Intel)]

Table 3. The description of the CE test for SubCE1

| Test# | Source | Description |
| --- | --- | --- |
| 1.1.1 | JVET-M0159 | case 1a+1b+1c. The network is same as the proposal (mandatory) |
| 1.1.2 | JVET-M0159 | case 2a+1b+1c. Replace all the traditional loop filter (optional, only need short test as Table 2 defined) |
| 1.2.1 | JVET-M0566 | case 1a+1b+1c. The network is same as the proposal (mandatory) |
| 1.2.2 | JVET-M0566 | case 2a+1b+1c. Replace all the traditional loop filter (optional, only need short test as Table 2 defined) |

  - SubCE2: Sequence-independent (one-pass coding) methods (4)

JVET-M0351 AHG9: Convolutional Neural Network Filter (CNNF) for Intra Frame [C. Lin, J. Yao, L. Wang (Hikvision)]

JVET-M0508 AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter [Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent)]

JVET-M0510 AHG9: CNN-based in-loop filter proposed by USTC [Y. Dai, D. Liu, Y. Li, F. Wu (USTC)]

JVET-M0872 AHG9: A Result of Convolutional Neural Network Filter [K. Kawamura, S. Naito (KDDI)]

Table 4. The description of the CE test for SubCE2

| Test# | Source | Description |
| --- | --- | --- |
| 2.1.1 | JVET-M0351 | case 1a+1b+1c. The network is same as the proposal (mandatory) |
| 2.1.2 | JVET-M0351 | case 2a+1b+1c. (optional, only need short test as Table 2 defined) |
| 2.1.3 | JVET-M0351 | case 2a+1b+1c. (optional, only need short test as Table 2 defined) |
| 2.2.1 | JVET-M0508 | case 1a+1b+1c. The network is same as the proposal (mandatory) |
| 2.2.2 | JVET-M0508 | case 2a+1b+1c. (optional, only need short test as Table 2 defined) |
| 2.2.3 | JVET-M0508 | case 3a+1b+1c. (optional, only need short test as Table 2 defined) |
| 2.2.4 | JVET-M0508 | case 1a+1b+2c. (optional, only need short test as Table 2 defined) |
| 2.2.5 | JVET-M0508 | case 1a+1b+3c. (optional, only need short test as Table 2 defined) |
| 2.3.1 | JVET-M0510 | case 1a+1b+1c. The network is same as the proposal (mandatory) |
| 2.3.2 | JVET-M0510 | case 2a+1b+1c. (optional, only need short test as Table 2 defined) |
| 2.3.3 | JVET-M0510 | case 3a+1b+1c. (optional, only need short test as Table 2 defined) |
| 2.3.4 | JVET-M0510 | case 1a+1b+2c. (optional, only need short test as Table 2 defined) |
| 2.3.5 | JVET-M0510 | case 1a+1b+3c. (optional, only need short test as Table 2 defined) |
| 2.4.1 | JVET-M0872 | case 1a+1b+1c. The network is same as the proposal (mandatory) |
| 2.4.2 | JVET-M0872 | case 2a+1b+1c. (optional, only need short test as Table 2 defined) |
| 2.4.3 | JVET-M0872 | case 3a+1b+1c. (optional, only need short test as Table 2 defined) |
| 2.4.4 | JVET-M0872 | case 1a+2b+1c. (optional, only need short test as Table 2 defined) |

  - Test condition

For full sequence test, CTC conditions as agreed at the 13th JVET meeting (Marrakech, MA) for SDR (JVET-L1010) are followed for the evaluation of objective performance. The VTM version 4.0 software is used with ‘vtm’ configuration files to produce the VTM anchor. Tools are implemented into the VTM version 4.0 software. Proposals are compared with respect to objective compression efficiency while complexity is also assessed.

For short test, they should also follow the CTC conditions, except for the frame length. Only the first intra period of the test sequences in RA configuration is necessary.