| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0908 |
| --- | --- |

| Title: | CE11-related: Specification text for combination of JVET-M0103 and JVET-M0294 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Enhorn, R. Yu, Z. Zhang, R. Sjöberg (Ericsson) Biao Wang, Anand Meher Kotra, Semih Esenlik, Han Gao, Jianle Chen (Huawei Technologies Co.,Ltd.) | Tel: Email: | kenneth.r.andersson@ericsson.com biao.wang@huawei.com |
| Source: | Ericsson, Huawei Technologies Co.,Ltd |  |  |

_____________________________

# Abstract

This contribution includes specification change for the combination of JVET-M0103 [1] and JVET-M0294 [2]. For CU/TU boundary it uses a boundary strength of 2 and for sub-block boundary it uses a boundary strength of 1.

# Specification Text

In section 8.4.5 Derivation process for intra prediction mode in combined merge and intra prediction

..

If IntraPredModeY[ xCb ][ yCb ] is equal to INTRA_ANGULAR50, NumSbY[ xCb ][ yCb ] is set equal to 4 and NumSbX[ xCb ][ yCb ]  is set equal to 1,

Otherwise, if IntraPredModeY[ xCb ][ yCb ] is equal to INTRA_ANGULAR18, , NumSbY[ xCb ][ yCb ] is set equal to 1 and NumSbX[ xCb ][ yCb ] is set equal to 4.

In section 8.6.2.5  Derivation process of boundary filtering strength

..

Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 has ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

Otherwise, if the sample p0 or q0 has ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 1.

..

Please find the detailed specification modification based on working draft [3] in the attached file JVET-M0908-Spec.doc.

# Reference

- K. Andersson, J. Enhorn, R. Yu, Z. Zhang, R. Sjöberg, “Deblocking for multi-hypothesis intra inter prediction” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-M0103,

- Biao Wang, Anand Meher Kotra, Semih Esenlik, Han Gao, Jianle Chen, “CE10-related: Modification for blocks applied with Combined Inter-Intra prediction” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-M0294,

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-L1001

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).