| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1021_v4 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1): Post-prediction and post-reconstruction filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output document from JVET |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Jacob Ström Sergey Ikonin Vadim Seregin | Tel: Email: | +46 702672192 jacob.strom@ericsson.com, sergey.ikonin@huawei.com, vseregin@qti.qualcomm.com. |
| Source: | CE1 coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to study technologies of post-prediction/reconstruction filtering methods. The techniques are evaluated according to BD-rate gain, complexity (in both encoder and decoder). Participants in this activity are Ericsson, Huawei, HHI, Qualcomm.

The software basis for the mandatory test in this CE is VTM-4.0, Test sequences, configurations and test conditions are according to JVET-M1021 for SDR.

# Test conditions and evaluation criteria

VTM-4.0 will be used as anchor in this CE. Proposals will primarily be compared with respect to objective quality, but complexity will also be measured according to the subtest requirements.

# Tool descriptions

## 2.1 Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 1.1 | Reduced complexity bilateral filter | JVET-M0885 |
| 1.2 | Hadamard transform domain filter | JVET-M0468 |
| 1.3 | Test on parametrizable bilateral filter from JVET-L0406 in VTM3.0 | JVET-M0894 |
| 1.4 | Uniform Luma Inter Prediction Filter | JVET-M0042 |
| 1.5 | Unidirectional illumination compensation | JVET-M0500 |
| 1.6 | LIC restriction for pipeline structure | JVET-M0088 |
| 1.7 | Simplified bi-directional illumination compensation without BPWA | JVET-M0115 |
| 1.8 | Simplification of local illumination compensation | JVET-M0182 |
| 1.9 | Intra Reference Sample Deblocking | JVET-M0138 |

## 2.2  Detailed description of tests

In this description, will use the term processing to refer to either filtering (1.1-1.4) or illumination compensation.

For all tests, the size of the smallest processed block will be 8×8 samples.

Processing should not be applied on intra block, nor should it be applied to combined intra/inter blocks (CIIB blocks) or intra block copy (IBC) / current picture reference (CPR) blocks.

For all tests, the following three constraint conditions may be tested:

- No restriction on using samples to the left and above the block (not mandatory?)

- Samples to the left and above can be used, but only if they come from an inter block (i.e., not from an intra block or a CIIB block or an ICB/CPR block). (not mandatory?)

- No samples outside the block can be used. (mandatory)

CE 1.1 Reduced complexity bilateral filter (JVET-M0885)

This sub-CE will investigate the complexity reductions obtained by using shifted factors in the multiplications, as exemplified in JVET-M0885.

1.1.1a Same as JVET-M0885 implemented in VTM-4.0, (a, no restriction)

1.1.1c Same as JVET-M0885 implemented in VTM-4.0, (c, no samples outside the block)

In JVET-M0885, the first multiplication was reduced to a two-bit multiplier. However, it is not clear that this is the best trade-off between BD-rate gain and multiplier complexity. Therefore the following test will be carried out:

1.1.2a Same as JVET-M0885 implemented in VTM-4.0, but where the multipliers have been shifted to provide a better trade-off between complexity and BD-rate , (a, no restriction)

1.1.2b Same as 1.1.2a, but with restriction b (samples can be used from inter blocks)

1.1.2c Same as 1.1.2a, but with restriction c (no samples outside the blocks)

Finally, it will also investigate the loss associated with removing the latency introduced by the non-local filtering step, since this was identified at the Macau meeting as needing an extra cycle of latency. Therefore the following tests will be carried out:

1.1.3b Same as 1.1.2b, but without non-local filtering (b, samples can be used from inter blocks)

1.1.3c Same as 1.1.3b, but with restriction c (no samples outside the blocks)

| # | Proposal | Crosschecker | Config | Description |
| --- | --- | --- | --- | --- |
| CE1.1.1a | JVET-xxxx | J. Rasch | RA, LD | See above |
| CE1.1.1c | JVET-xxxx | J. Rasch | RA, LD | See above |
| CE1.1.2a | JVET-xxxx | J. Rasch | RA, LD | See above |
| CE1.1.2b | JVET-xxxx | J. Rasch | RA, LD | See above |
| CE1.1.2c | JVET-xxxx | J. Rasch | RA, LD | See above |
| CE1.1.3b | JVET-xxxx | J. Rasch | RA, LD | See above |
| CE1.1.3c | JVET-xxxx | J. Rasch | RA, LD | See above |

CE 1.2 Hadamard transform domain filter (JVET-M0468)

This sub-CE will …

CE 1.3 Test on parametrizable bilateral filter from JVET-L0406 in VTM3.0 (JVET-M0894)

This sub-CE will test performance of the proposed P-BIF under CE-specified constrains.

| # | Proposal | Crosschecker | Config | Description |
| --- | --- | --- | --- | --- |
| CE1.3.1 | JVET-xxxx |  | RA, LD | P-BIF with restrictions on samples access |
| CE1.3.2 | JVET-xxxx |  | RA, LD | P-BIF with signaling adapting applicability |
| CE1.3.2 | JVET-xxxx |  | RA, LD | P-BIF adapting applicability to block type and sizes |

CE 1.4 Uniform Luma Inter Prediction Filter (JVET-M0042)

The luma inter prediction filter uses different uniform filter modes to improve the inter prediction signal. The following restrictions shall be tested on all tests:

- use reconstructed samples (r1)

- use only inter reconstructed samples (r2)

- use no reconstructed samples (r3)

| # | Proposal | Crosschecker | Config | Description |
| --- | --- | --- | --- | --- |
| CE1.4.1 | JVET-xxxx | J. Ström | RA, LD | - Adapt filter to block size - r1 |
| CE1.4.2 | JVET-xxxx | J. Ström | RA, LD | - Adapt filter to block size - r2 |
| CE1.4.3 | JVET-xxxx | J. Ström | RA, LD | - Adapt filter to block size - r3 |
| CE1.4.4 | JVET-xxxx | J. Ström | RA, LD | - Adapt encoder search - r1 |
| CE1.4.5 | JVET-xxxx | J. Ström | RA, LD | - Adapt encoder search - r2 |
| CE1.4.6 | JVET-xxxx | J. Ström | RA, LD | - Adapt encoder search - R3 |
| CE1.4.7 | JVET-xxxx | J. Ström | RA, LD | - Adapt filter application to certain inter modes - r1 |
| CE1.4.8 | JVET-xxxx | J. Ström | RA, LD | - Adapt filter application to certain inter modes - r2 |
| CE1.4.9 | JVET-xxxx | J. Ström | RA, LD | - Adapt filter application to certain inter modes - r3 |
| CE1.4.10 | JVET-xxxx | J. Ström | RA, LD | - Adapt boundary condition - r1 |

CE 1.5 Unidirectional illumination compensation (JVET-M0500)

Base LIC version as described in the meeting notes:

- Usage of reconstructed intra coded, CIIP and IBC blocks, as this would have impact on complexity and parallel implementation pipelining (test with and without, and further study the impact)

- Disabling LIC over CTU boundaries is not necessary

- Disable for CIIP blocks

- No temporal inheritance of LIC flag

- No pruning based on LIC flag in merging candidate list generation

- Disable LIC for all subblock modes, including triangle mode and BDOF

- It is not applied to 128xN and Nx128 blocks

- It is not applied to bi-prediction

When LIC is not applied, LIC flag is not signaled.

There are 2 variants of base version: in one all reconstructed samples except IBC are used, in the second version samples of intra, CIIP and IBC coded blocks are excluded.

Below tests will be performed for each base version.

A combination of the several CE tests will be performed for better coding efficiency.

| Test | Tester | Cross-checker |
| --- | --- | --- |
| Test 1. Base LIC version |  |  |
| Test 2 Test 1, where LIC is not applied for 4x4 blocks |  |  |
| Test 3 Test 1, where LIC parameters are derived for the first VPDU and are used for all other VPDUs for 128xN and Nx128 blocks | Kiyofumi Abe abe.kiyo@jp.panasonic.com | Saurav Bandyopadhyay Saurav.Bandyopadhyay @InterDigital.com |
| Test 4. Test 1, where LIC is applied after bi-prediction is derived. | Saurav Bandyopadhyay Saurav.Bandyopadhyay @InterDigital.com | Kiyofumi Abe abe.kiyo@jp.panasonic.com |
| Test 5. Test 4, but LIC is not applied with BPWA | Chia-Ming Tsai chia-ming.tsai@mediatek.com |  |
| Test 6. Test 1 with modified bS derivation of DBF using lic_flag | Kiyofumi Abe abe.kiyo@jp.panasonic.com | Saurav Bandyopadhyay Saurav.Bandyopadhyay @InterDigital.com |

CE 1.6 LIC restriction for pipeline structure (JVET-M0088)

The proposed LIC is based on JVET-M0087 with some restrictions to solve hardware pipeline issues. It is proposed to use only top and left neighboring samples of first VPDU of both current block and reference block, and share the calculated LIC parameter with other VPDUs in the current CU. And, it is proposed to change bS equal to 1 in DBF when at least one of the blocks uses LIC. Finally, we will shorten the encoding time pointed out at the meeting.

This sub-CE will experiment following conditions on top of CE1.5.

- modify bS derivation of DBF using lic_flag

- calculate LIC parameter using only 64x64 top-left brock and share it

- a + b + encoder speed up

CE 1.7 Simplified bi-directional illumination compensation without BPWA (JVET-M0224)

This sub-CE will investigate LIC in bi-prediction without BPWA. It will be based on test 4 (JVET-M0224) and JVET-M0115.

CE 1.8 Simplification of local illumination compensation (JVET-M0182)

In JVET-M0182, to simplify the local illumination compensation (LIC) in BMS2.2, an encoder side fast algorithm is proposed by reusing the LIC-off motion search results for testing LIC-on. And, the bi-prediction with weighted averaging and subblock-based temporal motion vector prediction (SbTMVP) are disallowed when LIC is on. Within this CE test, the performance of the method described in JVET-M0182 is to be studied and also consider the following constraints:

1.	Disallow the usage of neighboring reconstructed samples from intra coded, CIIP, and IBC blocks.

2.	Disable LIC for CIIP blocks, BIO mode, and all subblock modes, such as Affine, SbTMVP.

3.	No temporal inheritance of LIC flag

4.	No pruning based on LIC flag in merging candidate list generation

5.	LIC is applied to the bi-prediction template (or final prediction template)

6.	LIC should follow the VPDU constraints

CE 1.9 Intra Reference Sample Deblocking (JVET-M0138)

The proposed …

CE 1.10 New SubCE (JVET-M0xxx)

The proposed …

## Complexity analysis

For all sub-CEs, the following complexity figures should be reported.

| Test# | filter shape | Computational complexity (# of multiplications/ additions/shifts/checks*) per sample | Precision of multiply | Parallel friendly (each sample can be filtered independently from other samples) | Number of clock cycles | memory requirement | How to derive filter coeffs | Minimal CU Size for applying filters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 |  |  |  |  |  |  |  |  |
| 1.2 |  |  |  |  |  |  |  |  |
| 1.3 |  |  |  |  |  |  |  |  |
| 1.4 |  |  |  |  |  |  |  |  |
| 1.5 |  |  |  |  |  |  |  |  |
| 1.6 |  |  |  |  |  |  |  |  |
| 1.7 |  |  |  |  |  |  |  |  |
| 1.8 |  |  |  |  |  |  |  |  |
| 1.9 |  |  |  |  |  |  |  |  |

* max/min/abs are countered as checks

## Tester and cross-checkers

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE1.1 | Jacob Ström jacob.strom@ericsson.com JVET-M0885 | Jennifer Rasch Jennifer.Rasch@hhi.fraunhofer.de |
| CE1.2 | Sergey Ikonin sergey.ikonin@huawei.com |  |
| CE1.3 | Dmytro Rusanovskyy dmytror@qti.qualcomm.com JVET-M0894 |  |
| CE1.4 | Jennifer Rasch Jennifer.Rasch@hhi.fraunhofer.de JVET-M0042 |  |
| CE1.5 | Vadim Seregin vseregin@qti.qualcomm.com JVET-M500 |  |
| CE1.6 | Kiyofumi Abe abe.kiyo@jp.panasonic.com JVET-M0088 |  |
| CE1.7 | Bordes Philippe philippe.bordes@technicolor.com JVET-M0115 |  |
| CE1.8 | Chia-Ming Tsai Chia-ming.tsai@mediatek.com JVET-M0182 |  |
| CE1.9 | Zhang Zhi zhi.a.zhang@ericsson.com JVET-M0138 |  |

# Interdependency with other CEs

No known interdependencies are known.

# Test conditions and data to be provided

The common test conditions are used to evaluate the objective coding gains of different methods.

# Time-line and Responsibilities

2019-Feb-1:	Final CE description uploaded

2018-X:		(two weeks after the new VTM software available): Cross-checking begins, proponents provide software

2018-X+14:		Full test results must be provided at this time (at least for proposals targeting to be promoted to VTM at the next meeting)

# Appendix of Participants

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Jacob Ström | Ericsson | jacob.strom@ericsson.com | X | X |
| 2 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com | X |  |
| 3 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com | X |  |
| 4 | Jennifer Rasch | HHI | Jennifer.Rasch@hhi.fraunhofer.de | X |  |
| 5 | Vadim Seregin | Qualcomm | vseregin@qti.qualcomm.com | X |  |
| 6 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com | X |  |
| 7 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com | X |  |
| 8 | Tzu-Der Chuang | MediaTek | peter.chuang@mediatek.com | X |  |
| 9 | Chia-Ming Tsai | MediaTek | chia-ming.tsai@mediatek.com | X |  |
| 10 | Zhi-Yi Lin | MediaTek | zhi-yi.lin@mediatek.com | X |  |
| 11 | Man-Shu Chiang | MediaTek | man-shu.chiang@mediatek.com | X |  |
| 12 | Chun-Chia Chen | MediaTek | chunchia.chen@mediatek.com | X |  |
| 13 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com | X |  |
| 14 | Olena Chubach | MediaTek | Olena.Chubach@mediatek.com | X |  |
| 15 | Philippe Bordes | Technicolor | philippe.bordes@technicolor.com | X |  |
| 16 | Nan Hu | Qualcomm | nanh@qti.qualcomm.com | X |  |
| 17 | Masa Ikeda | Sony | Masaru.Ikeda@sony.com |  |  |
| 18 | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |  |  |
| 19 | Eiichi Sasaki | Sharp | eiichi.sasaki@sharp.co.jp |  |  |
| 20 | Takeshi Chujoh | Sharp | chujoh.takeshi@sharp.co.jp |  |  |
| 21 | Yukinobu Yasugi | Sharp | yasugi.yukinobu@sharp.co.jp |  |  |
| 22 | Zizheng Liu | Wuhan University | lzz8246@whu.edu.cn |  |  |
| 23 | Jungwon Kang | ETRI | jungwon@etri.re.kr |  |  |
| 24 | Sung-Chang Lim | ETRI | sclim@etri.re.kr |  |  |
| 25 | Hongbin Liu | Bytedance | liuhongbin.01@bytedance.com |  |  |
| 26 | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |  |  |
| 27 | Wade Wan | Broadcom | wade.wan@broadcom.com |  |  |
| 28 | Peisong Chen | Broadcom | peisong.chen@broadcom.com |  |  |
| 29 | Jie Chen | Alibaba | jiechen.cj@alibaba-inc.com |  |  |
| 30 | Yan Ye | Alibaba | yan.ye@alibaba-inc.com |  |  |
| 31 | Kiyofumi Abe | Panasonic | abe.kiyo@jp.panasonic.com |  |  |
| 32 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |  |  |
| 33 | Jingya Li | Panasonic | jingya.li@sg.panasonic.com |  |  |
| 34 | Franck Galpin | Technicolor | Franck.Galpin@technicolor.com |  |  |
| 35 | Saurav Bandyopadhyay | Interdigital | saurav.bandyopadhyay@interdigital.com |  |  |
| 36 | Yuwen He | Interdigital | yuwen.he@interdigital.com |  |  |
| 37 | Daniel Luo | Interdigital | Daniel.Luo@InterDigital.com |  |  |
| 38 | Jonathan Pfaff | HHI | jonathan.pfaff@hhi.fraunhofer.de |  |  |
| 39 | Anastasia Henkel | HHI | anastasia.henkel@hhi-extern.fraunhofer.de |  |  |
| 40 | Andre Seixasdias | BBC | andre.seixasdias@bbc.co.uk |  |  |
| 41 | Saverio Blasi | BBC | Saverio.Blasi@bbc.co.uk |  |  |

# References

[1] F.Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

[2] J. Ström, ”Non-CE: Reduced complexity bilateral filtering”, JVET-M0885, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[3] S. Ikonin, V. Stepin, J. Chen, “Non-CE: Hadamard transform domain filter”, JVET-M0468, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[4] M. Karczewicz, “Non-CE: Test on parametrizable bilateral filter from JVET-L0406 in VTM3.0”, JVET-M0894, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[5] J. Rasch, A. Henkel, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand. ”CE10: Uniform Directional Diffusion Filters For Video Coding”, JVET-M0042, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[6] V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, “CE10-related: Unidirectional illumination compensation”, JVET-M0500, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[7] K.Abe, T. Toma, J. Li, C.-W. Kuo, V. Drugeon, “CE10-related: LIC restriction for pipeline structure”, JVET-M0088, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[8] C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10-related: Simplification of local illumination compensation”, JVET-M0182, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019