| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1025-v2 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5): Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Vadim Seregin | Tel Email: | chingyeh.chen@mediatek.com vseregin@qti.qualcomm.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 5 (CE5) on Adaptive Loop Filter. The experiments are conducted after the 13th JVET meeting and the results, including cross-checks, are reported as input contributions to the 14th JVET meeting.

# Introduction

The following tests will be performed:

| Test# | Description | Document# |
| --- | --- | --- |
| CE5-1.1 | Virtual boundary processing with padding samples/gradients only | JVET-M0164 |
| CE5-1.2 | Virtual boundary processing with padding samples/gradients and weighted averaging | JVET-M0164 |
| CE5-1.3 | Virtual boundary processing with padding samples/gradients and weighted averaging and adaptive on/off control | JVET-M0164 |
| CE5-2.1 | Virtual boundary processing with padding samples and truncated filtering without adaptive on/off control | JVET-M0301 |
| CE5-2.2 | Virtual boundary processing with padding samples and truncated filtering with adaptive on/off control | JVET-M0301 |
| CE5-2.3 | Virtual boundary processing with padding samples and weighted averaging and adaptive on/off control | JVET-M0301 |
| CE5-3.1 | Non-linear ALF (NL-ALF) as in JVET-M0385. | JVET-M0385 |
| CE5-3.2 | Test CE5-3.1 + additional NL-ALF signaling + encoder optimization. | JVET-M0385 |
| CE5-3.3 | TEST CE5-3.1 + reduced number of clipping operations for NL-ALF. | JVET-M0385 |
| CE5-3.4 | TEST CE5-3.1 + reduced number of overall operations and line buffer for NL-ALF. | JVET-M0385 |
| CE5-4.1 | VTM with temporal filters mimicking APS |  |
| CE5-4.2 | Luma CTBs with ALF on can switch among 1 slice-level signaled filter set, 16 fixed filter sets and those available sets in APS. All chroma CTBs with ALF on in a picture can only use the slice-level signaled filter or one temporal filter | JVET-M0429 |
| CE5-4.3 | Test CE5-4.2 with QP = 12, 17, 22 and 27 | JVET-M0429 |
| CE5-4.4 | Test CE5-4.2 without fixed filters | JVET-M0429 |
| CE5-4.5 | Test CE5-4.2 without temporal filters | JVET-M0429 |
| CE5-4.6 | Test CE5-4.5 with QP = 12, 17, 22 and 27 | JVET-M0429 |

Tests CE5-1 and CE5-2 include subjective evaluation. Informal subjective viewing is planned to be conducted during the Geneva meeting for both the proposals. The informal subjective test mainly checks for any visual artifcats which are created or observed near the CTU boundary when compared to the anchor VTM-4.0. ABAB test comparison is planned to be used where A or B could randomly be the anchor or the test.

The following sequences and QP points are planned to be used:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 02 | Johnny | 1280x720p | 60fps | 600 | LDP | 37, 32 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 37, 32 |
| 06 | Cactus | 1920x1080 | 50fps | 500 | LDP | 37, 32 |

# Detailed description of tests

#### CE5.1 Adaptive loop filter with virtual boundary processing (JVET-M0164)

In JVET-M0164, ALF with virtual boundary (VB) processing is proposed to remove the line buffer usage. When one sample located at one side of a VB is filtered, accessing samples located at the other side of the VB is forbidden. The originally required samples at the other side of the VB are replaced with padded samples. To accommodate deblocking filter (DF) and sample adaptive offset (SAO) in VTM3.0, the VBs are set as four luma lines and two chroma lines above CTU row boundaries.

When a sample at one side of the VB is filtered, samples at the other side of the VB are replaced with padded samples during gradient calculation and filtering process. Figure 5 is an example for gradient calculation. The gradients are used in block-based classification and calculated by using four 3-tap gradient filters. In Figure 5, the center sample is the to-be-gradient-filtered sample, and a VB is right below the to-be-gradient-filtered sample. Each of the three samples below the VB is replaced with its nearest sample above the VB during gradient calculation.

Figure 5.  Modification for gradient calculations in VB processing.

The padding technique of accumulating gradients for a 4x4 block is different. As shown in Figure 6, the 4x4 green block needs to accumulate subsampled gradients in the 8x8 region, and the VB is right below the 4x4 green block. Instead of using padded samples to generate gradients below the VB, the gradients below the VB are replaced with the nearest gradient on the same column above the VB.

Figure 6.  Modification of accumulating gradients for a 4x4 block in VB processing.

A weighted averaging process is added after the filtering process. A final ALF output sample is a weighted averaging of the filtered sample and the ALF input sample. The weight for the filtered sample is dependent on the number of padded lines in the filtering process, and the sum of the two weights is 1.0. If there is no padded lines in the filter footprint, the weight of the filtered sample is equal to 1.0. If there is one padded line in the filter footprint, the weight of filtered sample is 0.5. If there are two padded lines in the filter footprint, the weight of filtered sample is 0.25. If there are three padded lines in the filter footprint, the weight of filtered sample is 0.125.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-1.1 | Virtual boundary processing with padding samples/gradients only | Jenny Lai (MediaTek) | Nan Hu (Qualcomm) |
| CE5-1.1 | Virtual boundary processing with padding samples/gradients and weighted averaging | Jenny Lai (MediaTek) | Anand Meher Kotra (Huawei) |
| CE5-1.2 | Virtual boundary processing with padding samples/gradients and weighted averaging and adaptive on/off control (combined with CE5-2.2) | Jenny Lai (MediaTek) | Anand Meher Kotra (Huawei) |

#### CE5.2 Non-CE: Loop filter line buffer reduction (JVET-M0301)

In JVET-M0301, ALF with virtual boundary processing is also used. Modified ALF classification only uses the samples which are above the VB to classify the given 4 x 4 block which is above VB. Similarly, for the classification of the 4 x 4 block below VB, samples belonging to the lines below the VB are used. Modified ALF filtering uses a combination of conditional disabling and truncated versions of the original ALF filter.

Modified ALF block classification is depicted in the figure below:

Modified ALF filtering is shown below:

Conditional disabling of the ALF filtering is as follows:

In Figure 4, if absolute sum of the padded coefficients is greater than a threshold (T) times the absolute sum of the non-padded coefficients, then the filter is disabled for those respective lines.

For e.g. with lines M and H, if (2*abs  c0>(T* 2*abs c1+abs c2+absc3+absc4+absc5+absc6+absc7+absc8+absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines M and H.

Similarly for lines L and I, if (2*(abs c0+ abs c1 + abs c2+ abs c3)>(T* 2*absc4+absc5+absc6+absc7+absc8+absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines L and I.

Similarly for lines J and K, if (2*(abs c0+ abs c1 + abs c2+ abs c3+absc4+absc5+absc6+absc7+absc8)>(T* 2*absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines J and K.

Where T is generally a threshold which can take a fractional value ranging between 0 and 1. For example, T value can be 0.5, 0.6 or 0.7

Similarly for Chroma, as depicted in Fig. 5, for lines N and K, if (2*abs  c0>(T* 2*abs c1+abs c2+absc3+absc4+absc5)+absc6), then the ALF filtering is disabled for lines N and K.

for lines M and L, if ((2*(abs c0+ abs c1 + abs c2+ abs c3)>(T* 2*absc4+absc5)+absc6), then the ALF filtering is disabled for lines M and L.

the function abs(x) is defined as follows: Abs( x ) =  x;x  >=  0-x;x < 0

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-2.1 | Virtual boundary processing with padding samples and truncated filtering without adaptive on/off control | Huawei | Jenny Lai (MediaTek) |
| CE5-2.2 | Virtual boundary processing with padding samples and truncated filtering with adaptive on/off control | Huawei | Jenny Lai (MediaTek) |
| CE5-2.3 | Virtual boundary processing with padding samples and weighted averaging and adaptive on/off control (combined with CE5-1.1) | Huawei | Jenny Lai (MediaTek) |

#### CE5.3 Non-linear Adaptive Loop Filter (JVET-M0385)

In JVET-M0385, an adaptive clipping operation is performed on the input samples values of the Adaptive Loop Filter in VTM3.0. The goal of this adaptive clipping is to introduce some non-linearities to cap the difference between the central input sample value and the other neighbor input sample values used during the ALF filtering process.

In VTM, the ALF filtering can be expressed as follows:

On=wc.Inc+i=0i<cwi.Ini-+Ini+.

On is the output sample of the filter at a position n. Inc is the filter input sample at the same position n as the output sample (and is the centre input sample in the filter pattern). Ini- is the i-th (in raster scan order) neighbouring input sample of Inc in the filter pattern. Ini+ is the neighbouring input sample at a mirrored spatial position of Ini- about the centre position of Inc. wi is the ALF filter coefficient associated with the neighbouring input samples Ini- and Ini+. wc is the ALF filter coefficient for the centre input sample Inc . And c is the number of encoded filter coefficients. The values of i correspond to the index values and the associated positions of Ini- and Ini+ correspond to the index values with their superscript sign (“+” or “-“) in the filter patterns of Figure 1: for Luma filters, c=12 and for Chroma filters, c=6.

Figure 1 – ALF filter patterns for Luma (left) and for Chroma (right).

Using same notations, NL-ALF filtering in JVET-M0385 can be expressed as follow:

On=Inc+i=0i<cwi.KIni--Inc,ki+KIni+-Inc,ki.

where ki is a filter clipping parameter associated with the filter coefficient wi, and where Kd,b=minb,max-b, d is the clipping function. The filter clipping parameters ki for i in 0; c-1 are provided/encoded for each filter.

With ALF in VTM, the total number of operations per filtered luma sample is 13 multiplications and 13 additions per filtered luma sample; and the total number of operations per filtered chroma sample it 7 multiplications, 7 additions. We may also add one addition and one bitshift for fixed point computationnal rounding, plus one clipping of the final result.

Using JVET-M0385 NL-ALF, the total number of operations per filtered luma sample can be: 12 multiplications, 13 additions, 24 subtractions, 24 clippings; and the total number of operations per filtered chroma sample can be 6 multiplications, 7 additions, 12 subtractions, 12 clippings. We may also add one addition and one bitshift for fixed point computationnal rounding, plus one clipping of the final result.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-3.1 | Non-linear ALF (NL-ALF) as in JVET-M0385. | Canon | Nan Hu (Qualcomm) |
| CE5-3.2 | Test CE5-3.1 + additional NL-ALF signaling + encoder optimization. | Canon | Nan Hu (Qualcomm) |
| CE5-3.3 | TEST CE5-3.1 + reduced number of clipping operations for NL-ALF. | Canon | Nan Hu (Qualcomm) |
| CE5-3.4 | TEST CE5-3.1 + reduced number of overall operations and line buffer for NL-ALF. | Canon | Nan Hu (Qualcomm) |

The performed tests correspond to various complexity versus coding gains tradeoffs that coud be made for using non-linear adaptive loop filter.

Test CE5-3.1 corresponds to the non-linear ALF as proposed in JVET-M0385 with SIMD implementation in VTM4.0.

Test CE5-3.2 uses the non-linear ALF of test CE5-3.1 in combination with a new encoder optimization algorithm. This provides a better rate distorsion compromise compared to the test CE5-3.1. It slightly increases the encoder complexity, but it does not change the decoder complexity: the number of ALF filtering operations remains exactly the same as the test CE5-3.1. This test offers a different compromise between encoder complexity and coding efficiency compared to test CE5-3.1

Test CE5-3.3 is based on the test CE5-3.1 where the clipping operations performed on the input samples are significantly reduced. For the 7x7 luma filter, the number of clipping operations is reduced from 24 clippings to 8 clippings. For the 5x5 Chroma filter, the clipping operations are also reduced, and the clipping operations are only applied on the 8 outer samples. This enables to reduce the number of clippings from 12 clippings to 8 clippings for the chroma filter. This test offers a different compromise between decoding complexity and coding efficiency compared to test CE5-3.1

Test CE5-3.4 is based on the test CE5.3.1 where the clipping operations performed on the input samples are also significantly reduced. Instead of using a 7x7 non-linear luma filter, a 5x5 non-linear luma filter is used. The number of clipping operations is thus reduced from 12 clippings to 8 clippings. Reducing the size of the filter from 7x7 to 5x5 also reduces the number of multiplications required by the NL-ALF filtering from 12 to 6 and allows reducing the number of samples line buffers. This test offers another compromise between decoding complexity and coding efficiency compared to test CE5-3.1

#### CE5.4 Coding tree block based adaptive loop filter (JVET-M0429)

In JVET-M0429, each luma CTB with ALF on can select one out of 22 filter sets (16 fixed, 5 temporal and 1 picture optimized). All chroma CTBs with ALF on in a picture can only use 1 picture optimized filter or 1 temporal chroma filter. The signaling mechanism will mimic adopted APS method (the APSs can carry up to 32 filter sets) for signaling ALF parameters in tile group header.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-4.1 | VTM with temporal filters mimicking APS | Nan Hu (Qualcomm) | Jonathan Taquet (Canon) |
| CE5-4.2 | Luma CTBs with ALF on can switch among 1 slice-level signaled filter set, 16 fixed filter sets and those available sets in APS. All chroma CTBs with ALF on in a picture can only use the slice-level signaled filter or one temporal filter | Nan Hu (Qualcomm) | Jonathan Taquet (Canon) |
| CE5-4.3 | Test CE5-4.2 with QP = 12, 17, 22 and 27 | Nan Hu (Qualcomm) |  |
| CE5-4.4 | Test CE5-4.2 without fixed filters | Nan Hu (Qualcomm) | Jonathan Taquet (Canon) |
| CE5-4.5 | Test CE5-4.2 without temporal filters | Nan Hu (Qualcomm) | Jonathan Taquet (Canon) |
| CE5-4.6 | Test CE5-4.5 with QP = 12, 17, 22 and 27 | Nan Hu (Qualcomm) |  |

# Test conditions

CTC conditions as agreed at the 13th JVET meeting (Marrakesh, Morocco) for SDR will be used for the evaluation of objective performance (JVET-M1010). The software basis and anchor are VTM-4. Proposals will be compared with respect to objective compression efficiency. And subjective quality check is required for those tests related to line buffer reduction.

# Timeline

| T1 | End of 13th JVET meeting + 3 weeks: Feb. 08, 2019 | Final CE description uploaded |
| --- | --- | --- |
| T2 | VTM4.0 release + 2 weeks or 4 March, whichever is earlier | Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector. • Initial study by cross-checkers can begin. • Proponents may continue to modify the software in this branch until T3 • 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes |
| T3 | 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later | Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. |

# Appendix of Participants

| # | Name | Company | E-mail |  | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |  |  |  |
| 2 | Wade Wan | Broadcom | wade.wan@broadcom.com |  |  |  |
| 3 | Peisong Chen | Broadcom | peisong.chen@broadcom.com |  |  |  |
| 4 | Li Zhang | Bytedance | lizhang.idm@bytedance.com |  |  |  |
| 5 | Jonathan TAQUET | Canon | Jonathan.TAQUET@crf.canon.fr |  | X |  |
| 6 | Patrice Onno | Canon | patrice.onno@crf.canon.fr |  | X |  |
| 7 | Christophe Gisquet | Canon | christophe.gisquet@gmail.com |  |  |  |
| 8 | Jungwon Kang | ETRI | jungwon@etri.re.kr |  |  |  |
| 9 | Sung-Chang Lim | ETRI | sclim@etri.re.kr |  |  |  |
| 10 | Anand Meher Kotra | Huawei | anand.meher.kotra@huawei.com |  | X |  |
| 11 | Alexey Filippov | Huawei | alexey.filippov@huawei.com |  | X |  |
| 12 | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com |  | X |  |
| 13 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |  | X |  |
| 14 | Victor Stepin | Huawei | stepin.victor@huawei.com |  | X |  |
| 15 | Roman Chernyak | Huawei | chernyak.roman@huawei.com |  | X |  |
| 16 | Semih Esenlik | Huawei | semih.esenlik@huawei.com |  | X |  |
| 17 | Xianglin Wang | Kwai | xianglinwang@kwai.com |  |  |  |
| 18 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |  |  |  |
| 19 | Xiao-Yu Xiu | Kwai | xiaoyuxiu@kwai.com |  |  |  |
| 20 | Tsung-Chuan Ma | Kwai | tsung-chuanma@kwai.com |  |  |  |
| 21 | Hyeongmun Jang | LGE | hm.jang@lge.com |  |  |  |
| 22 | Junghak Nam | LGE | junghak.nam@lge.com |  |  |  |
| 23 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  | X |  |
| 24 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com |  | X |  |
| 25 | Tzu-Der Chuang | MediaTek | peter.chuang@mediatek.com |  | X |  |
| 26 | Chen-Yen Lai | MediaTek | jenny.lai@mediatek.com |  | X |  |
| 27 | Vadim Seregin | Qualcomm | vseregin@qti.qualcomm.com |  | X |  |
| 28 | Nan Hu | Qualcomm | nanh@qti.qualcomm.com |  | X |  |
| 29 | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com |  | X |  |
| 30 | Marta Karczewicz | Qualcomm | martak@qti.qualcomm.com |  | X |  |
| 31 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com |  | X |  |
| 32 | Hilmi Enes Egilmez | Qualcomm | hegilmez@qti.qualcomm.com |  | X |  |
| 33 | Segall, Andrew | Sharp | asegall@sharplabs.com |  |  |  |
| 34 | Masa Ikeda | Sony | Masaru.Ikeda@sony.com |  |  |  |
| 35 | Galpin Franck | Technicolor | Franck.Galpin@technicolor.com |  |  |  |
| 36 | Eric Chai | ublnx | eric.chai@ublnx.com |  |  |  |
| 37 | Weimin Zeng | ublnx | weimin.zeng@ublnx.com |  |  |  |
| 38 | Peng Yin | Dolby | pyin@dolby.com |  |  |  |
| 39 |  |  |  |  |  |  |
| 40 |  |  |  |  |  |
| 41 |  |  |  |  |  |
| 42 |  |  |  |  |  |
| 43 |  |  |  |  |  |
| 44 |  |  |  |  |  |
| 45 |  |  |  |  |  |
| 46 |  |  |  |  |  |
| 47 |  |  |  |  |  |
| 48 |  |  |  |  |  |
| 49 |  |  |  |  |  |
| 50 |  |  |  |  |  |
| 51 |  |  |  |  |  |