| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1032-v1 |
| --- | --- |

| Title: | Description of Core Experiment 12 (CE12): Tile Set Boundary Handling |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Hendry Robert Skupin | Tel: Email: | fnu.hendry@huawei.com robert.skupin@hhi.fraunhofer.de |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to test impact of changing the algorithm for treating boundaries of tile set to the compression performance of VVC. Techniques included in this CE are:

- Treating all tile set boundaries as picture boundaries which includes padding to the tile set boundaries and allowing motion vectors to point to samples outside the boundaries in the reference pictures.

- Encoder-side restrictions of tile set to not use samples or motion vector candidates from outside the tile set

Participants in this activity are ETRI, HHI, Huawei, Intel, KDDI, Kwai, MediaTek, Nokia, Panasonic, Qualcomm, Samsung, Sharp, and Sony.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
| 1 | Gun Bang | ETRI | gbang@etri.re.kr |
| 2 | Jungwon Kang | ETRI | jungwon@etri.re.kr |
| 3 | Hahyun Lee | ETRI | hanilee@etri.re.kr |
| 4 | Robert Skupin | HHI | robert.skupin@hhi.fraunhofer.de |
| 5 | Roman Chernyak | Huawei | chernyak.roman@huawei.com |
| 6 | Hendry | Huawei | fnu.hendry@huawei.com |
| 7 | Seungwook Hong | Huawei | seungwook.hong@huawei.com |
| 8 | Jill Boyce | Intel | jill.boyce@intel.com |
| 9 | Kei Kawamura | KDDI | ki-kawamura@kddi.com |
| 10 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |
| 11 | Tsung-Chuan Ma | Kwai | tsung-chuanma@kwai.com |
| 12 | Shawn Wang | Kwai | xianglinwang@kuaishou.com |
| 13 | Xiaoyu Xiu | Kwai | xiaoyuxiu@kwai.com |
| 14 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com |
| 15 | Tzu-Der Chuang | MediaTek | peter.chuang@mediatek.com |
| 16 | Yu-Wen Huang | MediaTek | cw.hsu@mediatek.com |
| 17 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com |
| 18 | Alireza Aminlou | Nokia | alireza.aminlou@nokia.com |
| 19 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |
| 20 | Muhammed Coban | Qualcomm | mcoban@qti.qualcomm.com |
| 21 | Vadim Seregin | Qualcomm | vseregin@qti.qualcomm.com |
| 22 | Amith Ds | Samsung | amith.ds@samsung.com |
| 23 | Raj Gadde | Samsung | raj.gadde@samsung.com |
| 24 | Chirag Pujara | Samsung | chirag.p@samsung.com |
| 25 | Sachin Despande | Sharp | sdeshpande@sharplabs.com |
| 26 | Andrew Segall | Sharp | asegall@sharplabs.com |
| 27 | Takeshi Tsukuba | Sony | takeshi.tsukuba@sony.com |

# Test conditions and evaluation criteria

Tests in the CE shall be conducted and evaluated by using test conditions and evaluation procedure as described in JVET-M0870.

# Tests specification

| Test# | Description | Document# |
| --- | --- | --- |
| 12.1 | Padding tile group boundary and allow motion vectors to point to samples outside the boundaries in the reference pictures as if they are picture boundaries | JVET-M0136 JVET-M0261 JVET-M0530 |
| 12.2 | Padding tile boundary and allow motion vectors to point to samples outside the boundaries in the reference pictures as if they are picture boundaries. In this case, each tile is considered as MCTS | JVET-M0136 |
| 12.3 | Restrict motion vectors on encoder-side to disallow pointing to samples outside the tile group boundaries in the reference pictures | JVET-M0445 |

# CE 12.1: Treating tile group boundaries as picture boundaries

The proposal is summarized as follows:

- High-level syntax to specify whether each tile group boundary of the tile group is treated the same as picture boundary in the decoding process. For example, a flag, tile_group_boundary_treated_as_pic_boundary_flag, could be added to the tile group header.

- When the boundaries of a tile group are indicated to be treated as picture boundaries, the following applies:

- In the derivation process for temporal luma motion vector prediction, the right and bottom picture boundary positions used in the process, indicated by pic_height_in_luma_samples − 1 and pic_width_in_luma_samples − 1, respectively, are replaced with the right and the bottom boundary positions, respectively, of the tile group, in units of luma samples.

- In the luma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples − 1, 0, pic_width_in_luma_samples − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile group, in units of luma samples, respectively.

- In the chroma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples / SubWidthC − 1, 0, pic_width_in_luma_samples / SubWidthC − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile group, in units of chroma samples, respectively.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE12.1 | Alireza Aminlou alireza.aminlou@nokia.com Hendry fnu.hendry@huawei.com Muhammed Coban mcoban@qti.qualcomm.com JVET-M0136, JVET-M0261, JVET-M0530 | HHI |

# CE 12.2: Treating tile boundaries as picture boundaries

The proposal is summarized as follows:

- High-level syntax to specify whether each tile boundary is treated the same as picture boundary in the decoding process. For example a flag, tile_boundary_treated_as_pic_boundary_flag, could be added to the PPS,

- When the boundaries of tiles are indicated to be treated as picture boundaries, the following applies:

- In the derivation process for temporal luma motion vector prediction, the right and bottom picture boundary positions used in the process, indicated by pic_height_in_luma_samples − 1 and pic_width_in_luma_samples − 1, respectively, are replaced with the right and the bottom boundary positions, respectively, of the tile, in units of luma samples.

- In the luma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples − 1, 0, pic_width_in_luma_samples − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile, in units of luma samples, respectively.

- In the chroma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples / SubWidthC − 1, 0, pic_width_in_luma_samples / SubWidthC − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile, in units of chroma samples, respectively.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE12.2 | Hendry fnu.hendry@huawei.com JVET-M0136 | Nokia |

# CE 12.3: Encoder-side tile group restriction

Motion search is restricted on encoder-side to obey tile boundaries in the following way:

- No sample values outside each identified tile set (i.e., tile or tile group) or outside the picture are referenced for inter prediction.

- No usage of motion vector candidates derived from blocks outside each identified tile set .

- ALF and BIO are disabled.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE12.3 | Robert Skupin robert.skupin@hhi.fraunhofer JVET-M0445 | Huawei Qualcomm |

# Expected Gains

It is expected that the techniques being tested provide coding efficiency improvement.

# Time-line and Responsibilities

2019-January-31:	Final CE description uploaded

2019-February18:	Cross-checking begins, proponents provide software

2019-March-01: 	Final version of CE software, full test results must be provided at this time.

Note: The above time-line may be revised depending on the availability of tiles implementation in the VVC reference software.