| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0869 |
| --- | --- |

| Title: | Decoding Parameter Set with default ID equal to zero |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Wade Wan | Email: | stewe@stewe.org wade.wan@broadcom.com |
| Source: | Tencent, Broadcom |  |  |

_____________________________

# Abstract

This document contains the authors’ attempt to represent the compromise regarding the Decoding Parameter Set (DPS) reached on March 23 in the JVET Track B (joint meeting with systems).  A DPS with a DPS ID equal to 0 is defined in the VVC specification as a DPS which does not set any constraints on the bitstream.  It’s a requirement of bitstream compliance that, if a DPS with an ID equal to 0 is activated, it complies with that set of restrictions.

Defining a DPS with ID equal to 0, today, is asserted to be reasonably straightforward and provided below, with the exception of tiers, profiles, and the DPS extension mechanism.  Some thoughts are provided regarding potential specification problems related to tiers and profiles.  Solutions to these problems may well be better devised once we have a more thorough understanding of future Annex A, and therefore proposed for further study.

Future proponents to DPS-related technologies are encouraged/requested/required to provide text reflecting the values of proposed codepoints for the DPS with ID equal to zero.

# Introduction

During the discussions in Track B March 23 morning, a compromise was reached that would allow those applications not interested in including bitstream restrictions in a DPS into the bitstream to do so, without reducing the value of the DPS for those who need it.  The nature of that compromise is that VVC defines the DPS with an ID equal to 0 such that indicates no bitstream restrictions.

# Known Issues

At present, the constraint flags and the profile/level structure are in the SPS.  Per agreement, we have the same information also in the DSP with the understanding that, combined, they represent the most complex video bitstream a decoder can expect.

A major disconnect between the two authors has been whether or not the constraint flags are also present in the SPS.  The text below assumes that they are optionally present in either or both SPS and DPS, and that, if a flag is present in both SPS and DPS, they must not be inconsistent.  The content of the DPS cannot be more restrictive than the SPS because the scope of the DPS is a bitstream and the scope of a SPS is a CVS and the bitstream is made up of the CVSs.  Therefore, a constraint flag in the active DPS must be at least as permissive as the same flag etc. in the SPS.

At present, the mandatory presence of the profile_tier_level structure in the DPS implies only a single profile in a given video bitstream.  At least for profiles, it has been commented that it may be undesirable to restrict a video bitstream to a single profile.  Fixing this situation, either by disallowing multiple profiles per bitstream, by a “kitchen sink” profile/tier, by onion-shaped profiles/tiers, or by other means needs to be studied further, probably towards the end of the standard when we discuss Annex A.  Note that this issue is not directly related to default values for DPS 0, though it has of course implications for that case as well.

Finally, we note that the single bit “tier” and the levels are onion shaped in that when tier = 1, the bitrates of tier=0 are also possible.  The syntax below reflects that.

During discussions between the authors, a possible need was identified for a bitstream restriction to the extent that the DPS cannot be more restrictive than any SPS in the bitstream.  That is left for further study.

# Draft Text

Below is the syntax of the decoding_parameter_set and referenced syntax structures per JVET-M101, JVET-N0349 and JVET-M1001-v6.

From JVET-N0349, with an additional column showing the values for dps_decoding_parameter_set_id equal to 0.  That additional column may go into the draft text, or we could rely on language in the semantics; both are shown in the markup and we leave it to the editor to pick one way or the other (or both).

| decoding_parameter_set_rbsp( ) { | Descriptor | Value for DPS 0 |
| --- | --- | --- |
| dps_decoding_parameter_set_id | u(4) | 0 |
| dps_reserved_three_2bits | u(2) | 11b |
| dps_max_sub_layers_minus1 | u(3) | 111b |
| dps_reserved_zero_7bits | u(7) | 0 |
| profile_level( dps_max_sub_layers_minus1 , TRUE) |  | See Below |
| dps_extension_flag | u(1) | 0 |
| if( dps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| dps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

| sequence_parameter_set_rbsp( ) { | Descr iptor | Value for DPS 0 |
| --- | --- | --- |
| […[ |  |  |
| profile_level( dps_max_sub_layers_minus1 , FALSE) |  |  |
| […] |  |  |
| } |  |  |

Text below from JVET-N101-spec_r2

### x.x.x.x Decoding parameter set RBSP semantics

NOTE 1 – DPS NAL units are required to be available to the decoding process prior to their activation (either in the bitstream or by external means), as specified in clause 7.4.2.4.2. However, the DPS RBSP contains information that is not necessary for operation of the decoding process specified in clauses 2 through 10 of this Specification.

dps_decoding_parameter_set_id identifies the DPS for reference by other syntax elements.

dps_reserved_three_2bits shall be equal to 3 in bitstreams conforming to this version of this Specification. The value 0 for dps_reserved_zero_bit is reserved for future use by ITU-T | ISO/IEC.

dps_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each CVS referring to the DPS. The value of dps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.  When dps_decoding_parameter_set_id equal to 0, dps_max_sub_layers_minus1 shall be 7.

dps_reserved_zero_7bits shall be equal to 0 in bitstreams conforming to this version of this Specification. The other values for dps_reserved_zero_bit is reserved for future use by ITU-T | ISO/IEC.

dps_extension_flag equal to 0 specifies that no dps_extension_data_flag syntax elements are present in the DPS RBSP syntax structure. dps_extension_flag equal to 1 specifies that there are dps_extension_data_flag syntax elements present in the DPS RBSP syntax structure.  When dps_decoding_parameter_set_id equal to 0, dps_extension_flag shall be 0.  Further study encouraged.

dps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annex A. Decoders conforming to this version of this Specification shall ignore all dps_extension_data_flag syntax elements.

Text below change marked against JVET-M1001v6

#### x.x.x.x General profile, tier, and level syntax

| profile_tier_level( maxNumSubLayersMinus1 , inDPS) { | Descriptor | Value for DPS with id=0 |
| --- | --- | --- |
| general_profile_idc | u(7) | See below |
| general_tier_flag | u(1) | 1 |
| general_constraint_info( ) |  |  |
| general_level_idc | u(8) | 0xff |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |  |
| sub_layer_level_present_flag[ i ] | u(1) | 0 |
| while( !byte_aligned( ) ) |  |  |
| ptl_alignment_zero_bit | f(1) |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |  |
| if( sub_layer_level_present_flag[ i ] ) |  |  |
| sub_layer_level_idc[ i ] | u(8) |  |
| } |  |  |

When the profile_tier_level( ) structure is included in a DPS (inDPS equal to TRUE), and with dps_decoding_parameter_set_id equal to 0, decoders shall infer the value from general_profile_idc from the general_profile_idc of the SPS (where inDPS is equal to FALSE) that actives that DPS.

Note: there are other options to get out of the profile_idc problem.  For example, VVC could create a “kitchen sink” profile, that’s defined such to include all possible options feasible in the syntax.  In that case, DPS-level profile IDC could be set to the kitchen sink profile.

### Profile, tier, and level semantics

#### General profile, tier, and level semantics

When the profile_tier_level( ) structure is included in an SPS (inDPS is equal to FALSE), the BitstreamInScope is the CVS for which the SPS is the active SPS.

When the profile_tier_level( ) structure is incldued in a DPS (inDPS is equal to TRUE), the BistreamInScope is the video bitstream for which the DPS is the active DPS.

general_profile_idc indicates a profile to which BitstreamInScope conforms as specified in Annex A. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex A. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC. When in the DPS (inDPS equal to TRUE), and when the dps_decoding_parameter_set_id equal to 0, general_profile_idc shall be inferred to be the same general_profile_idc as the general_profile_idc that is present in the SPS that activates that DPS.

general_tier_flag specifies the tier context for the interpretation of general_level_idc as specified in Annex A.  When dps_decoding_parameter_set_id is equal to 0, general_tier_flag in that DPS is inferred to equal to 1.

general_level_idc indicates a level to which BitstreamInScope conforms as specified in Annex A. Bitstreams shall not contain values of general_level_idc other than those specified in Annex A. Other values of general_level_idc are reserved for future use by ITU-T | ISO/IEC.

NOTE 1 – A greater value of general_level_idc indicates a higher level.

NOTE 2 – When BitstreamInScope conforms to multiple profiles, general_profile_idc should indicate the profile that provides the preferred decoded result or the preferred bitstream identification, as determined by the encoder (in a manner not specified in this Specification).

When in the DPS (inDPS is equal to TRUE), and when the dps_decoding_parameter_set_id equal to 0, general_level_idc shall be 0xff.

sub_layer_level_present_flag[ i ] equal to 1 specifies that level information is present in the profile_tier_level( ) syntax structure for the sub-layer representation with TemporalId equal to i. sub_layer_level_present_flag[ i ] equal to 0 specifies that level information is not present in the profile_tier_level( ) syntax structure for the sub-layer representation with TemporalId equal to i.

When in the DPS, and when the dps_decoding_parameter_set_id equal to 0, sub_layer_level_present_flag[ i ] shall be 0.  Informative note: that means all sublayers can have the highest level.

ptl_alignment_zero_bits shall be equal to 0.

The semantics of the syntax element sub_layer_level_idc[ i ] is, apart from the specification of the inference of not present values, the same as the syntax element general_level_idc, but apply to the sub-layer representation with TemporalId equal to i.

#### General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| frame_only_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_pcm_constraint_flag | u(1) |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_mts_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_gbi_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_triangle_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_cpr_constraint_flag | u(1) |
| no_qp_delta_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| // ADD reserved bits for future extensions |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| } |  |

#### General constraint information semantics

general_progressive_source_flag and general_interlaced_source_flag are interpreted as follows:

–	If general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures in BitstreamInScope should be interpreted as progressive only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the source scan type of the pictures in BitstreamInScope should be interpreted as interlaced only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures in BitstreamInScope should be interpreted as unknown or unspecified.

–	Otherwise (general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 1), the source scan type of each picture in BitstreamInScope should be interpreted as unknown or unspecified.

[Ed. If picture timing SEI message is specified for VVC, general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 1 can be specified to indicate the source scan type of each picture at the picture level using the syntax element source_scan_type in a picture timing SEI message.]

NOTE 1 – Decoders may ignore the values of general_progressive_source_flag and general_interlaced_source_flag. Moreover, the actual source scan type of the pictures is outside the scope of this Specification and the method by which the encoder selects the values of general_progressive_source_flag and general_interlaced_source_flag is unspecified.

general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed. general_non_packed_constraint_flag equal to 0 indicates that the cropped output pictures of the decoded bitstream may require further processing to be suitable for displaying. [Ed. Consider a better naming of this constraint flag, e.g., display_suitability_flag or something like that.]

NOTE 2 – Examples for content that requires additional processing before displaying are bitstreams containing one or more frame packing arrangement SEI messages, equirectangular projection SEI messages, or cubemap projection SEI messages. [Ed. This NOTE should be updated at a later phase of VVC standardization based on which SEI messages are adopted into the VVC design.]

general_frame_only_constraint_flag equal to 1 specifies that BitstreamInScope conveys pictures that represent frames. general_frame_only_constraint_flag equal to 0 specifies that BitstreamInScope conveys pictures that may or may not represent frames.

NOTE 3 – Decoders may ignore the value of general_frame_only_constraint_flag, as there are no decoding process requirements associated with it.

intra_only_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that tile_group_type shall be equal to I. intra_only_constraint_flag equal to 0 does not impose a constraint.

max_bitdepth_constraint_idc specifies that it is a requirement of bitstream conformance that bit_depth_luma_minus8 and bit_depth_chroma_minus8 shall be in the range of 0 to  max_bitdepth_constraint_idc, inclusive.  When in the DPS, and when the dps_decoding_parameter_set_id equal to 0, max_bitdepth_constrainty_idc shall be the highest of all defined bitdepth supported by standard.

max_chroma_format_constraint_idc  specifies that it is a requirement of bitstream conformance that chroma_format_idc shall be in the range of 0 to max_chroma_format_constraint_idc, inclusive.  When in the DPS, and when the dps_decoding_parameter_set_id equal to 0, max_chroma_format_constraing_idc shall be the highest of all defined chroma formats supported by standard, assuming they are onion-shaped (which the currently seem to be.

frame_only_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that the CVS conveys pictures that represent  frames. frame_only_constraint_flag equal to 0 does not impose a constraint.

no_qtbtt_dual_tree_intra constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that qtbtt_dual_tree_intra_flag shall be equal to 0. no_qtbtt_dual_tree_intra constraint_flag equal to 0 does not impose a constraint.

no_sao_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_sao_enabled_flag shall be equal to 0. no_sao_constraint_flag equal to 0 does not impose a constraint.

no_alf_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_alf_enabled_flag shall be equal to 0.  no_alf_constraint_flag equal to 0 does not impose a constraint.

no_pcm_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_pcm_enabled_flag shall be equal to 0.  no_pcm_constraint_flag equal to 0 does not impose a constraint.

no_ref_wraparound_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_ref_wraparound_enabled_flag shall be equal to 0. no_ref_wraparound_constraint_flag equal to 0 does not impose a constraint.

no_temporal_mvp_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_temporal_mvp_enabled_flag shall be equal to 0. no_temporal_mvp_constraint_flag equal to 0 does not impose a constraint.

no_sbtmvp_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_sbtmvp_enabled_flag shall be equal to 0. no_sbtmvp_constraint_flag equal to 0 does not impose a constraint.

no_amvr_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_amvr_enabled_flag shall be equal to 0. no_amvr_constraint_flag equal to 0 does not impose a constraint.

no_bdof_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_bdof_enabled_flag shall be equal to 0. no_bdof_constraint_flag equal to 0 does not impose a constraint.

no_cclm_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_cclm_enabled_flag shall be equal to 0. no_cclm_constraint_flag equal to 0 does not impose a constraint.

no_mts_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_mts_enabled_flag shall be equal to 0. no_mts_constraint_flag equal to 0 does not impose a constraint.

no_affine_motion_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_affine_enabled_flag  shall be equal to 0. no_affine_motion_constraint_flag equal to 0 does not impose a constraint.

no_gbi_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_gbi_enabled_flag shall be equal to 0. no_gbi_constraint_flag equal to 0 does not impose a constraint.

no_ciip_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_ciip_enabled_flag shall be equal to 0. no_cipp_constraint_flag equal to 0 does not impose a constraint.

no_triangle_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_triangle_enabled_flag shall be equal to 0. no_triangle_constraint_flag equal to 0 does not impose a constraint.

no_ladf_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_ladf_enabled_flag  shall be equal to 0. no_ladf_constraint_flag equal to 0 does not impose a constraint.

no_cpr_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_cpr_enabled_flag shall be equal to 0. no_cpr_constraint_flag equal to 0 does not impose a constraint.

no_qp_delta_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that cu_qp_delta_enabled_flag shall be equal to 0. no_qp_delta_constraint_flag equal to 0 does not impose a constraint.

no_dep_quant_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that dep_quant_enabled_flag shall be equal to 0. no_dep_quant_constraint_flag equal to 0 does not impose a constraint.

no_sign_data_hiding_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sign_data_hiding_enabled_flag shall be equal to 0. no_sign_data_hiding_constraint_flag equal to 0 does not impose a constraint.

gci_alignment_zero_bits shall be equal to 0.

When general constrants information semantcs are present in the DPS (inDPS is equal to 1), and when the dps_decoding_parameter_set_id equal to 0, all above flags shall be 0.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Broadcom may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).