| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-N0007-v2 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall W. Husak E. Francois D. Rusanovskyy | Email: | asegall@sharplbas.com WJH@dolby.com Edouard.Francois@technicolor.com dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 14th meeting in Geneva, CH (19 – 27 March 2019) and the 15th meeting in Gothenburg, SE (3 – 12 July 2019).

A revision of the document was provided that updated the run-time data in Section 2.1.2.

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Prepare for expert viewing of HDR content at the 15th JVET meeting if feasible.

- Coordinate implementation of HDR anchor aspects in the test model software with AHG3.

- Study additional aspects of coding HDR/WCG content.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with an [AHG7] indication on message headers.  The primary activity of the AhG was related to the mandates of comparing the performance of the VTM for HDR/WCG content. This work is described in the following subsection.

## Performance Comparison

The AhG performed experiments comparing the performance of VTM 5.0 and VTM 4.0 codecs on HDR content.  A summary of the performance is provided below.

### VTM 5.0 versus VTM 4.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 5.58% | -2.48% | -2.94% | -2.51% | 5.74% | -2.73% | -1.34% | 8.25% | 165% | 109% |
| Class H2 |  |  |  |  |  | -1.31% | -3.96% | -8.62% | 143% | 105% |
| Overall | 5.58% | -2.48% | -2.94% | -2.51% | 5.74% | -2.21% | -2.29% | 2.11% | 157% | 108% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 6.47% | -1.44% | -2.15% | -2.02% | 8.75% | -2.04% | -1.65% | 10.99% | 128% | 109% |
| Class H2 |  |  |  |  |  | -2.76% | -3.86% | -9.07% | 112% | 105% |
| Overall | 6.47% | -1.44% | -2.15% | -2.02% | 8.75% | -2.30% | -2.46% | 3.70% | 122% | 108% |

### VTM 5.0 versus HM 16.18

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -40.05% | -24.57% | -24.40% | -53.41% | -56.13% | -22.80% | -49.33% | -48.68% | 2386% | 166% |
| Class H2 |  |  |  |  |  | -20.82% | -38.50% | -40.17% | 2198% | 172% |
| Overall | -40.05% | -24.57% | -24.40% | -53.41% | -56.13% | -22.08% | -45.39% | -45.59% | 2316% | 168% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -38.75% | -29.15% | -28.91% | -52.95% | -51.45% | -26.84% | -48.47% | -43.51% | 378% | 146% |
| Class H2 |  |  |  |  |  | -26.98% | -44.76% | -48.72% | 563% | 160% |
| Overall | -38.75% | -29.15% | -28.91% | -52.95% | -51.45% | -26.89% | -47.12% | -45.41% | 437% | 151% |

# Contributions

There are seven contributions related to HDR video coding:

| JVET-O0432 | AHG10: LMCS encoder improvement | F. Pu, S. McCarthy, T. Lu, P. Yin, W. Husak, T. Chen (Dolby) |
| --- | --- | --- |
| JVET-O0661 | AHG2/AHG17: Draft text to update BT.2100 references and ICTCP HLG coding equations | S. McCarthy, F. Pu, T. Lu, P. Yin, W. Husak, T. Chen |
| JVET-O0657 | AHG7: Tool-on/off experiments for HLG CTC sequences | S. Iwamura, S. Nemoto, A. Ichigaya (NHK) |
| JVET-O0658 | Crosscheck of JVET-O0432 on LMCS encoder improvements | S. Iwamura (NHK) |
| JVET-O675 | Crosscheck of JVET-O0432: LMCS encoder improvement | J. Zhao (LGE) |
| JVET-O0756 | HDRTools: VTM software integration | D. Mehlem, A.M. Tourapis, D. Singer |
| JVET-O0966 | Cross-check of JVET-O0657 (AHG7: Tool-on/off experiments for HLG CTC sequences) | P. Cowan, A. Segall (Sharp) |

# Additional Comments

## VTM Performance

It was observed that there was a rather large chroma loss between VTM 4.0 and VTM 5.0.  This was investigated during the AhG period, and it was identified that the integration of JVET-M0091 resulted in the loss (commit: 65c9bacdf38).

It is encouraged that QP related adoptions, including those felt to be non-normative, be tested with the HDR CTC.

## Common Test Condition

Computing the metrics for the common test conditions requires the use of the external HDRMetrics package, which in terms requires the management of additional configuration files to control the conversion of decoded YUV sequences to floating point representations in linear light.  In one case, a separate configuration file is used for the ShowGirl sequence.

It is noted that JVET-O0756 provides software that incorporates the HDRMetrics packages into the VTM software.  This is expected to streamline the calculation of the CTC metrics, and it will have a further benefit of reducing the number of configuration files that need to be managed.

# Recommendations

The AHG recommends the following:

- Review all input contributions