| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0021-v2 |
| --- | --- |

| Title: | CE1: Summary report on CABAC initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen Jie Dong Heiner Kirchhoffer | Tel: Email: | fbossen@sharpsec.com jiedong@qti.qualcomm.comheiner.kirchhoffer@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

Core experiment 1 on CABAC initialization was conducted between the 14th and 15th JVET meeting. Results are reported for five experiments, all aimed at simplifying the CABAC initialization process.

# Summary

This section provides a summary of the CE. Detailed reports are provided in the following input contributions, which were provided in a timely manner:

- JVET-O0065: CE1-1.2 and CE1-2.1: Simplification of CABAC initialization process

- JVET-O0085: CE1-1.1: Simplification of the initialization process for context variables

- JVET-O0112: CE1: CABAC Initialization, all experiments

All software related to this CE is available on a protected git repository: https://vcgit.hhi.fraunhofer.de/jvet-n-ce1/VVCSoftware_VTM.git

## Experiments description

Detailed descriptions of experiments are provided in the CE description document JVET-N1021. At a high-level, experiments can be distinguished between:

- Methods that use a 6-bit initValue (CE1-1.1, CE1-1.2, CE1-3.1.b) and methods that use an 8-bit initValue (CE1-2.1, CE1-3.1.a)

- Methods that use a linear space (CE1-1.1, CE1-1.2, CE1-2.1) and methods that use a logarithmic space (CE1-3.1.a, CE1-3.1.b)

## Coding efficiency

In addition to regular CTC conditions, the “random access” configuration was also tested at very low (2-17) and very high (42-57) QP values. The table below captures BD rate difference averages for the luma component.

|  |  | Experiment | AI | RA | LB | LP | RA (low) | RA (high) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6-bit | Linear Space | CE1-1.1 | -0.02% | 0.01% | 0.04% | 0.00% | -0.01% | 0.66% |
|  |  | CE1-1.2 | -0.02% | -0.01% | 0.03% | -0.09% | -0.01% | 0.22% |
| 8-bit |  | CE1-2.1 | -0.02% | -0.02% | -0.01% | -0.07% | -0.02% | 0.74% |
|  | Logarithmic Space | CE1-3.1.a | -0.01% | -0.02% | -0.01% | -0.02% | 0.00% | 0.33% |
| 6-bit |  | CE1-3.1.b | 0.00% | 0.00% | 0.00% | -0.04% | 0.01% | 0.65% |

Coding efficiency differences are minor except for the very high QP values case. In general, no significant differences in coding efficiency are observed between:

- Methods that use a 6-bit initValue (CE1-1.1, CE1-1.2, CE1-3.1.b) and methods that use an 8-bit initValue (CE1-2.1, CE1-3.1.a)

- Methods that use a linear space (CE1-1.1, CE1-1.2, CE1-2.1) and methods that use a logarithmic space (CE1-3.1.a, CE1-3.1.b)

## Complexity Analysis

No complexity analysis metric was defined in the CE description. Nevertheless, at a high level, the following can be said about the methods studied in this CE:

- all methods are less complex than the method in the current VVC draft because they avoid a large lookup table

- methods that use a logarithmic space (CE1-3.1.x) are a bit more complex because a log-to-linear conversion needs to be done

- methods that use a 6-bit initValue (CE1-1.1, CE1-1.2, CE1-3.1.b) do not require more operations than those that use an 8-bit initValue (CE1-2.1, CE1-3.1.a)

An analysis of operation counts for each method is provided below, based on the implementations provided in the CE software repository. Note that multiplications by a constant are counted as combinations of shift and add operations. Also note that the bit width of each operation is not considered here.

|  | Shift (<<, >>) | Multiplication (×) | Addition (+/-) | AND (&) | XOR (^) | Compare (if, Clip3) |
| --- | --- | --- | --- | --- | --- | --- |
| CE1-1.1 | 5 | 1 | 5 | 1 | 0 | 4 |
| CE1-1.2 | 5 | 1 | 5 | 1 | 0 | 2 |
| CE1-2.1 | 4 | 1 | 4 | 1 | 0 | 2 |
| CE1-3.1.a | 8 | 1 | 8 | 2 | 1 | 2 |
| CE1-3.1.b | 10 | 1 | 8 | 2 | 1 | 2 |

Additionally, cycle counts based on optimized software implementations are reported in JVET-O0191.

## Training time

A common initial state training algorithm was used for all experiments. The training algorithm is split in 3 steps:

- Bin extraction

- Rate computation

- Parameter selection

The first two steps can be run concurrently for each bitstream in the test set. The last step is run once and uses data extracted from all bitstreams.

Tools for training were kindly provided by Fraunhofer HHI. A revision to the tools was provided on May 24, 2019 after it was pointed out that the initial implementation included a bit count rounding operation that wasn’t described in JVET-N1021.

Training times were reported as follows:

| Source | Experiments | Time |
| --- | --- | --- |
| HHI | CE1-1.1 | All bitstreams combined: 3.37h for bin extraction 17.44h for rate computation, 0.72h for parameter selection. Slowest bitstream: 50min for rate computation (BQTerrace LB QP22) |
| Sharp | CE1-3.1.b | 2.51h for bin extraction, 15.05h for rate computation, 0.59h for parameter selection. Slowest bitstream: 89min for all steps |
| Sharp | CE1-3.1.a | 2.51h for bin extraction, 60.98h for rate computation, 0.86h for parameter selection. Slowest bitstream: 262min for all steps |

As expected, training for experiments that use a 6-bit initValue (CE1-1.1, CE1-1.2, CE1-3.1.b) is faster than for experiments that use an 8-bit initValue (CE1-2.1, CE1-3.1.a) by a factor of 3-4x. The amount of time required for training is much smaller than the amount of time required to generate bitstreams used for training. For instance, in a distributed environment, training can be completed in a matter of hours, where bitstream generation takes days.

## Testers and Crosscheckers

| Experiment | Tester | Crosschecker | Crosschecker comment |
| --- | --- | --- | --- |
| CE1-1.1 | HHI | Qualcomm, Sharp | The results provided in the first version of JVET-O0085 are incorrect (as mentioned in the document) as the incorrect version of the training tools was used. Otherwise everything as expected. |
| CE1-1.2 | Qualcomm | Sharp, HHI | Implementations and results are verified. |
| CE1-2.1 | Qualcomm | Sharp, HHI | Implementations and results are verified. |
| CE1-3.1.a | Sharp | HHI, Qualcomm | Implementations and results are verified. |
| CE1-3.1.b | Sharp | HHI, Qualcomm | Implementations and results are verified. |

## Other contributions related to CE1

The following contributions related to CE1 have been registered:

- JVET-O0191: CE1-related: Simplified CABAC initialization method

- JVET-O0554: Removal of infrequently used context models

- JVET-O0741: Non-CE1: Context memory reduction via selective adaptation

- JVET-O0946: CE1-related: Simplification of JVET-O0191 using 4 or 6 bit per initialization value

# Detailed results

## Experiment 1.1: 6-bit init value, linear space

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.02% | 0.00% | 101% | 101% |
| Class A2 | -0.01% | 0.01% | 0.00% | -0.01% | 99% | 100% |
| Class B | -0.01% | 0.06% | 0.07% | -0.01% | 101% | 102% |
| Class C | -0.03% | 0.01% | 0.11% | -0.02% | 102% | 101% |
| Class E | -0.05% | 0.06% | -0.06% | -0.04% | 101% | 100% |
| Overall | -0.02% | 0.04% | 0.03% | -0.01% | 101% | 101% |
| Class D | -0.04% | 0.20% | 0.08% | -0.01% | 100% | 100% |
| Class F | 0.00% | 0.11% | 0.11% | 0.02% | 100% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.17% | 0.21% | 0.01% | 100% | 101% |
| Class A2 | 0.03% | 0.32% | 0.11% | 0.06% | 100% | 101% |
| Class B | 0.03% | 0.24% | -0.14% | 0.03% | 100% | 101% |
| Class C | -0.03% | 0.08% | 0.12% | -0.03% | 101% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 0.20% | 0.05% | 0.02% | 100% | 101% |
| Class D | 0.01% | 0.35% | 0.37% | 0.04% | 101% | 101% |
| Class F | 0.00% | 0.17% | 0.16% | 0.01% | 100% | 101% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.01% | 0.68% | -0.15% | 0.04% | 102% | 102% |
| Class C | -0.01% | 0.57% | 0.15% | 0.03% | 100% | 101% |
| Class E | 0.14% | 1.26% | 0.40% | 0.20% | 100% | 103% |
| Overall | 0.04% | 0.79% | 0.09% | 0.08% | 101% | 102% |
| Class D | 0.04% | 0.04% | 0.83% | 0.07% | 101% | 101% |
| Class F | -0.02% | -0.25% | -0.68% | -0.11% | 100% | 101% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.02% | 0.49% | 0.28% | 0.00% | 104% | 103% |
| Class C | -0.02% | 0.33% | 0.35% | 0.00% | 101% | 100% |
| Class E | 0.05% | 1.09% | 0.65% | 0.12% | 99% | 101% |
| Overall | 0.00% | 0.58% | 0.39% | 0.03% | 102% | 102% |
| Class D | 0.01% | 0.20% | 0.95% | 0.04% | 97% | 98% |
| Class F | -0.08% | -0.11% | -0.43% | -0.02% | 99% | 102% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.77% | 1.85% | 0.90% | 0.84% | 98% | 99% |
| Class A2 | 0.55% | 0.76% | 0.24% | 0.56% | 96% | 100% |
| Class B | 0.81% | 1.01% | 0.84% | 0.82% | 94% | 100% |
| Class C | 0.46% | 1.05% | 0.65% | 0.48% | 98% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | 0.66% | 1.14% | 0.68% | 0.68% | 96% | 100% |
| Class D | 0.97% | 2.17% | 1.29% | 0.98% | 98% | 100% |
| Class F | 0.12% | 0.43% | 0.45% | 0.15% | 98% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VT-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.03% | 0.01% | 100% | 102% |
| Class A2 | -0.02% | 0.02% | 0.01% | 0.00% | 100% | 101% |
| Class B | -0.01% | 0.04% | 0.03% | 0.01% | 101% | 100% |
| Class C | -0.02% | 0.05% | 0.02% | -0.01% | 101% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.04% | 0.02% | 0.00% | 101% | 101% |
| Class D | -0.04% | 0.02% | 0.01% | -0.03% | 101% | 101% |
| Class F | 0.22% | 0.02% | 0.01% | -0.32% | 101% | 101% |

## Experiment 1.2: 6-bit init value, linear space

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.04% | 0.00% | 103% | 101% |
| Class A2 | 0.00% | 0.01% | -0.04% | 0.00% | 101% | 101% |
| Class B | -0.01% | 0.02% | 0.02% | -0.01% | 102% | 101% |
| Class C | -0.02% | -0.02% | 0.03% | -0.02% | 102% | 100% |
| Class E | -0.05% | -0.06% | -0.04% | -0.05% | 101% | 99% |
| Overall | -0.02% | -0.01% | -0.01% | -0.02% | 102% | 101% |
| Class D | -0.02% | 0.32% | -0.15% | -0.01% | 100% | 100% |
| Class F | -0.01% | 0.12% | -0.08% | 0.01% | 98% | 100% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.12% | -0.01% | 100% | 102% |
| Class A2 | -0.01% | 0.34% | 0.06% | 0.02% | 99% | 102% |
| Class B | 0.00% | 0.20% | -0.13% | 0.00% | 100% | 100% |
| Class C | -0.04% | 0.16% | -0.05% | -0.03% | 99% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.18% | -0.02% | 0.00% | 100% | 101% |
| Class D | 0.02% | 0.29% | 0.00% | 0.03% | 101% | 101% |
| Class F | 0.01% | 0.05% | 0.08% | 0.02% | 100% | 100% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.34% | -0.12% | 0.00% | 100% | 102% |
| Class C | -0.01% | 0.37% | -0.25% | 0.00% | 101% | 101% |
| Class E | 0.16% | -1.00% | 0.72% | 0.12% | 95% | 100% |
| Overall | 0.03% | 0.01% | 0.05% | 0.03% | 99% | 101% |
| Class D | -0.03% | 0.50% | -1.76% | -0.06% | 100% | 101% |
| Class F | -0.07% | -0.34% | -0.30% | -0.13% | 98% | 99% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.05% | 0.02% | -0.31% | -0.06% | 104% | 104% |
| Class C | -0.07% | 0.02% | -0.15% | -0.07% | 102% | 101% |
| Class E | -0.16% | -0.65% | 0.35% | -0.16% | 96% | 100% |
| Overall | -0.09% | -0.15% | -0.09% | -0.09% | 101% | 102% |
| Class D | 0.05% | 0.21% | -1.66% | 0.01% | 99% | 100% |
| Class F | -0.12% | -0.01% | -0.11% | -0.07% | 100% | 102% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.54% | 1.46% | 0.65% | 0.52% | 100% | 99% |
| Class A2 | 0.36% | 1.06% | 0.30% | 0.39% | 98% | 99% |
| Class B | 0.13% | 0.65% | 0.37% | 0.16% | 95% | 100% |
| Class C | -0.02% | 0.54% | 0.56% | 0.02% | 97% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.22% | 0.87% | 0.46% | 0.24% | 97% | 100% |
| Class D | 0.32% | 0.78% | 1.80% | 0.33% | 93% | 100% |
| Class F | -0.04% | 0.26% | 0.45% | -0.02% | 98% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | 0.08% | 0.07% | 0.02% | 100% | 102% |
| Class A2 | -0.01% | 0.02% | 0.02% | 0.00% | 100% | 101% |
| Class B | -0.01% | 0.05% | 0.04% | 0.01% | 100% | 100% |
| Class C | -0.03% | 0.06% | 0.01% | -0.01% | 101% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.05% | 0.04% | 0.00% | 100% | 101% |
| Class D | -0.03% | 0.13% | -0.11% | -0.02% | 99% | 101% |
| Class F | 0.15% | 0.04% | -0.02% | -0.32% | 99% | 100% |

## Experiment 2.1

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | -0.09% | -0.08% | 0.00% | 103% | 100% |
| Class A2 | -0.01% | -0.01% | -0.03% | -0.01% | 101% | 101% |
| Class B | -0.02% | -0.07% | -0.02% | -0.02% | 99% | 101% |
| Class C | -0.03% | -0.04% | 0.02% | -0.03% | 102% | 100% |
| Class E | -0.06% | 0.01% | 0.07% | -0.05% | 93% | 98% |
| Overall | -0.02% | -0.04% | -0.01% | -0.02% | 100% | 100% |
| Class D | -0.06% | 0.12% | -0.06% | -0.04% | 97% | 98% |
| Class F | -0.04% | 0.04% | -0.14% | -0.02% | 100% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.04% | 0.18% | 0.17% | -0.02% | 100% | 102% |
| Class A2 | 0.01% | 0.18% | 0.05% | 0.03% | 100% | 101% |
| Class B | 0.00% | 0.27% | -0.07% | 0.00% | 99% | 100% |
| Class C | -0.06% | 0.06% | 0.01% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | 0.18% | 0.02% | -0.01% | 100% | 101% |
| Class D | -0.05% | 0.31% | -0.08% | -0.03% | 102% | 101% |
| Class F | -0.02% | 0.13% | 0.05% | 0.00% | 100% | 101% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.52% | -0.10% | 0.02% | 100% | 101% |
| Class C | -0.10% | 0.13% | -0.09% | -0.09% | 100% | 101% |
| Class E | 0.12% | 0.16% | 0.29% | 0.12% | 99% | 101% |
| Overall | -0.01% | 0.30% | 0.00% | 0.01% | 100% | 101% |
| Class D | -0.09% | -0.60% | -0.29% | -0.12% | 100% | 100% |
| Class F | -0.15% | -0.19% | -0.41% | -0.24% | 98% | 100% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.04% | 0.26% | -0.19% | -0.04% | 104% | 102% |
| Class C | -0.10% | -0.16% | 0.19% | -0.10% | 98% | 99% |
| Class E | -0.09% | 0.44% | 0.20% | -0.05% | 99% | 100% |
| Overall | -0.07% | 0.16% | 0.03% | -0.06% | 100% | 100% |
| Class D | -0.04% | -0.83% | -0.53% | -0.09% | 99% | 100% |
| Class F | -0.12% | 0.47% | -0.25% | -0.04% | 99% | 102% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 1.26% | 2.71% | 1.37% | 1.28% | 99% | 98% |
| Class A2 | 1.00% | 1.91% | 0.94% | 1.04% | 97% | 98% |
| Class B | 0.80% | 1.34% | 1.17% | 0.82% | 94% | 99% |
| Class C | 0.07% | 0.45% | 0.13% | 0.08% | 95% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.74% | 1.49% | 0.89% | 0.76% | 96% | 99% |
| Class D | 0.35% | 0.93% | 0.45% | 0.37% | 93% | 99% |
| Class F | 0.10% | 0.40% | 0.30% | 0.13% | 97% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.03% | 0.01% | 101% | 102% |
| Class A2 | -0.02% | 0.01% | 0.01% | -0.01% | 101% | 101% |
| Class B | -0.01% | 0.03% | 0.02% | 0.00% | 102% | 103% |
| Class C | -0.04% | 0.01% | 0.00% | -0.03% | 100% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | 0.02% | 0.02% | -0.01% | 101% | 101% |
| Class D | -0.08% | -0.10% | -0.13% | -0.09% | 100% | 100% |
| Class F | 0.23% | 0.00% | 0.01% | -0.32% | 99% | 100% |

## Experiment 3.1.a: 8-bit init value, log space

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | 0.02% | -0.07% | 0.01% | 99% | 101% |
| Class A2 | 0.00% | -0.03% | -0.06% | -0.01% | 101% | 101% |
| Class B | -0.01% | -0.03% | -0.01% | -0.01% | 98% | 101% |
| Class C | -0.02% | 0.05% | 0.05% | -0.02% | 104% | 101% |
| Class E | -0.03% | -0.05% | -0.11% | -0.04% | 102% | 100% |
| Overall | -0.01% | -0.01% | -0.03% | -0.01% | 101% | 101% |
| Class D | -0.05% | 0.24% | -0.08% | -0.03% | 100% | 100% |
| Class F | -0.02% | -0.04% | 0.02% | -0.01% | 99% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.01% | -0.07% | 0.17% | -0.01% | 101% | 102% |
| Class A2 | -0.02% | 0.26% | -0.08% | 0.00% | 101% | 102% |
| Class B | 0.00% | 0.12% | -0.11% | 0.00% | 100% | 101% |
| Class C | -0.06% | 0.01% | 0.00% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | 0.08% | -0.02% | -0.02% | 100% | 101% |
| Class D | -0.02% | 0.30% | 0.21% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.17% | 0.24% | 0.01% | 99% | 101% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.68% | 0.12% | 0.02% | 101% | 102% |
| Class C | -0.08% | 0.46% | -0.19% | -0.06% | 100% | 101% |
| Class E | 0.08% | 0.01% | 0.02% | 0.08% | 98% | 102% |
| Overall | -0.01% | 0.44% | -0.01% | 0.01% | 100% | 102% |
| Class D | -0.11% | -0.44% | -0.05% | -0.12% | 102% | 101% |
| Class F | -0.11% | -0.07% | -0.45% | -0.15% | 99% | 101% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.05% | 0.32% | -0.13% | -0.04% | 105% | 103% |
| Class C | -0.07% | 0.06% | 0.25% | -0.05% | 102% | 103% |
| Class E | 0.08% | -0.69% | 0.42% | 0.06% | 97% | 101% |
| Overall | -0.02% | -0.02% | 0.14% | -0.02% | 102% | 102% |
| Class D | -0.07% | -0.01% | 0.04% | -0.06% | 100% | 101% |
| Class F | -0.09% | 0.16% | 0.05% | 0.03% | 100% | 103% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.44% | 1.60% | 0.82% | 0.52% | 101% | 100% |
| Class A2 | 0.41% | 0.87% | 0.43% | 0.42% | 99% | 101% |
| Class B | 0.50% | 1.11% | 0.67% | 0.53% | 99% | 100% |
| Class C | -0.05% | 0.30% | 0.28% | -0.03% | 100% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.33% | 0.94% | 0.55% | 0.36% | 100% | 100% |
| Class D | 0.29% | 0.89% | 0.78% | 0.28% | 97% | 99% |
| Class F | 0.15% | 0.17% | 0.39% | 0.17% | 100% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.04% | 0.02% | 101% | 101% |
| Class A2 | -0.02% | 0.01% | 0.01% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.01% | 0.02% | 0.00% | 102% | 103% |
| Class C | -0.01% | -0.01% | 0.03% | -0.01% | 101% | 102% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.03% | 0.00% | 101% | 102% |
| Class D | -0.02% | -0.07% | 0.00% | -0.03% | 101% | 101% |
| Class F | 0.22% | 0.01% | 0.00% | -0.30% | 101% | 102% |

## Experiment 3.1.b: 8-bit init value, log space

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | 0.06% | -0.02% | 0.02% | 101% | 100% |
| Class A2 | 0.00% | 0.06% | 0.03% | 0.01% | 100% | 101% |
| Class B | 0.00% | 0.08% | 0.10% | 0.01% | 98% | 100% |
| Class C | -0.01% | 0.00% | 0.21% | 0.01% | 101% | 97% |
| Class E | -0.02% | -0.05% | 0.06% | -0.02% | 98% | 95% |
| Overall | 0.00% | 0.03% | 0.09% | 0.01% | 99% | 99% |
| Class D | -0.01% | 0.23% | 0.14% | 0.03% | 93% | 96% |
| Class F | 0.01% | 0.13% | 0.00% | 0.02% | 99% | 99% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.03% | 0.08% | 0.27% | -0.01% | 100% | 102% |
| Class A2 | -0.02% | 0.42% | 0.13% | 0.02% | 100% | 102% |
| Class B | 0.03% | 0.36% | 0.02% | 0.05% | 97% | 98% |
| Class C | -0.01% | 0.23% | 0.00% | 0.00% | 99% | 98% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.28% | 0.09% | 0.02% | 99% | 100% |
| Class D | 0.04% | 0.72% | 0.49% | 0.09% | 101% | 100% |
| Class F | 0.07% | 0.35% | 0.29% | 0.09% | 98% | 98% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.01% | 0.80% | 0.31% | 0.05% | 99% | 102% |
| Class C | -0.01% | 0.67% | -0.06% | 0.03% | 98% | 102% |
| Class E | 0.02% | 1.37% | 1.50% | 0.15% | 96% | 101% |
| Overall | 0.00% | 0.90% | 0.48% | 0.07% | 98% | 102% |
| Class D | 0.06% | 0.27% | 0.14% | 0.06% | 99% | 96% |
| Class F | 0.00% | 0.27% | -0.31% | -0.02% | 98% | 99% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.03% | 0.67% | -0.31% | -0.01% | 103% | 102% |
| Class C | -0.02% | 0.39% | 0.21% | 0.01% | 99% | 100% |
| Class E | -0.10% | 1.56% | 1.27% | 0.05% | 97% | 100% |
| Overall | -0.04% | 0.80% | 0.26% | 0.01% | 100% | 101% |
| Class D | 0.08% | 0.41% | -0.38% | 0.07% | 99% | 97% |
| Class F | 0.01% | 0.56% | -0.04% | 0.10% | 99% | 101% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.58% | 1.70% | 0.93% | 0.60% | 101% | 99% |
| Class A2 | 0.47% | 1.08% | 0.28% | 0.48% | 100% | 99% |
| Class B | 0.88% | 1.61% | 1.38% | 0.91% | 101% | 100% |
| Class C | 0.55% | 1.13% | 0.59% | 0.55% | 103% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | 0.65% | 1.39% | 0.86% | 0.67% | 101% | 99% |
| Class D | 1.06% | 2.04% | 1.40% | 1.06% | 99% | 100% |
| Class F | 0.50% | 0.93% | 0.30% | 0.52% | 102% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.06% | 0.02% | 102% | 101% |
| Class A2 | 0.00% | 0.03% | 0.02% | 0.01% | 100% | 102% |
| Class B | 0.01% | 0.04% | 0.04% | 0.01% | 104% | 103% |
| Class C | 0.02% | 0.06% | 0.04% | 0.02% | 100% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.04% | 0.02% | 102% | 102% |
| Class D | 0.10% | 0.14% | 0.09% | 0.09% | 100% | 100% |
| Class F | 0.12% | 0.07% | 0.10% | -0.24% | 100% | 102% |