| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0029 |
| --- | --- |

| Title: | CE9: Summary report on decoder motion vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Xiaoyu Xiu | Tel: Email: | semih.esenlik@huawei.com, xiaoyuxiu@kwai.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

The tools in the scope of this CE include bi-directional optical flow (BDOF) and decoder motion vector refinement (DMVR).

The core experiment summary report is organized into 2 sub-tests as follows:

- CE9-1: BDOF early termination (5 tests)

- CE9-2: DMVR/BDOF enabling conditions (4 tests)

This report summarises the status of each experiment. Crosscheck results are integrated in the report.

# Introduction

The description of Core Experiment 9 (CE9) on decoder motion vector derivation [1] has been finalized on Apr. 17, 2019. After this date the CE description document is updated to include updated timeline and crosschecker informations and typo corrections. A total 7 tests are defined in CE9 description. No deviation from the deadlines have been observed. The goal of this Core Experiment (CE) is to further investigate the techniques of decoder motion vector refinement and bi-directional optical flow techniques.

# Experimental Conditions and Evaluation

## Software and Anchors

The VTM-5.0 software has been used in the CE9 as anchor software.

## Test Sequences, Bit Rates and Coding Conditions

CTC as described in [2].

## Reporting of the Results

The crosscheck results were uploaded to git servers and included in the CE9 summary report. The test results have been uploaded as part of the contribution documents to the JVET document website.

# CE9-1: BDOF early termination

## Test Specifications and Summary of Results

| Test | Description | Y | U | V | EncT | DecT | Doc# |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE9-1.1 | Use integer-distance DMVR cost to disable BDOF with threshold equal to 4x4 level TH/32 (only keep BDOF early termination when DMVR is disabled) | -0.02% | 0.00% | -0.02% | 100% | 98% | JVET-O0054 |
| CE9-1.2 | Use integer-distance DMVR cost to disable BDOF with threshold equal to 4x4 level TH/32 and disable BDOF early termination in all cases. | -0.02% | 0.00% | -0.02% | 100% | 100% | JVET-O0055 |
| CE9-1.3 | withdrawn |  |  |  |  |  |  |
| CE9-1.4 | Difference from CE9-1.2 is only threshold value. BDOF early termination by using the min SAD in DMVR first SAD stage or in integer search stage | 0.01% | -0.05% | -0.03% | 100% | 98% | JVET-O0055 |
| CE9-1.5a | Disabling application of BDOF based on best cost computed or derived in DMVR, when both DMVR and BDOF are applied. Use only best SAD cost obtained from integer distance refinement positions. When only BDOF is applied, VTM5 16x16 early termination is applied. Threshold used is same as that will be used in VTM5 for 16x16 4x4 early termination check is disabled for all cases | -0.01% | 0.01% | -0.02% | 100% | 100% | JVET-O0096 |
| CE9-1.5b | Use a derived best cost from parametric error surface when applicable. Otherwise, same as (CE9-1.5a). | -0.02% | 0.02% | -0.02% | 100% | 100% | JVET-O0096 |

## Details of Tests

|  | the SAD calculated in DMVR used to disable BDOF | Threshold | Early Termination of BDOF, when DMVR is enabled | Early Termination of BDOF, when DMVR is disabled |
| --- | --- | --- | --- | --- |
| CE9-1.1 | integer-distance SAD | BDOF 4x4Level TH | Disable | Subblock level, 4x4 level |
| CE9-1.2 | integer-distance SAD | BDOF 4x4Level TH | Disable | Disable |
| CE9-1.3 | integer-distance SAD | 4*W*(H>>1) for center position, otherwise 8*W*(H>>1) (=4x4 block level TH for BDOF) | Disable | Disable |
| CE9-1.4 | integer-distance SAD | 16*W*(H>>1) | Disable | Disable |
| CE9-1.5a | integer-distance SAD | BDOF Subblock-level TH | Disable | Subblock level |
| CE9-1.5b | fractional-distance SAD; integer-distance SAD, if no fractional-distance SAD | BDOF Subblock-level TH | Disable | Subblock level |

## Crosscheck Results

|  |  | Tester |  | Crosscheck |  | Crosschecker comments |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Crosschecker | EncT | DecT | EncT | DecT |  |
| CE9-1.1 | K. Unno | 100% | 98% | 100% | 100% | The cross-check results match with the results provided by the tester. |
| CE9-1.2 | X. Xiu | 100% | 100% | 99% | 100% | The cross-check results match with the results provided from the proponent. The software implementation was verified to be aligned with CE description. |
| CE9-1.3 | withdrawn |  |  |  |  |  |
| CE9-1.4 | S. Esenlik | 100% | 98% | 102% | 100% | The crosscheck results match with the results provided by the tester. Encoding time increase can be due to cluster instability, as no increase is expected. |
| CE9-1.5a | T. Chujoh | 100% | 100% | 100% | 101% | The cross-check results match with the results provided by the tester. |
| CE9-1.5b | T. Chujoh | 100% | 100% | 100% | 100% | The cross-check results match with the results provided by the tester. |

## Related Contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-O0054 | CE9-1.1 | CE9-1.1: BDOF early termination |
| JVET-O0055 | CE9-1.2 & CE9-1.4 | CE9-1.2 and CE9-1.4: BDOF early termination |
| JVET-O0096 | CE9-1.5a & CE9-1.5b | CE9-1.5: Best DMVR cost based early termination for BDOF |

## Description of the tests

### CE9-1.1: JVET-O0054

In VTM4.0, DMVR is adopted to refine motion vectors (MVs) at the decoder for a coding unit (CU) by minimizing the SAD between L0 and L1 predictors; BDOF refines the luma sample predictors with a two-level early termination process. The first early termination is at CU level, and the second early termination is at 4×4 block level. At each level, the SAD between L0 and L1 predictors in one unit, which is one CU in the first level or one 4×4 block in the second level, is calculated first. If the SAD is smaller than one threshold, then BDOF is not applied to the current unit. Otherwise, BDOF parameters are derived and used to generate the final luma sample predictors. When the CU is coded with regular merge/skip mode, one reference picture is before the current picture and the other reference picture is after the current picture in display order, the temporal distance between the current picture and one reference picture is equal to that between the current picture and the other reference picture, and BCW selects equal weights for L0 and L1 predictors, DMVR is applied first and followed by BDOF. In JVET-N0097, several methods are proposed to relieve the critical path of cascading DMVR and BDOF. In this CE test, two of them are tested. One is using DMVR cost to disable BDOF. And the other is further disabling the early termination in BDOF.

- Use DMVR cost to disable BDOF

- Use DMVR cost to disable BDOF and disable BDOF early termination

### CE9-1.2: JVET-O0055

In this experiment, an early termination for BDOF by using SAD value of DMVR without additional matching process is tested. Depending on the search result of motion vector refinement, if a matching cost of DMVR is greater than or equal to the threshold, BDOF apply, otherwise BDOF does not apply.

In this experiment, all early termination of BDOF is disabled.

### CE9-1.4: JVET-O0055

An alternative method of SAD based early termination for BDOF is tested. Details are as follows.

- SADs are not calculated in the BDOF stage for early termination.

- SADs calculated in the DMVR stage are reused for early termination.

- At a block not applied DMVR, no early terminations are applied.

- If the smallest SAD at integer distance search stage in DMVR is less than the threshold, BDOF is not applied to the sub-PU.

- The threshold is defined as 16 * W * (H >> 1). Here, W means width of the sub-PU, and H means height of the sub-PU.

### CE9-1.5: JVET-O0096

This test uses the lowest SAD cost computed or derived during DMVR at a sub-block level to disable BDOF for that sub-block, whenever both DMVR and BDOF are applied to the CU to which the sub-block belongs. Whenever DMVR is not applied, but BDOF is applied, the 16x16 early termination check present in VTM5 will be used. The test removes the second level early termination for BDOF at the 4x4 level for all cases. The threshold value used shall be the same threshold that is used at the 16x16 level in VTM5. The main purpose of the test is to enable clock-gating for BDOF in hardware using information available from the DMVR stage to achieve a lower average power consumption. In software implementations, it is aimed at preserving a large portion of the average decoding time reduction offered by the early termination checks employed in VTM5.

There will be 2 tests:

- Test-1: Using only the best SAD cost amongst the integer distance DMVR refinement positions to compare against a threshold, and disabling BDOF if the value is less than or equal to the threshold.

- Test-2: Using the derived SAD cost based on the parametric error surface function, when parametric function based refinement is present. Otherwise, the best SAD cost amongst the integer distance DMVR refinement positions is used.

# CE9-2: DMVR/BDOF enabling conditions

## Test Specifications and Summary of results

| Test | Description | Y | U | V | EncT | DecT | Doc# |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE9-2.1a | Harmonization of DMVR and MMVD with MMVD distance threshold (for reference). | -0.10% | -0.10% | -0.15% | 104% | 101% | JVET-O0077 |
| CE9-2.1b | CE9-2.1.a + encoder speed up | -0.10% | -0.10% | -0.13% | 102% | 100% | JVET-O0077 |
| CE9-2.2 | The CIIP with disabled DMVR and BDOF. | -0.01% | -0.02% | -0.01% | 99% | 98% | JVET-O0108 |
| CE9-2.3 | The CIIP with disabled BDOF | 0.00% | -0.01% | -0.02% | 99% | 98% | JVET-O0103 |

## Crosscheck Results

|  |  | Tester |  | Crosscheck |  | Crosschecker comments |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Crosschecker | EncT | DecT | EncT | DecT |  |
| CE9-2.1a | K. Unno | 104% | 101% | 103% | 101% | The cross-check results match with the results provided by the tester. |
| CE9-2.1b | K. Unno | 102% | 100% | 101% | 101% | The cross-check results match with the results provided by the tester. It was confirmed that the encoder was speeded-up. |
| CE9-2.2 | L. Zhao | 99% | 98% | 99% | 98% | - software matching CE description (Yes) - appropriateness of the text changes (Yes) - sufficient results to verify correctness of the test(yes). |
| CE9-2.3 | X. Xiu | 99% | 98% | 99% | 99% | The cross-check results match with the results provided from the proponent. The software implementation was verified to be aligned with CE description. |

## Related Contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-O0077 | CE9-2.1a & CE9-2.1b | CE9-2.1: Harmonization of DMVR and MMVD |
| JVET-O0108 | CE9-2.2 | CE9-2.2: Disabled decoder-side motion refinement (DMVR) and bidirectional optical flow (BDOF) for combined inter and intra prediction (CIIP) |
| JVET-O0103 | CE9-2.3 | CE9-2.3: CIIP with disabled BDOF |

## Description of the tests

### CE9-2.1: JVET-O0077

This test studies a harmonization of DMVR and MMVD. To reduce redundancy between DMVR and MMVD, DMVR is applied with MMVD when MMVD distance is larger than 2pel. Proposed method just changes condition to apply DMVR, hence there is no worst case increasement of decoder and DMVR process is also unchanged.

### CE9-2.2: JVET-O0108

In the current CIIP design, the DMVR and the BDOF are always enabled to generate the inter prediction samples when the current CIIP CU is bi-predicted. In this CE test, to reduce the computational complexity of the CIIP mode, the method of disabling the DMVR and BDOF for the CIIP mode is studied.

### CE9-2.3: JVET-O0103

In this CE test, to reduce the computational complexity of the CIIP mode, the method of disabling the BDOF for the CIIP mode is studied.

# Recommendations

- Review all CE tests’ contributions and cross-checks.

- Make decisions on each part on technology under consideration by reviewing CE results.

# Acknowledgement

- Thanks to all proponents for providing CE s/w in time with proper announcement and explanation about modification in the CE9 reflector.

- Thanks to all volunteers for providing the comprehensive cross-checks.

# References

- “Description of Core Experiment 9 (CE9): Decoder Motion Vector Derivation”, JVET-N1029, S. Esenlik, X.Xiu

- “JVET common test conditions and software reference configurations”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.