| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0031-v1 |
| --- | --- |

| Title: | CE11: Summary Report on Gradual Random Access |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui Jean-Marc Thiesse Hendry Kei Kawamura | Tel: Email: | kazui.kimihiko@fujitsu.com jean-marc.thiesse@vitec.com fhendry@futurewei.com ki-kawamura@kddi.com |
| Source: | CE11 coordinators |  |  |

_____________________________

- Abstract

A summary of Core Experiment 11 (CE11) on Gradual Random Access (GRA) is reported. The goal of CE11 is to study GRA tools/methods, for potential inclusion into the VVC standard.

Eight tests were defined in CE11:

- CE11-1: Encoder-only scheme (1 test)

- CE11-2: Virtual boundary (4 tests) (1 test was withdrawn)

- CE11-3: Tile/sub-picture (2 tests)

- CE11-4: Distributed IDR picture (1 test)

All tests are evaluated based on the common test conditions defined in JVET-N1031 [1]. All tests and crosscheck results are integrated in this report.

Test conditions and guidelines

CTC conditions for SDR [1] as agreed at the 14th JVET meeting are used for the evaluation of objective performance The following constraints are further applied as specified in the CE11 document [2].

- Low-delay B configuration only

- Interval of GRA or IRAP picture (1.0 second)

- QP setting (no offset for IDR and B picture)

- The GOP size is 1. Each picture refers up to 4 immediate preceding pictures in decoding order

The anchor is the ‘VVC Test Model’ or VTM. The VTM version 5.0 software is used with ‘vtm’ configuration files to produce the VTM anchor. Tools are implemented into the VTM version 5.0 software. Proposals are primarily compared with respect to objective compression efficiency while bitrate fluctuation and “exact match” aspects are also assessed.

Source codes corresponding with the test descriptions contained in this document are uploaded to the following CE11 GitLab project:

https://vcgit.hhi.fraunhofer.de/JVET-N-CE11/VVCSoftware_VTM.git

Cross-check reporting

Each test is covered by at least one cross-checker. Within CE11 it was agreed that cross-checkers answer the following questions:

- Did the cross-checker study the software? If so, did the cross-checker find any inconsistencies between the test description, the provided final software, and the VVC spec text changes?

- Do the cross-checked results match the proponent results for the test conditions? Were additional results cross-checked?

- Additional comments that may be helpful to the group?

Overview of CE11 test description document changes

No changes excluding membership were made to the CE11 test description document [2] after the T1 deadline (17 April 2019) had passed.

Defined tests in CE11

Summary

Table 4-1 – List of tested tools in CE11

| Test # | Short description | Doc. # | Cross-checker |
| --- | --- | --- | --- |
| CE11-1 | Encoder-only scheme - No explicit signalling of refresh boundary - Encoder-side restriction on MV and intra prediction mode - Redundant intra-coded region for disabling in-loop filter at refresh boundary | JVET-N0080 JVET-O0057 (Fujitsu) | Huawei |
| CE11-2.1.a | Virtual boundary - Refresh boundary = virtual boundary signalized at slice level - Encoder-side restriction on MV - Restriction on intra prediction with CIP at picture level - Disabling in-loop filter at refresh boundary | JVET-N0391 JVET-O0100 (VITEC) | KDDI |
| CE11-2.1.b | Virtual boundary - CE11-2.1.a - Enabling CIP locally with slice-level signalization |  | Huawei |
| CE11-2.1.c | Virtual boundary - CE11-2.1.b - Treating virtual boundary as picture boundary Withdrawn |  | n/a |
| CE11-2.2 | Virtual boundary - CE11-2.1.b - Dedicated signalling of GRA position with associated constraint derivation, and CU-level flags skipping. |  | Kimihiko Kazui (Fujitsu) |
| CE11-3.1 | Tile - Refresh boundary = tile boundary - Asymmetric MV restriction by treating tile boundary as picture boundary - Disabling intra prediction across tile boundary. This is inherited from tile, not particularly new for this test. - Disabling in-loop filtering across tile boundary | JVET-N0115 JVET-O0124 (Futurewei) | VITEC |
| CE11-3.2 | Sub-picture - Asymmetric MV restriction at sub-picture boundary - Refresh boundary = sub-picture boundary - Disabling intra prediction across tile boundary - Disabling in-loop filtering across tile boundary | JVET-N0310 JVET-O0063 (Nokia) | Huawei |
| CE11-4 | Distributed IDR - Extra intra-coded picture as reference for random access picture - Extra intra-coded picture is transmitted “slowly” and not output | JVET-N0116 JVET-O0116 (Huawei) | Nokia |

Detailed comparison of tested tools

Table 4-2 – Refreshing scheme

| Test # | Shape of refreshed area | Signalling of area | Precision of boundary position | Spatial direction of refresh |
| --- | --- | --- | --- | --- |
| CE11-1 | Vertically striped | n/a | 8 luma samples | Left to right |
| CE11-2 | Vertically striped | Virtual boundary | 8 luma samples | Left to right |
| CE11-3.1 | Vertically striped | Tile | New refresh area is coded as 1 tile (i.e., 1 intra slice) with width 1 CTU column | Left to right |
| CE11-3.2 | Vertically striped | Sub-picture | 8 luma samples | Left to right |
| CE11-4 | Picture | n/a | n/a | n/a |

Table 4-3 – Method for avoiding contamination of sample from dirty area to clean area

| Test # | In intra prediction | In inter prediction | In MV prediction | In in-loop filtering |
| --- | --- | --- | --- | --- |
| CE11-1 | Encoder-side mode restriction | Encoder-side MV restriction | Disabled at random access picture | Redundant intra-coded area |
| CE11-2 | Constrained intra prediction | Encoder-side MV restriction | Constrained by limiting the reference frame buffer when recycling (encoder-side) | Disabled at virtual boundary |
| CE11-3.1 | Disabled at tile boundary by default | Intrinsically constrained by treating clean area in the reference picture as picture boundary. This will effectively make blocks in the clean area never takes information (e,g., pel values, MV, etc) from non-clean area in the reference picture(s) | Intrinsically constrained by treating clean area in the reference picture as picture boundary | Disabled at tile boundary |
| CE11-3.2 | Disabled at sub-picture boundary by default | Encoder constraint such that blocks in clean area refer to clean area in the reference pictures. | Constrained by limiting area within the reference picture | Intrinsically disabled |
| CE11-4 | n/a | n/a | n/a | n/a |

Table 4-4 – Other aspects

| Test # | Additional normative changes | Decoding unit |
| --- | --- | --- |
| CE11-1 | n/a | One tile = One decoding unit = 1 CTU row |
| CE11-2 | CIP signalling (CE11-2.1.a) Virtual boundaries slice signalling (CE11-2.1.a) CIP enabled only at refreshed area and adjacent to refresh boundary coded in intra (CE11-2.1.b) Dedicated signalling method of CIP and virtual boundary (CE11-2.2) No explicit signalling of pred_mode_flag and cu_skip_flag of CU in in refreshed area (CE11-2.2) | If DU operation is desired, each CTU column can be coded as one tile. One tile one slice. |
| CE11-3.1 | - Signalling of clean area in slice header - Treating clean area boundary as picture boundary | If DU operation is desired, each CTU column can be coded as one tile. One tile one slice. |
| CE11-3.2 | Clean area and dirty areas are two different sub-pictures, width and height are signalled separately. |  |
| CE11-4 | Two NAL unit types are proposed with corresponded definitions | One tile = One decoding unit |

Table 4-5 – Other aspects

| Test # | Encoding condition | Cross-checker’s comment |
| --- | --- | --- |
| CE11-1 | HashME is disabled for Class-F GRA cycle of “BQSquare” is exceptionally set to 52. |  |
| CE11-2 | HashME is disabled for Class F |  |
| CE11-3.1 | ALF and LMCS are disabled | ALF and LCMS are disabled in both the anchor and the CE test. |
| CE11-3.2 | Class F case needs further investigation. | Slight mismatch between Windows and Linux simulations. Class F need more investigations. |
| CE11-4 | Encoder side ReuseCU is switched off (anchor doesn`t work with ReuseCU off) |  |

Test results

BD-Rate and runtime

Due to the bug reported in ticket #281, there is a mismatch between on Windows and Linux. In the CE11 anchor case, the error is within 0.1% (Class-D, U component). In each subtest, the error within the similar range is observed if the tester and cross-checker use different platform.

Table 5-1 – BD-rate and runtime
Numbers in hatched area are obtained on Windows. Others are on Linux.

|  | Low-delay B Main10 – Over VTM-5.0 with CE11 condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tester |  |  |  |  | Cross-checker |  |  |  |  |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE11-1 | 19.13% | 15.50% | 17.61% | 95% | 102% | (*1) |  |  |  |  |
| CE11-2.1.a | 9.54% | 11.28% | 16.61% | 96% | 114% |  |  |  |  |  |
| CE11-2.1.b | 7.96% | 8.66% | 13.02% | 101% | 115% | 7.99% | 8.60% | 13.02% | 98% | 102% |
| CE11-2.2 | 6.69% | 6.72% | 10.89% | 100% | 115% | 6.67% | 6.75% | 10.95% | 97% | 102% |
| CE11-3.1 | 2.41% | 7.25% | 11.27% | 139% | 125% | (*2) |  |  |  |  |
| CE11-3.2 | 6.26% | 4.05% | 7.90% | 98% | 109% | 6.26% | 4.05% | 7.90% | 97% | 98% |
| CE11-4 | 3.51% | -2.41% | -2.31% | 99% | 100% | (*2) |  |  |  |  |

(*1) Only Class-D sequences are tested so far and reportedly matched.

(*2) Only Class-C D and E sequences are tested so far and reportedly matched.

Bitrate fluctuation

Table 5-2 – Bitrate fluctuation

| Test # | Tester | Cross-checker |
| --- | --- | --- |
| CE11-1 |  |  |
| CE11-2 | Fully smooth with fluctuation only due to content. |  |
| CE11-3.1 |  |  |
| CE11-3.2 |  |  |
| CE11-4 | Spikes after RA points are near to mean value of average bit-stream. Few times less than IRAP pictures |  |

Exact match at recovery point picture

Table 5-3 – Exact match at recovery point picture

| Test # | Tester | Cross-checker |
| --- | --- | --- |
| CE11-1 | 1st GRA, matched Note: 4 mismatches at 2nd, 3rd, 5th GRA | Matched (Class-D) |
| CE11-2.1.a | 1st GRA, matched |  |
| CE11-2.1.b | 1st GRA, matched |  |
| CE11-2.2 | 1st GRA, matched | 1st GRA, matched |
| CE11-3.1 | Matched. | 1st GRA, matched for Class C D E and F |
| CE11-3.2 | 1st GRA, matched |  |
| CE11-4 | Matched |  |

References

Documents at previous meetings

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 5)”, Document of Joint Video Experts Team, JVET-N1001-v8, March 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010, March 2019

- K. Kazui, J-M. Thiesse, Hendry, K. Kawamura, “Description of Core Experiment 11 (CE11): Gradual random access”, Document of Joint Video Experts Team, JVET-N1031, March 2019

- Reference software VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- K. Kazui, “Revised software for ultra low-latency encoding”, Document of Joint Video Experts Team, JVET-N0080, March 2019

- J.-M. Thiesse, D. Gommelet, D. Nicholson “AHG14: Updates on Intra Refresh Proposal”, Document of Joint Video Experts Team, JVET-N0391, March 2019.

- M. M. Hannuksela, K.K. Sreedhar and A. Aminlou, “AHG14: On gradual decoding refresh” Document of Joint Video Experts Team, JVET-N0310, March 2019.

- Y-K. Wang, M. Sychev and Hendry, “AHG14/AHG17: DDR – Distributed decoding refresh” Document of Joint Video Experts Team, JVET-N0116, March 2019

CE documents

- K. Kazui, “CE11-1: GRA support by restriction to encoder operation”, Document of Joint Video Experts Team, JVET-O0075, July 2019.

- D. Gommelet, J-M. Thiesse, D. Nicholson, “CE11: Gradual Random Access (GRA) using encoder and normative restrictions (Tests CE11-2.1.a, CE11-2.1.b and CE11-2.2)” Document of Joint Video Experts Team, JVET-O0100, July 2019

- Hendry, Y-K. Wang, J. Chen, S. Hong, “CE11/AHG14: Test 3.1 - an approach for support of GRA/GDR” Document of Joint Video Experts Team, JVET-O0124, July 2019

- K. Kammachi-Sreedhar, M. M. Hannuksela, A. Aminlou, “CE11-3.2: Gradual Random Access (GRA) with Sub-Pictures” Document of Joint Video Experts Team, JVET-O0663, July 2019

- M. Sychev, “CE11-4: Distributed decoding refresh DDR” Document of Joint Video Experts Team, JVET-O0116, July 2019

CE-Related documents

- L. Wang, M. M. Hannuksela, “CE11-related: Wavefront-based GRA and Related Syntax” Document of Joint Video Experts Team, JVET-O0976, July 2019

- L. Wang, “CE11-related: Wavefront-based GRA Method” Document of Joint Video Experts Team, JVET-O0979, July 2019