| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0049-v1 |
| --- | --- |

| Title: | CE2-1.1: Simplified LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM5.0, reconstruction of a chroma transform block (TB) has to be delayed until luma prediction of the corresponding luma TB is totally done because of the chroma residual scaling when the luma mapping with chroma scaling (LMCS) is enabled. This contribution presents a method to reduce the delay. In the proposed method, one luma region of up to 256 luma samples corresponding to the chroma TB is used to derive one chroma residual inverse scale value, which is shared by the entire chroma TB. It is reported that Y/Cb/Cr BD-rates are 0.00%/-0.04%/-0.04% and 0.00%/0.04%/-0.06% for VTM5.0-RA and VTM5.0-LB, respectively, with negligible runtime changes.

# Introduction

## Chroma residual scaling

In VTM5.0, chroma residual scaling [1] is applied to chroma TBs to compensate for luma signal interaction with the chroma signal. The average value of luma prediction samples in the luma TB corresponding to the chroma TB is calculated and mapped to CScaleInv for chroma residual scaling at the decoder. The chroma residual scaling is derived as below.

Decoder side:CRes=CResScale/CScale=CResScale*CScaleInv

In the above equation, CResScale is the residual after inverse quantization and inverse transform, and CRes is the output of chroma residual scaling. As shown in Figure 1, the chroma residual scaling is done in the reshaped domain. Therefore, for inter TBs, a forward reshaping is applied to the luma prediction samples before calculating the average value of luma prediction samples in the luma TB.

Figure 1.  Intra mode and inter mode reconstruction with chroma residual scaling, where luma LF, luma DPB, and luma Motion Compensation are in the original domain and the rest are in the reshaped domain.

In case that intra subpartition (ISP) mode is applied to a coding unit (CU), the luma coding block (CB) is divided into multiple subpartitions, where each subpartition is a TB. The chroma CBs are not divided (i.e., CB = TB for chroma). The CResScale is derived using the last luma TB in the CU.

## Problem definition

In VTM3.0 reconstruction process, the chroma reconstruction has no dependency on luma prediction. Chroma reconstruction can be processed right after chroma prediction (assuming chroma inverse quantization and chroma inverse transform are ready before chroma prediction).

In VTM5.0, because of the chroma residual scaling shown in Figure 1, the chroma residuals of the entire chroma TB depend on all the luma prediction samples of the entire corresponding luma TB. The chroma reconstruction process cannot start until the luma prediction samples of the entire corresponding luma TB are available. The worst case happens when the luma TB size is 64×64, where the chroma reconstruction is delayed by the processing time of generating the 64×64 luma prediction samples.

# Proposed method

In the proposed method, one luma region of up to 256 luma samples corresponding to the chroma TB is used to derive one CResScale. The derived CResScale is shared by the entire chroma TB.

The luma region is defined as follows. If the corresponding luma TB size is less than or equal to 256 luma samples, the luma region is the entire luma TB. Otherwise, if both the corresponding luma TB width and height are larger than 16, the luma region is the top-left 16×16 luma samples. Otherwise (in this case the corresponding luma TB width or height is less than 16), the luma region is the top-left 256 luma samples of the luma TB.

# Experiment results

The proposed methods have been implemented on VTM5.0 [2]. The common test conditions [3] are used for evaluation. The results of the proposed method are summarized in Table 1.

Table 1.  Results of the proposed method

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.14% | 0.09% | 100% | 100% |
| Class A2 | -0.01% | 0.04% | -0.06% | 101% | 104% |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 97% |
| Class C | 0.00% | -0.05% | -0.15% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.04% | 100% | 99% |
| Class D | -0.01% | -0.03% | 0.10% | 100% | 100% |
| Class F | 0.00% | 0.02% | -0.01% | 101% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.04% | -0.04% | 100% | 101% |
| Class C | 0.02% | 0.17% | -0.15% | 100% | 101% |
| Class E | -0.04% | -0.13% | 0.03% | 100% | 99% |
| Overall | 0.00% | 0.04% | -0.06% | 100% | 100% |
| Class D | -0.06% | 0.24% | 0.61% | 99% | 98% |
| Class F | 0.04% | -0.11% | 0.07% | 98% | 98% |

# Conclusions

This contribution presents a method to reduce the delay of chroma reconstruction when enabling LMCS. The proposed method reportedly achieves 0.00%/-0.04%/-0.04% and 0.00%/0.04%/-0.06% Y/Cb/Cr BD-rates with negligible encoding and decoding time changes for VTM5.0-RA and VTM5.0-LB, respectively.

# References

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, and T. Chen, “CE12: Mapping functions (test CE12-1 and CE12-2),” Document of Joint Video Experts Team, JVET-M0427.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).